3GPP TSG-RAN WG3 #108-e	R3-204060

Online, 1-11 June 2020

Agenda Item:	9.3.8.1

Source:	China Telecom (moderator)

Title:	Summary of Offline Discussion on CB#87 5GS TAC clarification

Document for:	Approval

# Introduction

CB: # 87_5GS_TAC_clarification

- is early deployment text in 37.340 something that we should base further specification work on? If so, OK just for Rel-16?

- in case of conflict, CB on DC operation mode for SN-only shall take precedence

(CT - moderator)

Summary of offline disc R3-204060

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion

According to the progress in CB#87, as specified in TS37.340 as below and the “MIB only” case has been acknowledged.

TS 37.340 specifies that in MR-DC, the SN is not required to broadcast system information other than for radio frame timing and SFN.

Some clarification are needed in the corresponding specs in order to allow such case.

## Issue 1: Whether the Stage2 clarification for TS37.340 is needed or not? If Yes, how?

During the email discussion, some companies raised that some Note for TS37.340 may be needed, while some companies do not think it is needed.

The proposed wording could be:

Alt1) NOTE: The option that the SN does not broadcast system information other than radio frame timing and SFN relies on proper OAM configuration.

Alt2) NOTE: The option that the SN does not broadcast system information other than radio frame timing and SFN relies on proper OAM configurationand implementation and is not explicitlysupported on E-UTRAN or NG-RAN network interfaces.

Companies are invited to provide their opinion over this issue.

| Company | Comment |
| --- | --- |

## Issue2: Whether the stage 3 clarifications on the support of "MIB only” are needed? If Yes, how?

Per TS37.340, in MR-DC, the SN is not required to broadcast system information other than for radio frame timing and SFN. System information for initial configuration is provided to the UE by dedicated RRC signalling via the MN.

Form operators’ point of view, only broadcast MIB is not an” early drop” option.

- Per 37.340, UE is not required to read system information of SN .

- the System information for initial configuration is provided to the UE by dedicated RRC signalling via the MN.

- upon change of the relevant system information of a configured SCell, the network releases and subsequently adds the concerned SCell (with updated system information), via one or more RRC reconfiguration messages sent on SRB1 or SRB3, if configured.

- Not broadcasting SIB message can benefit to improve system performance

- According to our test result, it can improve the data peak rate (+3%~+5%) in downlink direction

- Energy saving: less downlink transmission in off-peak hour

Due to operators usually configure not broadcast any SIB message in SN node, the semantic description of 5GS TAC and served PLMN are not clear in current spec. Therefore, the stage 3 clarification on the support of only broadcast MIB in SN is needed.

The following options are proposed:

- Alt1) The possible clarification in stage3 are proposed in [2][3][4][5].

Per TS38.423/TS36.423 spec, the Broadcast PLMNs come from the broadcast SIB1. However, the SN may not broadcast SIB1 via air interface.  Therefore, the semantic description of Broadcast PLMNs and Broadcast PLMN Identity Info List E-UTRA need to be updated to avoid any confusion which brings IOT issue under inter-vendor deployments.

Take below IE for example:

| Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs. It may not broadcast when it acts as SN only in the case of MR-DC as defined in TS 37.340 [8]. | – |  |
| --- | --- | --- | --- | --- | --- | --- |

- Alt2) Add a Note under EN-DC X2 Setup procedure and EN-DC Configuration Update procedure for TS36.423, and the Xn Setup procedure and NG-RAN node Configuration Update procedure for TS38.423.

### 8.7.1	EN-DC X2 Setup

#### 8.7.1.1	General

The purpose of the EN-DC X2 Setup procedure is to exchange application level configuration data needed for eNB and en-gNB to interoperate correctly over the X2 interface. This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also resets the X2 interface like a Reset procedure would do.

NOTE:	If X2-C signalling transport is shared among multiple X2-C interface instances, one EN-DC X2 Setup procedure is issued per X2-C interface instance to be setup, i.e. several X2 Setup procedures may be issued via the same TNL association after that TNL association has become operational.

NOTE:   In the case of the SN does not broadcast system information, the transmission of serving cell information of the SN is allowed.

The procedure uses non UE-associated signalling.

Companies are invited to provide their opinion over this issue.

| Company | Comment |
| --- | --- |

## Issue 2: both Rel-15 and Rel-16 CRs are needed?

According to chairmen notes, we need to discuss whether both R15 and R16 CRs are needed.

Companies are invited to provide their opinion over this issue.

| Company | Comment |
| --- | --- |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-203128 Further Discussion on the semantics description of 5GS TAC and Served PLMNs (China Telecom, ZTE, CATT, Huawei, China Unicom)

- R3-203130 Update on semantics description of 5GS TAC and Served PLMN (China Telecom, ZTE, CATT, Huawei, China Unicom)  CR1493r, TS 36.423 v15.9.0, Rel-15, Cat. F

- R3-203133 Update on semantics description of 5GS TAC and Served PLMN (China Telecom, ZTE, CATT, Huawei, China Unicom) CR1494r, TS 36.423 v16.1.0, Rel-16, Cat. A

- R3-203250 Update on semantics description of 5GS TAC and Served PLMN (ZTE, China Telecom, CATT, China Unicom) CR0379r, TS 38.423 v15.7.0, Rel-15, Cat. F

- R3-203252 Update on semantics description of 5GS TAC and Served PLMN (ZTE, China Telecom, CATT, China Unicom) CR0379r, CR0381r, TS 38.423 v16.1.0, Rel-16, Cat. A