3GPP TSG-RAN WG3 #108-e	R3-204006

Online, 1-11 June 2020

Agenda Item:	17.2.3

Source:	Huawei (Moderator)

Title:	Summary of Offline Discussion on Time Reference Information

Document for:	Approval

# Introduction

CB: # NRIIOT6-time_ref_info

- the UTC reference timing in SIB9 should be obtained from gNB-DU similar to 5G accurate reference time?

- the reference time type shall be indicated in Reporting Request Type from gNB-CU in terms of UTC only, 5G only or UTC and 5G?

- check RAN2 progress, if agreeable, check details and revise as needed

(HW - moderator)

Summary of offline disc R3-204006

# For the Chairman’s Notes

Propose the following:

Proposal 1: To be added.

# Discussion

There are proposals in R3-203658 to transfer the UTC reference timing over F1 interface as follows.

- Proposal 1: The UTC reference timing in SIB9, which is delivered in dedicated RRC signalling should be obtained from gNB-DU similar to 5G accurate reference time.

- Proposal 2: The reference time type shall be indicated in Reporting Request Type from gNB-CU in terms of UTC only, 5G only or UTC+5G.

For connected UE, RAN2 already agreed to use the UEAssistanceInformation to request the 5G accurate reference time.

- The request of the reference time information is sent via the UEAssistanceInformation message.

Also in TS 38.331 v16.1.0, the RRC DLInformationTransfer message includes the referenceTimeInfo. But at this RAN2#110 meeitng, there are papers to further discuss whether the UTC reference is included in the dedicated DL RRC message (possibly either in DLInformationTransfer or in RRCReconfiguraiton message).

Hence the moderator suggests to wait RAN2 further progress, then continues our discussion (the moderator may make instant update if any need).

Company can provide views: Whether the TP for F1AP is needed, i.e. The UTC reference timing is delivered from gNB-DU to gNB-CU upon the gNB-CU request?

| Company | Comments |
| --- | --- |
| Huawei | We can wait RAN2 discussion first. If RAN2 agrees to include the UTC reference is included in the dedicated DL RRC message, then the TP is needed. |

# Conclusion, Recommendations [if needed]

If needed

# References

| R3-203658 | (TP for NR_IIoT BL CR for TS 38.425): Finalization of enhancement 3 (Huawei) | other |
| --- | --- | --- |