3GPP TSG-RAN WG3 #109-e	R3-205494

Online, 17-27 August 2020

Agenda Item:	20.2.4

Source:	Ericsson (moderator)

Title:	CB: # 38_NTN_FeederSwitch - Summary of email discussion

Document for:	Approval

# Introduction

Our chairman’s summary:

CB: # 38_NTN_FeederSwitch

- ZTE

The “On ground NTN gNB” shall know as little as possible about LEO satellite(s) flying in space, as NTN-GW(s) will do everything with NR-Uu signal in consequence as planned.

In principle, the “On ground NTN gNB” is not responsible for feeder link switch over, and the NTN-GW(s) performs feeder link switch over with each other in planned way.

To support “hard switch over” and “ LEO satellite sharing” across multiple neighbor NTN-GWs/gNBs with higher priority.

- HW

feeder link switch should be performed without causing service disruption to the served UEs.

feeder link hard switch impact on RAN3 is pending RAN2 progress.

Unless RAN3 issue is detected this topic should be put on hold pending to RAN2 progress.

- E///

Add to Xn Setup and NG-RAN Configuration Update procedures the list of satellites to which the gNB connects, and for each satellite on the list include at least the list of cells from the gNB served through the satellite, and the ephemeris data.

Introduce a new XnAP Class 1, non-UE-associated Satellite Connection Preparation procedure to support satellite feeder link switchover for transparent satellites.

We should not limit the scope of satellite link switchover to LEO only, but rather adopt a generic wording in specification text if possible.

Introduce all necessary NTN-related IEs in RAN3 protocols as optional with criticality “reject”.

- Th

Supporting NTN soft feeder link switch over shall be considered as first priority for Rel. 17.

Supporting NTN hard feeder link switch over shall be considered as second priority for Rel. 17.

- SS (5395)

During feeder link switch over for one satellite served by two GWs simultaneously, it is necessary to take more consideration how to avoid the issue of RLF and RRC re-establishment so as to keep the service continuity for the UE after the state transition from IDLE to CONNECTED.

During feeder link switch over for one satellite served by two GWs simultaneously, it is beneficial to ensure UE in idle state directly access in cell2 due to DL paging or UL signaling/data, since handover procedure can be avoidable and it also can reduce signaling overhead.

- SS (5405)

beneficial to exchange assistance information over Xn to mitigate RACH congestion for HOs due to feeder link switch over

- CATT

Both hard and soft hard link switch should be supported in NTN Rel-17 WI.

A new Xn procedure should be introduced to exchange the necessary info for feeder link switch, including satellite information, served cell(s) information, and an optional time T for the target gNB to start the establishment of the new feeder link.

Detailed design of the Uu interface is pending to the discussion of RAN1 and RAN2.

Introduce a Container to transfer the satellite configuration in UPLINK RAN CONFIGURATION TRANSFER and DOWNLINK RAN CONFIGURATION TRANSFER NGAP messages.

++ Suggested guidelines/topics for discussion from Chair (looking at possible consensus):

+ Consensus to support feeder link switchover in specs; full solution requires both RAN2 and RAN3 impact

+ Consensus to support both hard and soft switchover? If so, prioritize soft switchover? Potential WA to support both with e.g. same procedure(s)?

+ Xn impact: information to be signaled between “old” and “new” gNBs? List of satellites/cells? Ephemeris? Timers? Others?

+ Any NG impact? E.g. sat config container in RAN config transfer messages? Others?

+ st2/3 TPs (XnAP, NGAP)? (lots of FFSs)

+ need to liaise RAN2?

(E/// - moderator)

Summary of offline disc R3-205494

# For the Chairman’s Notes

It is proposed to agree on the following:

1.	Agree on support of feeder link switchover - with the assumption of RAN2 and RAN3 specification impact.

2.	Agree to support soft and hard switchover in Rel-17, with priority on soft switchover.

3.	Agree on stage 2 as modified in [R3-20xxxx was R3-205161].

4.	Agree on a preliminary set of functions executed between involved (source and target) gNBs, if possible:

-	Xn based exchange of neighbouring GW/gNB configuration data of currently served satellites/cells.

-	Xn based preparation of switch over to prepare handover to the target GW/gNB

-	[any other topic that may come during offline discussion]

# Discussion

## Support of feeder link switch over - General

There seems to be consensus to support feeder link switchover in Rel-17. It is therefore proposed to agree on the following.

1.	Agree on support of feeder link switchover - with the assumption of RAN2 and RAN3 specification impact.

Description…

| Company | Comment |
| --- | --- |

## On soft and hard switchover

There seems to be a common understanding that soft switchover is necessary due to service continuity reasons, while hard switchover might almost come “for free”, although for the latter more details on feasibility seems to be clarified. So it looks like we can agree on the following:

2.	Agree to support soft and hard switchover in Rel-17, with priority on soft switchover.

Description…

| Company | Comment |
| --- | --- |

## Stage 2 description following the 2 items above

It seems to valid to start discussions on stage 2 text. Please provide your comments on R3-205161 which will be modified as you wish/demand. The final aim is to have the following agreement:

3.	Agree on stage 2 as modified in [R3-20xxxx was R3-205161].

| Company | Comment |
| --- | --- |

## Preliminary set of functions executed between involved gNBs:

This topic aims at identifying protocol functions that can be foreseen for support of soft switchover.

The final text for agreement would be as shown below, please provide under c) and ff) subitems if you wish to include a further item in the list. Probably this step needs more iterations.

This is the text for the proposal:

4.	Agree on a preliminary set of functions executed between involved (source and target) gNBs, if possible:

a)	Xn based exchange of neighbouring GW/gNB configuration data of currently served satellites/cells.

b)	Xn based preparation of switch over to prepare handover to the target GW/gNB.

c)	[any other topic that may come during offline discussion]

| Company | Comment |
| --- | --- |

## Comments on stage 3 in R3-205160

Outcome of this subitem might be a commented, “FFS”-ed version of R3-205160. If there is no agreement to deal with a revision of R3-205160 as the BL CR for next meeting, it would be nevertheless good to start collecting comments.

Agreement text to be provided based on feedback.

| Company | Comment |
| --- | --- |

# Conclusion, Recommendations [if needed]

To be updated, if needed.

# References

- R3-204666 Initial Thoughts on NTN LEO Feeder Link Switch-Over  (ZTE)

- R3-204965 Discussion on feeder link switch for NTN (Huawei)

- R3-205159 Supporting Feeder Link Switchover (Ericsson LM)

- R3-205160 Support for Feeder Link Switchover for Transparent Architecture (Ericsson LM)

- R3-205161 Feeder Link Switchover for Transparent NTN - Stage 2 (Ericsson LM)

- R3-205173 On NTN Feeder link switch over (THALES)

- R3-205395 Discussion for the served UE during feeder link switch over (Samsung)

- R3-205405 Discussion on RACH issue in feeder link switch over (Samsung)

- R3-205450 Discussion on Feeder Link Switch (CATT)