3GPP TSG-RAN WG3 #109-e	Draft-R3-205562

Online, 17-28 August 2020

Agenda Item:	9.3.1

Source:	Ericsson (moderator)

Title:	Summary of Offline Discussion on CB: # 63_NASnonDelivery

Document for:	Approval

# Introduction

CB: # 63_NASnonDelivery

- clarify scenario:

1) Which NAS PDU is included in PDU session resources setup message?

2) In which case NG-RAN will fail to transmit the Non-PDU NAS to the UE during PDU session resource setup procedure(/initial context setup request)?

- extend use case beyond inactive case vs. stick to SA2 scenarios?

- consensus on usage for initial context setup

- clarify case of PDU session resource setup

- need for st2 change?

- reply LS to SA2

(E/// - moderator)

Summary of offline disc R3-205562

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion

As the first step let us discuss the scenario and problems. When the above is sorted out and have an agreement on the problems to be solved, the implementation of the specification impacts would be easier to conclude.

## Issue 1:

Should we only discuss solution for UE in RRC Inactive state, or in general how to handle when non PDU session related NAS PDU is not delivered to the UE?

| Company | Comment |
| --- | --- |
| Ericsson | It is beneficial to discuss one solution for all cases, including UE in RRC Inactive state. And one solution for handling the “same” non PDU session related NAS PDU received in different NGAP messages. |

## Issue 2:

Which NAS PDU is included in PDU session resources setup message?

Here we can include the “Selective activation of UP connection” case and the other cases, if any.

| Company | Comment |
| --- | --- |
| Ericsson | Seems no restriction for AMF to put other NAS-PDU (non-MM, non-SM) in the PDU session setup. One example is MO data: UE send the SR in connected mode to setup UP. In this case, AMF need to send Service Accept. Could be piggybacked in one message. |

## Issue 3:

In which case NG-RAN will fail to transmit the Non-PDU NAS to the UE during PDU session resource setup procedure or initial context setup request?

| Company | Comment |
| --- | --- |
| Ericsson | It is similar to the Downlink Data Transfer case when the NAS PDU is not delivered. When RRC is failed. |

In below we discuss about the solutions. We can take this part as a second step.

## LS Reply?

| Company | Comment |
| --- | --- |

## NGAP TS 38.413?

| Company | Comment |
| --- | --- |

## Stage 2 TS 38.300?

| Company | Comment |
| --- | --- |

# Conclusion, Recommendations

If needed

# References

- R3-204825	Correction of NAS NON Delivery (Nokia, Nokia Shanghai Bell, Orange)

- R3-204827	Correction of NAS NON Delivery (Nokia, Nokia Shanghai Bell, Orange)

- R3-204828	Correction of NAS NON Delivery (Nokia, Nokia Shanghai Bell, Orange)

- R3-204826	Response LS on NAS NON Delivery for RRC INACTIVE State (Nokia, Nokia Shanghai Bell, Orange)

- R3-205132	Correction of NAS Non Delivery Indication (CATT)

- R3-205133	Correction to NAS Non Delivery Indication (CATT)

- R3-205134	Correction to NAS Non Delivery Indication (CATT)

- R3-204988	Further discussion on NAS Non-Delivery (Huawei)

- R3-204989	Further discussion on NAS Non-Delivery (Huawei)

- R3-204990	Further discussion on NAS Non-Delivery (Huawei)

- R3-204991	[Draft] Reply LS on NAS Non delivery for RRC Inactive state (Huawei)

- R3-204997	NAS PDU non delivery (Ericsson)

- R3-204998	NAS PDU handling (Ericsson)

- R3-204999	NAS PDU handling (Ericsson)

- R3-205000	NAS PDU handling (Ericsson)

- R3-205001	NAS PDU handling (Ericsson)