3GPP TSG-RAN WG3 #109-e							R3-205481

17-28 August 2020

Online

Agenda Item:	15.3

Source:	Huawei - Moderator

Title:	Summary of Offline Discussion on latency measurement

Document for:	Approval

# Introduction

CB: # NRQoE3-Others

-  For E2E RAN delay, it should indicate the time duration of a packet being transmitted from an ingress point of PDCP in sending side to an egress point of PDCP in receiving side, investigate mechanisms to achieve a more precise way for E2E RAN delay measurements? (HW)

-  Check the issue, capture agreements and and open issues in the summary, if identified

(HW - moderator)

Summary of offline disc

# For the Chairman’s Notes

Propose to capture the following:

# Discussion

## The RAN part delay measurement in R16 is not precise

As analysed in [1], the current mechanism for RAN part delay measurement is divided into several parts, the total delay is the sum of average values of each part. In addition, the delay over radio is estimated by the granularity of slot. We could see imprecisions here with such mechanisms:

- the way of averaging each delay with a period of time would hide the longest delay which weaken the worst case

- while the granularity of slot is a bit coarse for some certain URLLC services which would require the precision in microsecond level.

Needless to say that for DL delay, neither the PDCP re-ordering delay nor the RLC delay of UE for UM mode is included, which is also not helpful to evaluate E2E delay for URLLC services.

With such brief analysis, we would suggest that the RAN3 group reach the consensus that the current mechanism for RAN part delay measurement in R16 is not precise, and new mechanisms are needed.

Issue 1: Acknowledge that the current mechanism for RAN part delay measurement in R16 is not precise, and new precise mechanisms are needed.

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Yes | New precise mechanism are needed, since more precise RAN part delay measurement is needed for new services, e.g. URLLC. |
| Qualcomm |  | The accuracy optimization/enhancement can be endless. We should justify the need of enhancement by concrete use case. So, in the TR, we can have one section to discuss the use case of delay measurement enhancements. |
| China Telecom | Yes | More precise mechanism is needed in principle. But we also suggest to analyze briefly the performance gain for the precise enhancement mechanism. |

## The requirements for more precise RAN part delay

According to the analysis above, we could see that the imprecisions are mainly due to the following shortcomings, the first one is coarse timing precision, the second one is the division of an E2E calculation into serval parts/segments, and thirdly each part is calculated with average values.

In order to achieve a more precise E2E delay, the mechanism should at least target a calculation of a whole and complete E2E path (not several divided segments), and the timing precision should be improved, slot based calculation should be avoided.

As also suggested in [1] that, for the whole and complete E2E path, we should be have an explicit definition as a path for a packet being transmitted from an ingress point of PDCP in sending side to an egress point of PDCP in receiving side, and the timing info could be based on absolute time stamp or other means which could be further discussed.

Issue 2: Acknowledge that more precise RAN part delay should be calculated based on whole and complete E2E path (e.g. not sum of several segments) with more precise timing info (e.g. not slot based).

| Company | Answer | Comment |
| --- | --- | --- |
| Huawei | Yes | A complete E2E path with more precise timing info are key factors to make a more precise E2E delay measurement. |
| Qualcomm |  | Similar to answer to last question, we should discuss the use case of accuracy enhancement first. |
| China Telecom | Yes | Agree with Huawei |

## Other KPI to be investigated

Since the E2E latency is just one type of UE KPI information, the scope of this SI should be open to discuss other potential UE KPI information, here we would like to welcome companies’ input.

Question. Any other potential UE KPI info to be investigated?

| Company | Answer | Comment |
| --- | --- | --- |

# Conclusion, Recommendations

See section 2.

# References

[1]	R3-205282, Initial discussion on latency measurement (Huawei)	discussion