TSG-RAN Working Group 3 Meeting #11	R3-000917

Sophia Antipolis, France, 28th February – 3rd March 2000

Source:	TSG-RAN WG3 / Iu SWG

To:		TSG-RAN WG2, TSG-CN WG1

Cc:

Contact:	Pierre Lescuyer / lescuyer@nortelnetworks.com

+33 6 85 74 36 96

# Title:	LS on Paging cause in UTRAN protocols

R3 has recently discussed the provision of an explicit "paging cause" in the paging messages defined in RANAP protocol. Currently, this cause (which is also seen by RRC in the paging message) may have the following values:

- speech call,

- CS data call,

- PS data call,

- SMS

Based on those values, the impression of R3 is that this paging cause is not needed by UTRAN and should be passed transparently from CN to UE in the paging message, in order to keep AS and NAS independency. Therefore, and if it is confirmed that the "paging cause" is actually needed by the UE, it should not be seen as such by UTRAN, but rather as "NAS transparent information".

If R2 and N1 agree on following these principles, the specifications need to be updated as indicated below, before RAN#7:

- in R2: replacement of "paging cause" by a "transparent NAS paging information" in the RRC protocol (25.331)

- in N1: definition of the content of "transparent NAS paging information", possibly using the causes listed above, or any other N1 would require

- in R3: replacement of "paging cause" by a "transparent NAS paging information" in the RANAP protocol (25.413)