| 3GPP-RAN-WG3 Meeting #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  |  | TSGR3#11(00)0924 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sophia-Antipolis, 28 Feb - 3 Mar 2000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | e.g. for 3GPP use the format TP-99xxx or for SMG, use the format P-99-xxx |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 25.413 |  |  |  |  |  | CR |  |  | 001r3 |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | 3.0.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  | TSG-RAN#7 |  |  |  |  | for approval |  |  |  |  |  |  | x |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  |  | UTRAN / Radio |  |  |  |  |  |  |  |  |  | x | Core Network |  |  |  |  |  |  | x |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | Nortel Networks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  | 02.03.2000 |  |  |  |  |  |
| Subject: |  |  | Clarification and correction of the CN broadcast procedure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  | x |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |  |
| Reason for change: |  |  | Enhancement of the CN Broadcast procedure, and completion of the coding section. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 8.24	CN Information Broadcast

### 8.24.1	General

The purpose of  the CN Information Broadcast procedure is to broadcast repetitively to all users information as provided by the core network. The procedure uses connectionless signalling.

### 8.24.2	Successful Operation

Figure 1: CN Information Broadcast procedure. Successful operation.

A core network element sets or modifies the CN broadcast information by sending a  CN INFORMATION BROADCAST REQUEST message whichcontains:

-	The information pieces to be broadcast. The internal structure of these information pieces is transparent to UTRAN, and is specified as part of the CN-UE protocols.

-	With each broadcast information piece, a geographical area where to broadcast it.

-	With each broadcast information piece, a priority used by UTRAN to schedule the information

-	With each broadcast information piece, a request for the UTRAN to turn on or off the broadcast of the information piece

If the UTRAN can broadcast the information as requested, a  CN INFORMATION BROADCAST CONFIRM message is returned to the CN.

Whether or not UTRAN shall treat equally broadcast request from different CN and having the same priority is under operator control.

Each information piece is broadcast in the intersection between the indicated geographical area and the area under control by the receiving RNC. It is broadcast until explicitly changed or a Reset occurs

### 8.24.3	Unsuccessful Operation

Figure 2: CN Information Broadcast procedure. Unsuccessful operation.

If after receiving the CN INFORMATION BROADCAST REQUEST, the RNC can not broadcast the information as requested, a  CN INFORMATION BROADCAST REJECT message shall be returned to the CN and the procedure is terminated.

### 9.1.33	CN INFORMATION BROADCAST REQUEST

This message is sent by the CN and includes information to be broadcasted to all users.

Direction: CN  RNC

Signalling bearer mode: Connectionless.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  |
| CN Domain Indicator | M |  | 9.2.1.5 |  |
| CN Broadcast Information piece |  | 1 to <maxnoofPieces> |  |  |
| Information Identity | M |  | 9.2.3.X |  |
| NAS Broadcast Information | C-ifBroadcast |  | 9.2.3.5 |  |
| Area Identity | C-ifBroadcast |  | 9.2.3.11 |  |
| Information Priority | C-ifBroadcast |  | 9.2.3.X |  |
| Information Control | M |  | 9.2.3.X |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPieces | Maximum no. of Broadcast Information Pieces in one message. Value is 16. |

| Condition | Explanation |
| --- | --- |
| IfBroadcast | This IE is only present if CN requests the Broadcast of the corresponding information piece |

#### 9.2.3.X Information Identity

#### This element is used to identify Broadcast Information piece for a given CN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Information Identity | M |  | INTEGER (0..255) |  |

#### 9.2.3.X Information Priority

This element is the priority of the corresponding Information piece. Thie IE is used by UTRAN to schedule the NAS Broadcast Information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Information Priority | M |  | INTEGER (0..15) | spare (0), highest (1), lowest (14), no priority used (15)} (0..15) |

#### 9.2.3.X Information Control

This element is used to control the Broadcast of an Information piece.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Information Control | M |  | ENUMERATED(on,off) | on: UTRAN shall start broadcasting the information piece off: UTRAN shall stop broadcasting the information piece |

### 9.3.3	PDU Definitions

IMPORTS

DataVolumeReference,

AreaIdentity,

CN-DomainIndicator,

Cause,

CriticalityDiagnostics,

ChosenEncryptionAlgorithm,

ChosenIntegrityProtectionAlgorithm,

ChosenUP-Version,

ClassmarkInformation2,

ClassmarkInformation3,

DL-GTP-PDU-SequenceNumber,

DL-N-PDU-SequenceNumber,

DataVolumeReportingIndication,

EncryptionInformation,

IntegrityProtectionInformation,

IuTransportAssociation,

L3-Information,

LAI,

NAS-BindingInformation,

NAS-BroadcastInformation,

InformationIdentity,

InformationPriority,

InformationControl,

NAS-PDU,

NonSearchingIndication,

NumberOfSteps,

OMC-ID,

OldBSS-ToNewBSS-Information,

PagingAreaID,

PagingCause,

PermanentNAS-UE-ID,

RAB-ID,

RAB-Parameters,

RAC,

RelocationType,

RequestType,

SAI,

SAPI,

SourceID,

SourceRNC-ToTargetRNC-TransparentContainer,

TargetID,

TargetRNC-ToSourceRNC-TransparentContainer,

TemporaryUE-ID,

TraceReference,

TraceType,

UnsuccessfullyTransmittedDataVolume,

TransportLayerAddress,

TriggerID,

UE-ID,

UL-GTP-PDU-SequenceNumber,

UL-N-PDU-SequenceNumber,

UP-ModeVersions,

UserPlaneMode

FROM RANAP-IEs

-- **************************************************************

--

-- CN Information Broadcast Request

--

-- **************************************************************

CN-InformationBroadcastRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {CN-InformationBroadcastRequestIEs} },

protocolExtensions		ProtocolExtensionContainer { {CN-InformationBroadcastRequestExtensions} }		OPTIONAL,

...

}

CN-InformationBroadcastRequestIEs RANAP-PROTOCOL-IES ::= {

{ ID id-CN-DomainIndicator			CRITICALITY ignore	TYPE CN-DomainIndicator				PRESENCE mandatory	} |

{ ID id-CN-BroadcastInformationPieceList	CRITICALITY ignore	TYPE CN-BroadcastInformationPieceList 		PRESENCE mandatory	},

...

}

CN-BroadcastInformationPieceList 			::= CN-BroadcastInfPiece-IE-ContainerList { {CN-BroadcastInformationPieceIEs} }

CN-BroadcastInformationPieceIEs RANAP-PROTOCOL-IES ::= {

{ ID id-CN-BroadcastInformationPiece		CRITICALITY ignore	TYPE CN-BroadcastInformationPiece		PRESENCE mandatory	},

...

}

CN-BroadcastInformationPiece ::= SEQUENCE {

informationIdentity				InformationIdentity,

nAS-BroadcastInformation			NAS-BroadcastInformation		OPTIONAL

--Included if CN resquests UTRAN to broadcast the information piece,

areaIdentity					AreaIdentity		OPTIONAL

--Included if CN resquests UTRAN to broadcast the information piece,

informationPriority				InformationPriority		OPTIONAL

--Included if CN resquests UTRAN to broadcast the information piece,

informationControl				InformationControl,

iE-Extensions					ProtocolExtensionContainer { {CN-BroadcastInformationPiece-ExtIEs} }			OPTIONAL,

...

}

CN-BroadcastInformationPiece-ExtIEs RANAP-PROTOCOL-EXTENSION ::= {

...

}

CN-InformationBroadcastRequestExtensions RANAP-PROTOCOL-EXTENSION ::= {

...

}

### 9.3.4	Information Element Definitions

-- C

-- I

InformationIdentity ::= INTEGER (0..255)

InformationPriority ::= INTEGER (0..15)

InformationControl ::= ENUMERATED {

on,

off

}