3GPP TSG-RAN3 Meeting #110-e	R3-206018

E-Meeting, 02 – 12 November, 2020

Title:	LS on information needed for MRO in SCG Failure Report

Response to:

Release:	Rel-17

Work Item:	          NR_ENDC_SON_MDT_enh

Source:	Samsung

To:	RAN2

Cc:	-

Contact Person:

Name:	Lixiang Xu

E-mail Address:	lx.xu@samsung.com

- Overall Description:

RAN3 discussed the solution for the optimization of SN change failure for MRO in case of MR-DC. RAN3 reached agreement on the information which is necessary for the optimization of SN change failure in SCG Failure Report:

- CGI of the Source PSCell: the source PSCell of the last SN change. The source PSCell could be E-UTRA cell or NR cell.

- CGI of the Failed PSCell: the PSCell in which SCG failure is detected or the target PSCell of the failed SN change. The Failed PSCell could be E-UTRA cell or NR cell.

- timeSCGFailure: the time elapsed since the last SN change initialization until SCG failure.

- 2. Actions:

To RAN2:

ACTION: RAN3 respectfully asks RAN2 to take the above into account and to update their corresponding specifications.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #111-e	    25th Jan - 5th Feb.   2021