3GPP TSG-RAN WG3 Meeting #110-e                                                            	R3-206023

2-12 November 2020

Title:	LS on MRO for CHO and DAPS

Response to:

Release:	Rel-17

Work Item:	          NR_ENDC_SON_MDT_enh-Core

Source:	Samsung

To:	RAN2

Cc:	-

Contact Person:

Name:	Lixiang Xu

E-mail Address:	lx.xu@samsung.com

- Overall Description:

RAN3 discussed the MRO for CHO and DAPS, and identified the following information for UE reporting:

- CHO

  - Timer since CHO execution to connection failure

  - The existing timeConnFailure is used to indicate timer elapsed from receiving RRC Reconfiguration message including CHO configuration to the connection failure

  - For CHO failure followed by CHO recovery failure, the information related with the two failure are useful in the network side. RAN3 understand RAN2 will decide whether to include two failure cell ids and two sets of timer related with each failure in RLF Report or to define Two RLF Report.

- DAPS

  - The time to the success RACH to the target cell since the RLF in source cell

  - Measurement results when the RLF occurs at the source cell in additional to measurement results contained in the RLF report for the RLF/HOF at the target cell

  - DAPS HOF type indication

- 2. Actions:

To RAN2:

ACTION: RAN3 respectfully asks RAN2 to take the above into account and to decide the parts involving UE behavior.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #111-e	    25th. Jan -  5th   Feb.   2021