3GPP TSG RAN WG3#110-e	R3-206080

2nd November-12th November, 2020

Online

Title: 	(TP for 38.423) Introducing the successful HO report in XnAP

Source: 	China Telecom

Agenda item:	10.2.1.3

Document for:	Approval

Introduction

In the last RAN3 meeting, we have reached a WA that the Access and Mobility Indication message is used to transmit Successful Report to the source NG-RAN node over Xn, and we have agreed to Define “Successful HO Report” as RRC container in XnAP.

In this document, we introduce a new Successful HO Report List IE in the ACCESS AND MOBILITY INDICATION message contains the Successful HO Report Container IE received from UE, the max number of Successful HO Report need FFS.

Text Proposal

Following changes depend on the 3GPP TS38.423 V16.3.0:

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

8.4.12	Access And Mobility Indication

8.4.12.1	General

The purpose of the Access and Mobility Indication procedure is to transfer Access and Mobility related information between NG-RAN nodes.

8.4.12.2	Successful Operation

Figure 8.2.12.2-1: Access And Mobility Indication. Successful operation

NG-RAN node1 initiates the procedure by sending the ACCESS AND MOBILITY INDICATION message sent to NG-RAN node2.

If the Successful Handover Successful Report is received in the NG-RAN node1 from the UE as described in TS 38.331[10], the NG-RAN node1 shall include it as Successful Handover Report Container IE in the Successful HO Report List IE contained in the ACCESS AND MOBILITY INDICATION message.

8.4.12.3	Abnormal Conditions

Not applicable.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

9.1.3.25	ACCESS AND MOBILITY INDICATION

This message is sent by NG-RAN node1 to transfer access and mobility related information to NG-RAN node2.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| RACH Report List |  | 0..1 |  |  | YES | ignore |
| >RACH Report List Item |  | 1 .. <maxnoofRACHReports> |  |  | EACH | ignore |
| >>RACH Report Container | O |  | OCTET STRING | RACH-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [10]. | YES | ignore |
| Successful HO Report List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report List Item |  | 1 .. <maxnoof SuccessfulHOReports> |  |  | EACH | ignore |
| >>Successful HO Report Container | O |  | OCTET STRING |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHReports | Maximum no. of RACH Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is FFS. |

/////////////////////////////////////////////////////////////////////////change end////////////////////////////////////////////////////////////////////////////////////

Reference

- TR37.816, Study on RAN-centric data collection and utilization for LTE and NR, (Release 16)

- TS38.423, XnAP, (Release 16)

- TS38.413, NGAP, (Release 16)

- TS38.473, F1AP, (Release 16)

- R3-205510, Summary of Offline Discussion on Successful Handover Report, Samsung, 2020-08.

- R3-206063, Discussion on support of Successful Handover report, China Telecom, 2020.11.