3GPP TSG-RAN WG3 #110-e	R3-206134

E-meeting, 2nd – 12th November 2020	Revise of R3-205111

Source:	CATT

Title:	Discussion on Rel-16 leftover issues for PRACH coordination

Agenda Item:	10.2.1.7

Document for:	Discussion and decision

Introduction

During the offline discussion last meeting [1], the key opinion split is over the number of cells which PRACH configuration a gNB-CU can provide toward a gNB-DU in a F1AP message for PRACH coordination, and similarly, an eNB can provide toward an en-gNB for PRACH coordination. Some companies prefer a “high number”, and others prefer a “low number” with some extra enhancement. In this contribution we would like to emphasise again why a “high number” is proper, and extra enhancement is not needed or even problematic.

Discussion

Take gNB-CU/DU split as an example. We don’t think there is any real necessity for a gNB-DU to provide any report toward the gNB-CU in order to trigger the DL PRACH configuration delivery, and on the opposite, we find the potential problem of such report:

On one side, the signalling size of NR Cell PRACH Configuration is not that huge—it is typically comparable with E-UTRA’s. The number of intra-frequency neighbour cells is not issue either.

For a network consisting of intra-frequency cells with similar sizes, one cell will typically have only a few intra-frequency neighbour cells. And for the case that one macro cell “covers” 1000 intra-frequency micro cells (assuming that such case exists), it should be the work of the 1000 micro cells to read the one configuration of the one macro cell and to adjust configuration in order to avoid interference, rather than the macro cell to read the 1000 configurations of the 1000 micro cells and make some adjustments itself.

Based on the analysis above, for any case we need not deliver too many “neighbour cell configurations” toward the gNB-DU based on filtering by gNB-CU. The length of the PRACH list delivered from the gNB-CU toward the gNB-DU would naturally be comparable with the one reported from the gNB-DU toward the gNB-CU.

Considering the fact that we have already permitted a gNB-DU to report at most 512 cell’s PRACH configuration toward the gNB-CU (as already specified in Rel-16 TS 38.473), it is reasonable to permit a gNB-CU to deliver at most 512 cell’s PRACH configuration toward the gNB-DU as well.

Please consider the following scenario: one gNB-DU hosts 512 cells, and generate a long F1AP message toward the gNB-CU which contains the PRACH configuration of all these 512 cells. Then, it is the time for gNB-CU to deliver neighbouring cell’s PRACH configuration toward the gNB-DU. And someone suddenly comes and says, “We are very concerning that the gNB-DU does not have the capability to decode / store such a long F1AP message; so a max length of 16 must be enforced.” Is his word reasonable?

Observation 1: A gNB-CU/eNB can filter the PRACH configuration of intra-frequency neighbour cells on its own to prevent huge signalling, e.g. not to include the PRACH configuration of a micro neighbour cell which is a neighbour to a macro cell served by the gNB-DU/en-gNB.

Observation 2: Based on proper filtering, the length of the PRACH list of the neighbour cells provided to the gNB-DU/en-gNB would be comparable with the one reported from the gNB-DU/en-gNB.

On the other side, the method of “conflict detection” would likely incur persistent signalling wasting. Many possible reasons can cause a cell receiving the MSG1 but no consecutive MSG3, e.g. RF issue over MSG2/3 delivering. Such wasting will easily overtake the other solution, in which PRACH configurations are only needed to be exchanged only upon e.g. cell setup since they are usually semi-static.

Observation 3: The method of “potential PRACH conflict detection” would likely incur persistent signalling wasting, since RF issue may also cause that a gNB receives a MSG1 but no consecutive MSG3.

Proposal 1: No need for the gNB-DU/en-gNB to report any “potential PRACH conflict”.

The next question is on where to include the PRACH configuration of neighbour cells.

For X2AP the solution is quite straightforward: there is one § 9.2.98 “NR Neighbour Information” and including it there seems working well. However this structure lacks of IEs that can indicate the location and bandwidth of carriers (except the one of the SUL), the TDD pattern and the number of SSB, so we have to add them as well.

Proposal 2: An optional IE “NR Cell PRACH Configuration” is proposed to be added into the § 9.2.98 “NR Neighbour Information” inside TS 36.423, as well as some necessary IEs to deliver the location and bandwidth of carriers, the TDD pattern and the number of SSB.

But for the F1AP the case is a little bit complex. It has ever been proposed to include it into the existing Neighbour Cell Information List IE, but there has been some concern that it may disturb the CLI function. According to the current version of TS 38.473:

If the Intended TDD DL-UL Configuration NR IE is absent from the Neighbour Cell Information List IE, whereas the corresponding NR CGI IE is present, the receiving gNB-DU shall remove the previously stored Neighbour Cell Information IE corresponding to the NR CGI.

The field Intended TDD DL-UL Configuration NR is defined as a “release-if-absent” one, making the Neighbour Cell Information List not suitable to extend for any function not related to CLI.

Observation 4: The field Intended TDD DL-UL Configuration NR is defined as a “release-if-absent” one, making the Neighbour Cell Information List not suitable to extend for any function not related to CLI.

Therefore we propose to add a list of neighbour cells directly within some F1AP DL messages, namely “Cell Information Notification List”, with PRACH coordination-related parameters included within it as optional IEs. Unlike the feature TDD CLI, there is already a method to indicate a release of PRACH configuration: setting the list length as zero. Therefore we do not need the “absence of PRACH configuration” serves as an indication of release—instead it means “no change”, similar to most interface IEs. As the result, “Cell Information Notification List” can be further expanded for other purposes in the future.

There was some doubt on whether it is necessary to add into the F1 SETUP RESPONSE message as well, claiming that there is no use case as F1 interface is always setup before Xn interface. However such claim is not true. In gNB-CU/DU split structure one gNB-CU can cover a considerably large area, and adding new gNB-DU into it after every other interfaces were setup could be a common case. Thus there is still some use case to deliver neighbour cells’ PRACH configuration toward a gNB-DU within the F1 SETUP RESPONSE message.

Considering the abovementioned observation that its length should be similar to the one reported from the gNB-DU toward the gNB-CU, we propose to define its maximum length as maxCellingNBDU for alignment. This is also the length of the Neighbour Cell Information List.

Proposal 3: A new IE, namely “Cell Information Notification List”, is proposed to be included into the following F1AP messages: F1 SETUP RESPONSE, GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE and GNB-CU CONFIGURATION UPDATE.

Proposal 4: The maximum length of the “Cell Information Notification List” should be maxCellingNBDU for alignment.

Proposal 5: IEs related to PRACH coordination should be added into each item of the new “Cell Information Notification List” IE in order to carry the PRACH configuration of the cells belonging to other gNB(-DU)s toward the gNB-DU.

Conclusion

Observation 1: A gNB-CU/eNB can filter the PRACH configuration of intra-frequency neighbour cells on its own to prevent huge signalling, e.g. not to include the PRACH configuration of a micro neighbour cell which is a neighbour to a macro cell served by the gNB-DU/en-gNB.

Observation 2: Based on proper filtering, the length of the PRACH list of the neighbour cells provided to the gNB-DU/en-gNB would be comparable with the one reported from the gNB-DU/en-gNB.

Observation 3: The method of “potential PRACH conflict detection” would likely incur persistent signalling wasting, since RF issue may also cause that a gNB receives a MSG1 but no consecutive MSG3.

Proposal 1: No need for the gNB-DU/en-gNB to report any “potential PRACH conflict”.

Proposal 2: An optional IE “NR Cell PRACH Configuration” is proposed to be added into the § 9.2.98 “NR Neighbour Information” inside TS 36.423, as well as some necessary IEs to deliver the location and bandwidth of carriers, the TDD pattern and the number of SSB.

Observation 4: The field Intended TDD DL-UL Configuration NR is defined as a “release-if-absent” one, making the Neighbour Cell Information List not suitable to extend for any function not related to CLI.

Proposal 3: A new IE, namely “Cell Information Notification List”, is proposed to be included into the following F1AP messages: F1 SETUP RESPONSE, GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE and GNB-CU CONFIGURATION UPDATE.

Proposal 4: The maximum length of the “Cell Information Notification List” should be maxCellingNBDU for alignment.

Proposal 5: IEs related to PRACH coordination should be added into each item of the new “Cell Information Notification List” IE in order to carry the PRACH configuration of the cells belonging to other gNB(-DU)s toward the gNB-DU.

Based on abovementioned proposals, we draft two TPs accordingly [2–3].

Reference

[1] R3-205663; CB: # 1007_SONMDT_RACH - Summary of email discussion - second round; Nokia (moderator).

[2] R3-206135; (TP on SON for 38.473) TP on PRACH coordination for F1AP; CATT.

[3] R3-206136; (TP on SON for 36.423) TP on PRACH coordination for X2AP; CATT.