3GPP TSG-RAN3 Meeting #110-e	R3-206147

2 – 12 November 2020, E-Meeting

Title: 	(TP on UE history information for 36.423) Addition of UE history information for SN

Source: 	CATT

Agenda item:	10.2.1.4

Document Type:	other

# 1. Introduction

This contribution is the TP for TS36.423 v16.3.0 with addition of UE history information for SN.

# 2. Discussion

As discussed in R3-206145, UE history information for SN in MR-DC is introduced.

# 3. Annex – TP

Start of the first change

9.2.38	UE History Information

The UE History Information IE contains information about cells that a UE has been served by in active state prior to the target cell. The overall mechanism is described in TS 36.300 [15].

NOTE:	The definition of this IE is aligned with the definition of the UE History Information IE in TS 36.413 [4].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Last Visited Cell List |  | 1..<maxnoofCells> |  | Most recent information is added to the top of this list | – |  |
| >Last Visited Cell Information | M |  | 9.2.39 |  | – |  |
| > Last Visited PSCell List |  | 0..1 |  |  |  |  |
| >> Last Visited PSCell Item |  | 1..<maxnoofPSCells> |  | Most recent information is added to the top of this list |  |  |
| >>> Last Visited PSCell Information | M |  | 9.2.X |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCells | Maximum number of last visited cell information records that can be reported in the IE. Value is 16. |
| maxnoofPSCells | Maximum number of last visited PSCell information records that can be reported in the IE. Value is 16. |

9.2.X	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Last Visited PSCell Information | M |  |  |  | - |  |
| >E-UTRAN Cell |  |  |  |  | - |  |
| >>Last Visited E-UTRAN PSCell Information | M |  | 9.2.Y |  | - |  |
| >NG-RAN Cell |  |  |  |  |  |  |
| >>Last Visited NG-RAN PSCell Information | M |  | OCTET STRING | Defined in TS 38.413 [39]. |  |  |

9.2.Y	Last Visited E-UTRAN PSCell Information

The Last Visited E-UTRAN PSCell Information contains information about a cell that is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PSCell Global Cell ID | O |  | ECGI 9.2.14 |  | - |  |
| PCI | O |  | INTEGER (0..503, …) | Physical Cell Identifier of PSCell |  |  |
| EARFCN | O |  | 9.2.26 | DL EARFCN for FDD or EARFCN for TDD |  |  |
| EARFCN Extension | O |  | 9.2.65 | DL EARFCN for FDD or EARFCN for TDD. If this IE is present, the value signalled in the EARFCN IE is ignored. |  |  |
| Time UE stayed in Cell | M |  | INTEGER (0..4095) | The duration of the time the UE stayed in the PSCell in seconds. If the UE stays in a PSCell more than 4095s, this IE is set to 4095. | - |  |
| Time UE stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of the time the UE stayed in the PSCell in 1/10 seconds. If the UE stays in a PSCell more than 4095s, this IE is set to 40950. | YES | ignore |

End of the last change