3GPP TSG-RAN WG3 #110-e						R3-206158

E-meeting, 02 – 12 November 2020

Agenda Item:  10.2.6

Source:       NEC

Title:		Mobility Enhancement Optimization: data forwarding on HO to wrong cell

Document for:	dicision

- 1. Introduction

- 10.2.6. Mobility Enhancement Optimization Scope: SON Enhancements for CHO (i.e MRO for CHO) will be supported. SON Enhancements for DAPS handover will be supported. Postpone SON Enhancements for CPC with waiting for the progress of R17 CPC enhancements and SON enhancements for CHO. It is FFS whether SON enhancements for conditional PSCell change should be supported. Study resource optimization for CHO, based on contributions Decide if the problem of data forwarding in case of a HO to wrong cell is part of the SON WI (SON for Mobility Enhancements) or is to be treated as TEI-17. MRO for CHO: FFS whether CHO specific failure types are needed. The existing definitions of too late handover /too early handover/ handover to wrong cell are the starting point for further study. From RAN3 point of view, in order to support MRO for CHO, more information is needed from UE. (FFS on the details). Study the contents of the RLF INDICATION or HANDOVER REPORT message to support MRO enhancements for CHO. In order to progress in this area it is necessary to converge on the CHO failure case definition. SON Enhancements for DAPS handover: Reporting of failure information of the source link from UE may be needed for DAPS handover (FFS: Need further discussion). From RAN3 point of view, in order to support SON enhancements for DAPS handover, more information is needed from UE. (FFS on the details). Study the contents of the RLF INDICATION or HANDOVER REPORT message for the failure scenarios in DAPS HO. In order to progress in this area it is necessary to converge on the DAPS failure case definition.

This contribution discuss the open point regarding the “Decide if the problem of data forwarding in case of a HO to wrong cell is part of the SON WI (SON for Mobility Enhancements) or is to be treated as TEI-17.”

- 2. Discussion

If it is possible, it would be good to enhance the data forwarding during the handover failure so can increase the reliability, which has been a basic requirement from the 5G URLLC.

Handover to Wrong Cell

For the case when the handover failure before the UE setup connection to the original target cell, but the UE re-established to other cell belong to other gNB, this is the “handover to wrong cell”.

When the source gNB receive the Retrieve UE Context Request from the re-established gNB, the source gNB know it has handover the UE to wrong cell, but since the data forwarding has been executed, likely the source gNB does not buffer the data, it then send the Handover Cancel message to the original target gNB with the TNL address of the re-established gNB, for the purpose of asking the original target gNB to forward the data to the re-established gNB. (refer [2] R3-204986)

Checking the impact on the existing Handover Cancel function in the XNAP, it is likely that the target gNB will cancel the handover preparation including release all the resource related with the UE Context, then adding this new data forwarding function require the original target gNB to keep the original forwarding TNL tunnel as well as establish another forwarding TNL tunnel towards the re-established gNB. The original target gNB need to keep these forwarding TNL tunnels until it receive end-marker of the GTP packet from the source gNB. However the impact does not seem to be complicated nor impossible to realize.

Conclusion 1: it is beneficial to have data forwarding enhancement for the case of Handover to wrong cell.

Too Early Handover

For the case when the handover failure after the UE setup connection to the original target cell, but the UE re-established to other cell belong to other gNB (or the source gNB), this is the “too early handover”.

Since the handover has once been successful in the original target gNB, it is highly likely the original target gNB executed path switch towards the 5GC and has started sending DL data to the UE. Then if the UE re-establish to other gNB, it will be like the existing re-establishment procedure, which does not require any further adding nor enhancement.

Conclusion 1: there is no need to have data forwarding enhancement for the case of Too Early Handover.

Regarding where to have such work (TEI17 of in Rel-17 SON WI), since this is related with the handover failure and handover to wrong cell, for better tracking, it would be preferable to work in this Rel-17 Enhancement of Data Collection for SON/MDT in NR WI instead of TEI17.

Proposal: it is proposed to consider the data forwarding enhancement in case of handover to wrong cell, as the scope of this Rel-17 Enhancement of Data Collection for SON/MDT in NR WI

- 3. proposal

Proposal: it is proposed to consider the data forwarding enhancement in case of handover to wrong cell, as the scope of this Rel-17 Enhancement of Data Collection for SON/MDT in NR WI

References:

[1] R3-204985 Data forwarding in case of a HO to wrong cell

[2] R3-204986 Enabling fetching DL data in case of a HO to wrong cell

[3] R3-205519. Summary of Offline Discussion on Mobility Enhancement Optimization