3GPP TSG-RAN3 Meeting #110-e	R3-206181

# Online, , 2nd Nov 2020 - 12th Nov 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0486 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inter MN resume without SN change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-10-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | MN change without SN change is supported in handover but not in RRC resume. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add UE Context Reference at the S-NG-RAN node IE into RETRIEVE UE CONTEXT RESPONSE - Defined new message RETRIEVE UE CONTEXT CONFIRM Impact assessment towards the previous version of the specification: This CR has no backward compatibility issue. If old MN does not support, these changes will not be triggered. If target MN does not support, the new IE in RETRIEVE UE CONTEXT RESPONSE would be ignored and legacy procedure is used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SN has to be released in RRC resume from different MN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1, 8.2.4.2, 8.2.x, 9.1.1.9, 9.1.1.y, 9.3.3, 9.3.4, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

====================================1st Change====================================

## 8.1	Elementary procedures

In the following tables, all Eps are divided into Class 1 and Class 2 Eps.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA – NR Cell Resource Coordination | E-UTRA – NR CELL RESOURCE COORDINATION REQUEST | E-UTRA – NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Retrieve UE Context Confirm | RETRIEVE UE CONTEXT CONFIRM |

====================================2nd Change====================================

### 8.2.4	Retrieve UE Context

#### 8.2.4.1	General

The purpose of the Retrieve UE Context procedure is to either retrieve the UE context from the old NG-RAN node and transfer it to the NG-RAN node where the UE RRC Connection has been requested to be established, or to enable the old NG-RAN node to forward an RRC message to the UE via the new NG-RAN node without context transfer.

The procedure uses UE-associated signalling.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: Retrieve UE Context, successful operation

The new NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT REQUEST message to the old NG-RAN node.

If the old NG-RAN node is able to identify the UE context by means of the UE Context ID, and to successfully verify the UE by means of the integrity protection contained in the RETRIEVE UE CONTEXT REQUEST message, and decides to provide the UE context to the new NG-RAN node, it shall respond to the new NG-RAN node with the RETRIEVE UE CONTEXT RESPONSE message.

If the Index to RAT/Frequency Selection Priority IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall store this information and use it as defined in TS 23.501 [7].

If the Location Reporting Information IE is included in the RETRIEVE UE CONTEXT RESPONSE message, then the new NG-RAN node should initiate the requested location reporting functionality as defined in TS 38.413 [5].

If the Trace Activation IE is included in the RETRIEVE UE CONTEXT RESPONSE message which includes

-	the MDT Activation IE set to "Immediate MDT and Trace", then the target NG-RAN node shall if supported, initiate the requested trace session and MDT session as described in TS 32.422 [23].

-	the MDT Activation IE set to "Immediate MDT Only" or "Logged MDT only", the target NG-RAN node shall, if supported, initiate the requested MDT session as described in TS 32.422 [23] and the target NG-RAN node shall ignore the Interfaces To Trace IE, and the Trace Depth IE.

-	the MDT Location Information IE, within the MDT Configuration IE, the target NG-RAN node shall, if supported, store this information and take it into account in the requested MDT session.

-	the MDT Activation IE set to "Immediate MDT Only" or "Logged MDT only", and if the Signalling based MDT PLMN List IE is included in the MDT Configuration IE, the target NG-RAN node may use it to propagate the MDT Configuration as described in TS 37.320 [43].

-	the Bluetooth Measurement Configuration IE, within the MDT Configuration IE, the target NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [43].

-	the WLAN Measurement Configuration IE, within the MDT Configuration IE, the target NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [43].

-	the Sensor Measurement Configuration IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [43].

-	the MDT Configuration IE and if the target NG-RAN Node is a gNB at least the MDT Configuration-NR IE shall be present, while if the target NG-RAN Node is an ng-eNB at least the MDT Configuration-EUTRA IE shall be present.

For each QoS flow in the RETRIEVE UE CONTEXT RESPONSE message, if the QoS Monitoring Request IE is included in the QoS Flow Level QoS Parameters IE in the PDU Session Resources To Be Setup List IE, the new NG-RAN node shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [7].

If the 5GC Mobility Restriction List Container IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, store this information in the UE context and use it as specified in TS 38.300 [9].

V2X:

-	If the NR V2X Services Authorized IE is included in the RETRIEVE UE CONTEXT RESPONSE message and it contains one or more IEs set to "authorized", the new NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

-	If the LTE V2X Services Authorized IE is included in the RETRIEVE UE CONTEXT RESPONSE message and it contains one or more IEs set to "authorized", the new NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

-	If the NR UE Sidelink Aggregate Maximum Bit Rate IE is included in the UE Context Information Retrieve UE Context Response IE in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for NR V2X services.

-	If the LTE UE Sidelink Aggregate Maximum Bit Rate IE is included in the UE Context Information Retrieve UE Context Response IE in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for LTE V2X services.

If the PC5 QoS Parameters IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the target NG-RAN node shall, if supported, use it as defined in TS 23.287[38].

In case of RRC Re-establishment, the old NG-RAN may include the UE History Information IE or the UE History Information from the UE IE in the RETRIEVE UE CONTEXT RESPONSE message. Upon reception of the UE History Information IE or the UE History Information from the UE IE in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, store the collected information and use it for future handover preparations.

If the UE Radio Capability ID IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG- RAN node shall, if supported store this information in the UE context and use it as defined in TS 23.501 [7] and TS 23.502 [13].

If the UE Context Reference at the S-NG-RAN IE is contained in the RETRIEVE UE CONTEXT RESPONSE message the new NG-RAN node may use it to establish dual connectivity with the S-NG-RAN node. In this case, the old NG-RAN node may expect the new NG-RAN node to include the UE Context Kept Indicator IE set to "True" in the RETRIEVE UE CONTEXT CONFIRM message.

#### 8.2.4.3	Unsuccessful Operation

Figure 8.2.4.3-1: Retrieve UE Context, unsuccessful operation

If the old NG-RAN node is not able to identify the UE context by means of the UE Context ID, or if the integrity protection contained in the RETRIEVE UE CONTEXT REQUEST message is not valid, or, if it decides not to provide the UE context to the new NG-RAN node, it shall respond to the new NG-RAN node with the RETRIEVE UE CONTEXT FAILURE message.

If the old NG-RAN node decides to keep the UE context in case of periodic RNAU, it shall store the Allocated C-RNTI IE and the Access PCI IE in the UE Context ID IE, as described in TS 38.300 [9].

If the Old NG-RAN node to New NG-RAN node Resume Container IE is included in the RETRIEVE UE CONTEXT FAILURE message, the new NG-RAN node should transparently forward the content of this IE to the UE as described in TS 38.300 [9].

#### 8.2.4.4	Abnormal Conditions

Void.

====================================3rd Change====================================

### 8.2.x	Retrieve UE Context Confirm

#### 8.2.x.1	General

The Retrieve UE Context Confirm procedure is used to confirm S-NG-RAN node is reused by the new NG-RAN node after UE context retrieval.

The procedure uses UE-associated signalling.

#### 8.2.x.2	Successful Operation

Figure 8.2.3.2-1: Retrieve UE Context Confirm, successful operation

The new NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT CONFIRM message to the old NG-RAN node, when S-NG-RAN node is reused by the new NG-RAN node after UE context retrieval.

If any DRB of the S-NG-RAN node is moved to the new NG-RAN node, DRBs transferred to MN IE is included in the RETRIEVE UE CONTEXT CONFIRM message. The old NG-RAN node uses this information in data forwarding.

#### 8.2.x.3	Unsuccessful Operation

Not applicable.

#### 8.2.x.4	Abnormal Conditions

If the RETRIEVE UE CONTEXT CONFIRM message refers to a context that does not exist, the old NG-RAN node shall ignore the message.

====================================4th Change====================================

#### 9.1.1.9	RETRIEVE UE CONTEXT RESPONSE

This message is sent by the old NG-RAN node to transfer the UE context to the new NG-RAN node.

Direction: old NG-RAN node  new NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node | YES | ignore |
| Old NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the old NG-RAN node | YES | ignore |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information – Retrieve UE Context Response | M |  | 9.2.1.13 |  | YES | reject |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

====================================5th Change====================================

#### 9.1.1.y	RETRIEVE UE CONTEXT CONFIRM

This message is sent by the new NG-RAN node to the old NG-RAN node to indicate S-NG-RAN node is not released.

Direction: old NG-RAN node  new NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Old NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the old NG-RAN node | YES | ignore |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List | M |  | 9.2.1.2 |  | YES | ignore |
| UE Context Kept Indicator | M |  | 9.2.3.68 |  | YES | ignore |
| DRBs transferred to MN | O |  | DRB List 9.2.1.29 | In case of DC, indicates that SN Status is needed for the listed DRBs from the S-NG-RAN node. | YES | ignore |

====================================6th Change====================================

### 9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

XnAP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM XnAP-CommonDataTypes

HandoverRequest,

HandoverRequestAcknowledge,

HandoverPreparationFailure,

SNStatusTransfer,

UEContextRelease,

HandoverCancel,

NotificationControlIndication,

RANPaging,

RetrieveUEContextRequest,

RetrieveUEContextResponse,

RetrieveUEContextConfirm

RetrieveUEContextFailure,

XnUAddressIndication,

SecondaryRATDataUsageReport,

SNodeAdditionRequest,

SNodeAdditionRequestAcknowledge,

SNodeAdditionRequestReject,

SNodeReconfigurationComplete,

SNodeModificationRequest,

SNodeModificationRequestAcknowledge,

SNodeModificationRequestReject,

SNodeModificationRequired,

SNodeModificationConfirm,

SNodeModificationRefuse,

SNodeReleaseRequest,

SNodeReleaseRequestAcknowledge,

SNodeReleaseReject,

SNodeReleaseRequired,

SNodeReleaseConfirm,

SNodeCounterCheckRequest,

SNodeChangeRequired,

SNodeChangeConfirm,

SNodeChangeRefuse,

RRCTransfer,

XnRemovalRequest,

XnRemovalResponse,

XnRemovalFailure,

XnSetupRequest,

XnSetupResponse,

XnSetupFailure,

NGRANNodeConfigurationUpdate,

NGRANNodeConfigurationUpdateAcknowledge,

NGRANNodeConfigurationUpdateFailure,

E-UTRA-NR-CellResourceCoordinationRequest,

E-UTRA-NR-CellResourceCoordinationResponse,

ActivityNotification,

CellActivationRequest,

CellActivationResponse,

CellActivationFailure,

ResetRequest,

ResetResponse,

ErrorIndication,

PrivateMessage,

DeactivateTrace,

TraceStart,

HandoverSuccess,

ConditionalHandoverCancel,

EarlyStatusTransfer,

FailureIndication,

HandoverReport,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusFailure,

ResourceStatusUpdate,

MobilityChangeRequest,

MobilityChangeAcknowledge,

MobilityChangeFailure,

AccessAndMobilityIndication

FROM XnAP-PDU-Contents

id-handoverPreparation,

id-sNStatusTransfer,

id-handoverCancel,

id-notificationControl,

id-retrieveUEContext,

id-rANPaging,

id-xnUAddressIndication,

id-uEContextRelease,

id-secondaryRATDataUsageReport,

id-sNGRANnodeAdditionPreparation,

id-sNGRANnodeReconfigurationCompletion,

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-mNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeCounterCheck,

id-sNGRANnodeChange,

id-activityNotification,

id-rRCTransfer,

id-xnRemoval,

id-xnSetup,

id-nGRANnodeConfigurationUpdate,

id-e-UTRA-NR-CellResourceCoordination,

id-cellActivation,

id-reset,

id-errorIndication,

id-privateMessage,

id-deactivateTrace,

id-traceStart,

id-handoverSuccess,

id-conditionalHandoverCancel,

id-earlyStatusTransfer,

id-failureIndication,

id-handoverReport,

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

id-mobilitySettingsChange,

id-accessAndMobilityIndication,

id-retrieveUEContextConfirm

FROM XnAP-Constants;

-- **************************************************************

--

-- Interface Elementary Procedure Class

--

-- **************************************************************

XNAP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome				OPTIONAL,

&UnsuccessfulOutcome				OPTIONAL,

&procedureCode			ProcedureCode	UNIQUE,

&criticality			Criticality		DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE		&InitiatingMessage

[SUCCESSFUL OUTCOME		&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE			&procedureCode

[CRITICALITY			&criticality]

}

-- **************************************************************

--

-- Interface PDU Definition

--

-- **************************************************************

XnAP-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

XNAP-ELEMENTARY-PROCEDURES XNAP-ELEMENTARY-PROCEDURE ::= {

XNAP-ELEMENTARY-PROCEDURES-CLASS-1			|

XNAP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

XNAP-ELEMENTARY-PROCEDURES-CLASS-1 XNAP-ELEMENTARY-PROCEDURE ::= {

handoverPreparation										|

retrieveUEContext										|

sNGRANnodeAdditionPreparation							|

mNGRANnodeinitiatedSNGRANnodeModificationPreparation	|

sNGRANnodeinitiatedSNGRANnodeModificationPreparation	|

mNGRANnodeinitiatedSNGRANnodeRelease					|

sNGRANnodeinitiatedSNGRANnodeRelease					|

sNGRANnodeChange										|

xnRemoval												|

xnSetup													|

nGRANnodeConfigurationUpdate							|

e-UTRA-NR-CellResourceCoordination						|

cellActivation											|

reset													|

resourceStatusReportingInitiation						|

mobilitySettingsChange									,

...

}

XNAP-ELEMENTARY-PROCEDURES-CLASS-2 XNAP-ELEMENTARY-PROCEDURE ::= {

sNStatusTransfer						|

handoverCancel							|

rANPaging								|

xnUAddressIndication					|

uEContextRelease						|

sNGRANnodeReconfigurationCompletion		|

sNGRANnodeCounterCheck					|

rRCTransfer								|

errorIndication							|

privateMessage							|

notificationControl						|

activityNotification					|

secondaryRATDataUsageReport 			|

deactivateTrace							|

traceStart								|

handoverSuccess							|

conditionalHandoverCancel				|

earlyStatusTransfer						|

failureIndication						|

handoverReport							|

resourceStatusReporting					|

accessAndMobilityIndication				|

retrieveUEContextConfirm				,

...

}

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

handoverPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverRequest

SUCCESSFUL OUTCOME		HandoverRequestAcknowledge

UNSUCCESSFUL OUTCOME	HandoverPreparationFailure

PROCEDURE CODE			id-handoverPreparation

CRITICALITY				reject

}

sNStatusTransfer	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNStatusTransfer

PROCEDURE CODE			id-sNStatusTransfer

CRITICALITY				ignore

}

handoverCancel	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverCancel

PROCEDURE CODE			id-handoverCancel

CRITICALITY				ignore

}

retrieveUEContext	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RetrieveUEContextRequest

SUCCESSFUL OUTCOME		RetrieveUEContextResponse

UNSUCCESSFUL OUTCOME	RetrieveUEContextFailure

PROCEDURE CODE			id-retrieveUEContext

CRITICALITY				reject

}

rANPaging	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RANPaging

PROCEDURE CODE			id-rANPaging

CRITICALITY				reject

}

xnUAddressIndication	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		XnUAddressIndication

PROCEDURE CODE			id-xnUAddressIndication

CRITICALITY				reject

}

uEContextRelease	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextRelease

PROCEDURE CODE			id-uEContextRelease

CRITICALITY				reject

}

sNGRANnodeAdditionPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeAdditionRequest

SUCCESSFUL OUTCOME		SNodeAdditionRequestAcknowledge

UNSUCCESSFUL OUTCOME	SNodeAdditionRequestReject

PROCEDURE CODE			id-sNGRANnodeAdditionPreparation

CRITICALITY				reject

}

sNGRANnodeReconfigurationCompletion	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeReconfigurationComplete

PROCEDURE CODE			id-sNGRANnodeReconfigurationCompletion

CRITICALITY				reject

}

mNGRANnodeinitiatedSNGRANnodeModificationPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeModificationRequest

SUCCESSFUL OUTCOME		SNodeModificationRequestAcknowledge

UNSUCCESSFUL OUTCOME	SNodeModificationRequestReject

PROCEDURE CODE			id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation

CRITICALITY				reject

}

sNGRANnodeinitiatedSNGRANnodeModificationPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeModificationRequired

SUCCESSFUL OUTCOME		SNodeModificationConfirm

UNSUCCESSFUL OUTCOME	SNodeModificationRefuse

PROCEDURE CODE			id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation

CRITICALITY				reject

}

mNGRANnodeinitiatedSNGRANnodeRelease	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeReleaseRequest

SUCCESSFUL OUTCOME		SNodeReleaseRequestAcknowledge

UNSUCCESSFUL OUTCOME	SNodeReleaseReject

PROCEDURE CODE			id-mNGRANnodeinitiatedSNGRANnodeRelease

CRITICALITY				reject

}

sNGRANnodeinitiatedSNGRANnodeRelease	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeReleaseRequired

SUCCESSFUL OUTCOME		SNodeReleaseConfirm

PROCEDURE CODE			id-sNGRANnodeinitiatedSNGRANnodeRelease

CRITICALITY				reject

}

sNGRANnodeCounterCheck	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeCounterCheckRequest

PROCEDURE CODE			id-sNGRANnodeCounterCheck

CRITICALITY				reject

}

sNGRANnodeChange		XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeChangeRequired

SUCCESSFUL OUTCOME		SNodeChangeConfirm

UNSUCCESSFUL OUTCOME	SNodeChangeRefuse

PROCEDURE CODE			id-sNGRANnodeChange

CRITICALITY				reject

}

rRCTransfer	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RRCTransfer

PROCEDURE CODE			id-rRCTransfer

CRITICALITY				reject

}

xnRemoval	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		XnRemovalRequest

SUCCESSFUL OUTCOME		XnRemovalResponse

UNSUCCESSFUL OUTCOME		XnRemovalFailure

PROCEDURE CODE			id-xnRemoval

CRITICALITY				reject

}

xnSetup	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		XnSetupRequest

SUCCESSFUL OUTCOME		XnSetupResponse

UNSUCCESSFUL OUTCOME		XnSetupFailure

PROCEDURE CODE			id-xnSetup

CRITICALITY				reject

}

nGRANnodeConfigurationUpdate	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NGRANNodeConfigurationUpdate

SUCCESSFUL OUTCOME		NGRANNodeConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	NGRANNodeConfigurationUpdateFailure

PROCEDURE CODE			id-nGRANnodeConfigurationUpdate

CRITICALITY				reject

}

e-UTRA-NR-CellResourceCoordination	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-UTRA-NR-CellResourceCoordinationRequest

SUCCESSFUL OUTCOME		E-UTRA-NR-CellResourceCoordinationResponse

PROCEDURE CODE			id-e-UTRA-NR-CellResourceCoordination

CRITICALITY				reject

}

cellActivation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CellActivationRequest

SUCCESSFUL OUTCOME		CellActivationResponse

UNSUCCESSFUL OUTCOME	CellActivationFailure

PROCEDURE CODE			id-cellActivation

CRITICALITY				reject

}

reset	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResetRequest

SUCCESSFUL OUTCOME		ResetResponse

PROCEDURE CODE			id-reset

CRITICALITY				reject

}

errorIndication	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-errorIndication

CRITICALITY				ignore

}

notificationControl			XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NotificationControlIndication

PROCEDURE CODE			id-notificationControl

CRITICALITY				ignore

}

activityNotification		XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ActivityNotification

PROCEDURE CODE			id-activityNotification

CRITICALITY				ignore

}

privateMessage			XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY				ignore

}

secondaryRATDataUsageReport	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SecondaryRATDataUsageReport

PROCEDURE CODE			id-secondaryRATDataUsageReport

CRITICALITY				reject

}

deactivateTrace XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DeactivateTrace

PROCEDURE CODE			id-deactivateTrace

CRITICALITY				ignore

}

traceStart XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TraceStart

PROCEDURE CODE			id-traceStart

CRITICALITY				ignore

}

handoverSuccess			XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverSuccess

PROCEDURE CODE			id-handoverSuccess

CRITICALITY				ignore

}

conditionalHandoverCancel	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ConditionalHandoverCancel

PROCEDURE CODE			id-conditionalHandoverCancel

CRITICALITY				ignore

}

earlyStatusTransfer		XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		EarlyStatusTransfer

PROCEDURE CODE			id-earlyStatusTransfer

CRITICALITY				ignore

}

failureIndication XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		FailureIndication

PROCEDURE CODE			id-failureIndication

CRITICALITY				ignore

}

handoverReport XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverReport

PROCEDURE CODE			id-handoverReport

CRITICALITY				ignore

}

resourceStatusReportingInitiation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE				ResourceStatusRequest

SUCCESSFUL OUTCOME				ResourceStatusResponse

UNSUCCESSFUL OUTCOME			ResourceStatusFailure

PROCEDURE CODE					id-resourceStatusReportingInitiation

CRITICALITY						reject

}

resourceStatusReporting XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResourceStatusUpdate

PROCEDURE CODE			id-resourceStatusReporting

CRITICALITY				ignore

}

mobilitySettingsChange	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE				MobilityChangeRequest

SUCCESSFUL OUTCOME				MobilityChangeAcknowledge

UNSUCCESSFUL OUTCOME			MobilityChangeFailure

PROCEDURE CODE					id-mobilitySettingsChange

CRITICALITY						reject

}

accessAndMobilityIndication XNAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE 		AccessAndMobilityIndication

PROCEDURE CODE			id-accessAndMobilityIndication

CRITICALITY 				ignore

}

retrieveUEContextConfirm	XNAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE 		RetrieveUEContextConfirm

PROCEDURE CODE			id-retrieveUEContextConfirm

CRITICALITY 				ignore

}

END

-- ASN1STOP

====================================7th Change====================================

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT RESPONSE

--

-- **************************************************************

RetrieveUEContextResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ RetrieveUEContextResponse-IEs}},

...

}

RetrieveUEContextResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-oldNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-GUAMI									CRITICALITY reject		TYPE GUAMI											PRESENCE mandatory}|

{ ID id-UEContextInfoRetrUECtxtResp				CRITICALITY reject		TYPE UEContextInfoRetrUECtxtResp				PRESENCE mandatory}|

{ ID id-TraceActivation							CRITICALITY ignore		TYPE TraceActivation								PRESENCE optional }|

{ ID id-MaskedIMEISV							CRITICALITY ignore		TYPE MaskedIMEISV									PRESENCE optional }|

{ ID id-LocationReportingInformation			CRITICALITY ignore		TYPE LocationReportingInformation				PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-NRV2XServicesAuthorized			CRITICALITY ignore	TYPE NRV2XServicesAuthorized		PRESENCE optional}|

{ ID id-LTEV2XServicesAuthorized		CRITICALITY ignore	TYPE LTEV2XServicesAuthorized		PRESENCE optional}|

{ ID id-PC5QoSParameters				CRITICALITY ignore	TYPE PC5QoSParameters			PRESENCE optional }|

{ ID id-UEHistoryInformation				CRITICALITY ignore	TYPE UEHistoryInformation						PRESENCE optional}|

{ ID id-UEHistoryInformationFromTheUE	CRITICALITY ignore	TYPE UEHistoryInformationFromTheUE				PRESENCE optional }|

{ ID id-MDTPLMNList								CRITICALITY ignore		TYPE MDTPLMNList									PRESENCE optional }|

{ ID id-UEContextRefAtSN-HORequest			CRITICALITY ignore	TYPE UEContextRefAtSN-HORequest				PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT CONFIRM

--

-- **************************************************************

RetrieveUEContextConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{RetrieveUEContextConfirm-IEs}},

...

}

RetrieveUEContextConfirm-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-oldNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSessionResourcesAdmitted-List		CRITICALITY ignore		TYPE PDUSessionResourcesAdmitted-List		PRESENCE mandatory}|

{ ID id-UEContextKeptIndicator					CRITICALITY ignore		TYPE UEContextKeptIndicator						PRESENCE optional }|

{ ID id-DRBs-transferred-to-MN					CRITICALITY ignore		TYPE DRB-List										PRESENCE optional },

...

}

END

-- ASN1STOP

====================================8th Change====================================

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

id-xnUAddressIndication											ProcedureCode ::= 5

id-uEContextRelease												ProcedureCode ::= 6

id-sNGRANnodeAdditionPreparation								ProcedureCode ::= 7

id-sNGRANnodeReconfigurationCompletion							ProcedureCode ::= 8

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 9

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 10

id-mNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 11

id-sNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 12

id-sNGRANnodeCounterCheck										ProcedureCode ::= 13

id-sNGRANnodeChange												ProcedureCode ::= 14

id-rRCTransfer													ProcedureCode ::= 15

id-xnRemoval													ProcedureCode ::= 16

id-xnSetup														ProcedureCode ::= 17

id-nGRANnodeConfigurationUpdate									ProcedureCode ::= 18

id-cellActivation												ProcedureCode ::= 19

id-reset														ProcedureCode ::= 20

id-errorIndication												ProcedureCode ::= 21

id-privateMessage												ProcedureCode ::= 22

id-notificationControl											ProcedureCode ::= 23

id-activityNotification											ProcedureCode ::= 24

id-e-UTRA-NR-CellResourceCoordination							ProcedureCode ::= 25

id-secondaryRATDataUsageReport									ProcedureCode ::= 26

id-deactivateTrace												ProcedureCode ::= 27

id-traceStart													ProcedureCode ::= 28

id-handoverSuccess												ProcedureCode ::= 29

id-conditionalHandoverCancel									ProcedureCode ::= 30

id-earlyStatusTransfer											ProcedureCode ::= 31

id-failureIndication												ProcedureCode ::= 32

id-handoverReport												ProcedureCode ::= 33

id-resourceStatusReportingInitiation							ProcedureCode ::= 34

id-resourceStatusReporting										ProcedureCode ::= 35

id-mobilitySettingsChange										ProcedureCode ::= 36

id-accessAndMobilityIndication									ProcedureCode ::= 37

id-retrieveUEContextConfirm										ProcedureCode ::= 38