3GPP TSG-RAN WG3 Meeting #110-e	R3-206191

E-meeting, 2 – 12 Nov 2020

Title: 	(TP for SON BLCR for 38.423) MRO for SN Change Failure

Source: 	Huawei

Agenda item:	10.2.1.6

Document Type:	Discussion and approval

# 1. Introduction

The SN change failure was discussed during RAN3-109 e-meeting. In this document, we will further discuss the potential MRO solutions for SN change failure cases.

# 2. Discussion

## 2.1 Definition

Based on the current definition in TR 37.816 [1], the SN changed-related failures can be categorized as follows:

-	Failures due to too late SN change triggering: an SCG failure occurs after the UE has stayed for a long period of time in the cell of the SN; the MN makes decisions for UE, making UE to establish the radio link connection in a different SN.

-	Failures due to too early SN change triggering: an SCG failure occurs shortly after a successful SN change from a source SN to a target SN or a SN change failure occurs during the SN change procedure; the MN makes decisions for UE, making UE to re-establish the radio link connection in the source SN.

-	Failures due to change to wrong SN triggering: an SCG failure occurs shortly after a successful SN change from a source SN to a target SN or a SN change failure occurs during the SN change procedure; the MN makes decisions for UE, making UE to establish the radio link connection in a SN other than the source SN or target SN.

The current definition focuses on the inter-SN mobility cases. For the MRO of SN change failure, it also refers to the intra-SN mobility cases. To make the definition more accurate, it is reasonable to modify the SN related mobility cases description to the PScell. Besides, we should also include the MN initiated SN add failure case. Due to different RRM strategy in MN after SCG failure, it is possible for the MN not to reconfigure a new PScell for the UE though there is suitable potential target PScell. We can take the definition of the three MRO connection failures as a reference.

Based on the above consideration, we have the new definition for SN change failure cases as the following:

- Redefine SN change failures:

- -	too late PScell change: an SCG failure occurs after the UE has stayed for a long period of time in the PScell; the MN finds a suitable different PScell.

- -	too early PScell change: an SCG failure occurs shortly after a successful PScell change from a source PScell to a target PScell or a PScell change failure occurs during the PScell change procedure; the MN finds the source PScell as a suitable PScell.

- -	change to wrong PScell: an SCG failure occurs shortly after a successful PScell change, e.g., after PScell addition, or from a source PScell to a target PScell or a PScell add/change failure occurs during the PScell add/change procedure; the MN finds a suitable PScell other than the source PScell or target PScell.

## 2.2 MN based detection and analysis solution

When the UE detects SCG failure, it can send the SCG Failure Information to the MN. Currently, as described in the TS 38.331 [2], there are potential measurements results in the SCG Failure Information related messaged reported by the UE.

SCGFailureInformation-IEs ::=             SEQUENCE {

failureReportSCG                              FailureReportSCG            OPTIONAL,

nonCriticalExtension                         SEQUENCE {}                  OPTIONAL

}

FailureReportSCG ::=                       SEQUENCE {

failureType                                    ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx,

synchReconfigFailureSCG, scg-ReconfigFailure,

srb3-IntegrityFailure,  spare2, spare1},

measResultFreqList                           MeasResultFreqList                       OPTIONAL,

measResultSCG-Failure                       OCTET STRING (CONTAINING MeasResultSCG-Failure)                              OPTIONAL,

...

}

To quickly recover the SCG configuration and maintain the UE experience, it is highly anticipated that the MN can make the new target PScell for the UE upon reception of the SCGFailureInformation message for both MN and SN initiated SN mobility.

If the SCG failure is caused for SN initiated SN mobility and we wait for the source SN to re-decide the new target PScell, it will bring additional latency due to the delivery of the SCGFailureInformation and the SN Modification Required messages.

Observation 1: In case of SCG failure, if MN decides the new target PScell, it can reduce the latency of reconfiguration of the SCG.

For SCG failure, MN makes the decision and initiates the new SN addition procedure if needed.

It is noted that the MN makes and initiates the SCG addition decision after receiving the SCGFailureInformation message. In this case, the UE cannot report the new target PScell related information into the SCGFailureInformation message.

Observation 2: The UE cannot provide the new target PScell related information in the SCGFailureInformation message.

Therefore, in case of the SN initiated mobility failure, it is desirable for the MN to inform the new target PScell related information to the last SN node. One possible solution is to send the new target PScell together with the SCGFailureInformation message to avoid unnecessary signalling. The last SN node can adjust the mobility parameters based on the new target PScell information.

For SN initiated SCG failure, MN sends the new PScell information together with the SCGFailureInformation to the last SN node.

As analysed in the above section, the MN can have the full information provided in the SCGFailureInformation message and know the new target PScell information. All these information can be used for the initial root cause analysis irrespective of the MN or SN initiated SN mobility failure. It is an intuitive idea to consider the MN based detection solution.

Technically speaking, the MN based detection solution can well support the SN change failure for both MN initiated and SN initiated PScell change:

- If the PScell change is triggered by MN or too late SN change failure, the MN has all information reported from the UE and the parameter setting of the triggering condition is performed by MN

- If the PScell change is triggered by the source SN, the MN has all information reported from the UE, is control of the context during the failure, and decides the new PScell for the UE

Therefore, if the SN change is triggered by MN or too late PScell change failure, the MN can decide the failure type and adjust the parameters setting.

If the SN change is triggered by the source SN, one alternative is that MN performs the initial analysis, decide the failure type (too early PScell change, or change to wrong PScell) and inform the source SN, together with UE report (e.g., SCGFailureInformation) and the new target PScell information.  The resulting solution would require a single new “SCG report” message sent from MN to the source SN, which can be SCG CHANGE REPORT. The potential signalling procedures are provided for too early PScell change failure.

Fig 1 MN based detection solution

For MN based detection solution, we leave more responsibility to the MN and bring less impact on current specification. The MN has the SCG failure information reported by UE and can better understand the complete SN change procedure. Besides, the MN can finally decide the failure type based on the new PScell decided by MN. More important, the MN based detection solution can work well for all failure cases and only one new signalling is introduced.

In addition, MN based detection solution can also apply to the pre-Rel-17 UE, which has been supported in last RAN3-109 e-meeting:

“To support pre-Rel-17 UE, in case of SCG failure, the MN shall be able to identify if the last PSCell change was initiated by itself or an SN, and which SN it was.”

Observation 3: MN based detection mechanism brings the below benefits:

- Obtain the full necessary failure information naturally

- Decide the exact failure cases(too late PScell change, too early PScell change, change to wrong PScell) and identify the too early PScell change from change to wrong PScell

- Work well for all SN change failure cases

- Only one new signalling is introduced

- Support the pre-Rel-17 UE

Taking the above analysis into consideration, we can propose that

Proposal 3: MN based detection mechanism should be supported:

- If the PScell change is triggered by MN, the MN performs initial analysis and adjust the parameter based on the SCG Failure information from UE.

- If the PScell change is triggered by the source SN:

  - the MN performs initial analysis and sends the SCG CHANGE REPORT message to the source SN;

  - The SCG CHANGE REPORT message can include the SN failure type, new target PScell information and the SCGFailureInformation container.

Accounting for different SCG change failures, we give detailed solutions as below.

Too late PScell change

For this case, the SCG failure occurs before the SN change procedure is triggered. Neither MN nor SN initiates PScell change before SCG failure. Therefore, the parameter setting of the triggering condition can be optimized by MN or the source SN. The MN could firstly perform root cause analysis. The MN receives the SCG failure information from UE and identifies that too late PScell change occurred i.e. there is no recent handover for the UE prior to the SCG failure. If the MN could determine a suitable PScell according to measurement results in the SCG failure information, the MN can optimize the parameter setting for the MN initiated PScell change.

Optionally, MN can also send SCG change report to the source SN. Consequently, the source SN can know that too late PScell change  occurred, and finally adjust the parameter setting for the SN initiated PScell change as well. This is helpful for the source SN to avoid future the failures.

Too early PScell change

In this case, the MN knows whether the PScell change is triggered by the MN or the source SN, and also knows which node is the source SN and which node is target SN. The MN determines to reconfigure the source SN as the new one according to the measurement results in the SCG failure information. Besides, the MN can know that the PScell change is uncompleted based on the SCG failure type, or the MN compares the failure time recorded by itself with one configured threshold, e.g., Tstore_UE_cntxt.  Finally, the MN can determine the SN change failure type as too early PScell change.

If the PScell change is triggered by the source SN, the MN can send the SCG change report message to the source SN. Then the source SN can adjust the parameters.

Change to wrong PScell

In this case, the MN determines the third PScell is better according to the measurement results in the SCG failure information. The MN determines to reconfigure a third PScell different from the source and the target ones as the new one according to the measurement results in the SCG failure information. Besides, the MN can know that the PScell change is uncompleted based on the SCG failure type, or the MN compares the failure time recorded by itself with one configured threshold, e.g., Tstore_UE_cntxt.  Finally, the MN can determine the PScell change failure type as change to wrong PScell.

If the PScell change is triggered by the source SN, the MN can send the SCG change report message to the source SN. Then the source SN can adjust the parameters.

Besides, to retrieve more relevant information collected at the source SN side as part of the UE context, the MN provides UE ID for the source SN to identify the UE. But in MR-DC, the MN does not know the C-RNTI of UE in the source SN. Therefore it is preferred to use the SN UE X2AP ID as a reference to the UE context in the source SN.

Proposal 4: The SCG CHANGE REPORT message could include the SN UE X2AP ID for the source SN to identify the UE.

One important part for deciding on failure type is the time threshold (i.e. the Tstore_UE_cntxt). This is key information for MN to decide whether the SCG failure occurred after ”shortly” successful SN change to the target SN. It is natural for the MN to decide the threshold for the PScell change initiated by MN.

As for the PScell change initiated by the source SN, we can see the following possible solutions:

- Option 1: Apply the same time threshold as configured in MN

- Option 2: OAM informs the time threshold to MN used for the target SN

- Option 3: the target SN(s) informs the time threshold to MN in non-UE associated signalling (e.g. in Xn setup message), or UE-associated signalling (e.g., SN Addition Request Acknowledge)

For this, we believe both option 1 and option 2 are is sufficient. However, if necessary, it is also desirable to consider option 3.

Proposal 5: It is proposed to discuss whether the signalling of the time threshold from the target SN to MN is required.

## 2.3 RACH information for SN

As stated in the previous section, there is the randomAccessProblem failure type in the SCG Failure Information. There is no additional details of the RACH procedure. A LS [1] has been sent to RAN2 to discuss the necessity of introducing the RACH report for SgNB. The RACH related information can be used to further identify the potential failure cause.

Thus, if possible, UE can include the RACH access information related to the SCG failure. In Rel-15, there was 4-step RACH. In Rel-16, 2-step RACH procedure was introduced. Currently, there can be separate 2-step RACH, 2-step fallback to 4-step RACH and separate 4-step RACH cases. If we introduce the RACH information into SCG Failure Information message, it is proposed that:

Observation 4: If RACH information is introduced into SCG Failure Information message, it should support separate 2-step RACH, 2-step fallback to 4-step RACH and separate 4-step RACH cases.

For too late PScell change, if the RACH information is included in the SCG Failure Information, the MN can forward it in the SCGFailureInformation container to the source SN. However, this will introduce additional signalling from the MN to the failed SN, e.g., the target SN in case that the target PScell isn’t connected to the source SN for too early PScell change/change to wrong PScell.

Besides, if the SN is a CU-DU architecture, the delivery of the RACH report from CU to DU should also be supported.

Observation 5: If RACH information for SN is introduced, RAN3 will introduce new additional signallings to deliver the RACH information e.g., from MN to the failed target SN in case of too early PScell change and change to wrong PScell, and from CU to DU.

## 2.4 Intra-system SN change failure in 5GC

In NR, there are NR-NR and NR-LTE DC use cases. In the TS 38.331 [2], we have defined both SCGFailureInforamtion message for NR-NR DC and SCGFailureInformationEUTRA message for NR-LTE DC respectively. Correspondingly, in LTE in case of the LTE MN node, there are defined SCGFailureInforamtion message for LTE-LTE DC and SCGFailureInformationNR message for LTE-NR DC respectively.

Firstly, the SN change failure enhancement should focus on the intra-system use cases in 5GC, in order not to bring additional complexity on the legacy LTE node.

In addition, naturally speaking, we prefer not to introduce the impact on LTE air interface. If the agreed SN change failure scheme has no impact on air interface. We can support all intra-system SN change failure cases in 5GC. Otherwise, only NR-NR and NR-LTE DC use cases should be enhanced to support SN change failure feature.

Proposal 6: Support NR-NR DC and NR-eLTE DC scenarios for SN change failure scheme.

Proposal 6a: Whether to support eLTE-NR or eLTE-eLTE DC scenarios is FFS.

# 3. Conclusion

Based on the discussion in this paper, we propose the following:

Observation 1: In case of SCG failure, if MN decides the new target PScell, it can reduce the latency of reconfiguration of the SCG.

Observation 2: The UE cannot provide the new target PScell related information in the SCGFailureInformation message.

Observation 3: MN based detection mechanism brings the below benefits:

- Obtain the full necessary failure information naturally

- Decide the exact failure cases(too late PScell change, too early PScell change, change to wrong PScell) and identify the too early PScell change from change to wrong PScell

- Work well for all PScell change failure cases

- Only one new signalling is introduced

- Support the pre-Rel-17 UE

Observation 4: If RACH information is introduced into SCG Failure Information message, it should support separate 2-step RACH, 2-step fallback to 4-step RACH and separate 4-step RACH cases.

Observation 5: If RACH information for SN is introduced, RAN3 will introduce new additional signallings to deliver the RACH information e.g., from MN to the failed target SN in case of too early PScell change and change to wrong PScell, and from CU to DU.

- Redefine SN change failures:

- -	too late PScell change: an SCG failure occurs after the UE has stayed for a long period of time in the PScell; the MN finds a suitable different PScell.

- -	too early PScell change: an SCG failure occurs shortly after a successful PScell change from a source PScell to a target PScell or a PScell change failure occurs during the PScell change procedure; the MN finds the source PScell as a suitable PScell.

- -	change to wrong PScell: an SCG failure occurs shortly after a successful PScell change, e.g., after PScell addition, or from a source PScell to a target PScell or a PScell add/change failure occurs during the PScell add/change procedure; the MN finds a suitable PScell other than the source PScell or target PScell.

For SCG failure, MN makes the decision and initiates the new SN addition procedure if needed.

For SN initiated SCG failure, MN sends the new PScell information together with the SCGFailureInformation to the last SN node.

The SCG CHANGE REPORT message could include the SN UE X2AP ID for the source SN to identify the UE.

It is proposed to discuss whether the signalling of the time threshold from the target SN to MN is required.

Support NR-NR DC and NR-eLTE DC for SN change failure scheme.

- Proposal 6a: Whether to support eLTE-NR or eLTE-eLTE DC scenarios is FFS.

# 4. Reference

[1] R3-205662, LS to RAN2 on RACH report for SgNB

# Annex 1– TP on TS 38.423

Start of the first change

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| SN change report | SN CHANGE REPORT |

Start of the next change

### 8.4.z	SN change Report

#### 8.4.z.1	General

The purpose of the SN change Report procedure is to transfer mobility related information between NG-RAN nodes.

The procedure uses non UE-associated signalling.

#### 8.4.z.2	Successful Operation

Figure 8.4.z.2-1: SN Change Report, successful operation

An M-NG-RAN node initiates the procedure by sending an SN CHANGE REPORT message to S-NG-RAN node. By sending the message M-NG-RAN node indicates to S-NG-RAN node that a SN mobility-related problem was detected.

If the SN change Report Type IE is set to "too early PScell change" or "change to wrong PScell", then the M-NG-RAN node indicates to S-NG-RAN node that, following a successful PScell change from a source PScell to a target PScell node, a SCG failure occurred and the UE established connection either at the original cell of PScell (too early PScell change), or at another cell (change to wrong PScell). If the SN change Report Type IE is set to "too late PScell change", then the M-NG-RAN node indicates to S-NG-RAN node that, following a before PScell change trigger from a source PScell to a target PScell, a SCG failure occurred.

#### 8.4.z.3	Unsuccessful Operation

Not applicable.

#### 8.4.z.4	Abnormal Conditions

Void.

Start of the next change

#### 9.1.3.z	SN CHANGE REPORT

This message is sent by the M-NG-RAN node to report a PScell change failure event to S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| CHOICE SN change Report Type | M |  |  |  |  |  |
| > too early PScell change |  |  |  |  |  |  |
| >>Source PScell CGI | M |  | NR CGI 9.2.2.7 | CGI of source PScell for SN change procedure | YES | ignore |
| >>Failure PScell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | CGI of target PScell for SN change procedure | YES | ignore |
| > too late PScell change |  |  |  |  |  |  |
| >>Source PScell CGI | M |  | NR CGI 9.2.2.7 | CGI of source PScell for SN change procedure | YES | ignore |
| >>Target PScell CGI |  |  | NG-RAN CGI 9.2.2.x | CGI of the new suitable PScell | YES | ignore |
| > change to wrong PScell |  |  |  |  |  |  |
| >>Source PScell CGI | M |  | NR CGI 9.2.2.7 | CGI of source PScell for SN change procedure | YES | ignore |
| >>Failure PScell CGI | M |  | NG-RAN CGI 9.2.2.x | CGI of target PScell for SN change procedure | YES | ignore |
| >>Target PScell CGI | M |  | NG-RAN CGI 9.2.2.x | CGI of the new suitable PScell | YES | ignore |
| UE SCGfailureinformation Report Container | O |  | OCTET STRING | The received SCGFailureInformation or SCGFailureInformationEUTRA message defined in TS 38.331 [10] from UE. | YES | ignore |

End of the last change

# Annex 2– TP on TS 38.300

Start of the first change

#### 15.5.2.x	SN change failure

##### 15.5.2.x.1	General

For analysis of PScell add/change failures, the UE makes the SCG Failure Information to the MN. The MN stores the PScell add/change related information and performs the initial analysis. The MN may transfer the SN Change Report to the source SN if the PScell change was initiated by the source SN (see the section 13.x in TS 37.340 [21]).

End of the last change