3GPP TSG-RAN WG3 Meeting #110-e	R3-206199

E-meeting, 2 - 12 November, 2020

Agenda item:	9.3.7.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Further correction on broadcast PLMN information on F1

Document for:	Discussion and Decision

# 1	Introduction

Updated semantics for PLMN information was agreed for Rel-15 at RAN3#107bis-e for X2AP, XnAP and F1AP (R3-202814). Furthermore, in order to further clarify the interpretation of "broadcast PLMNs", RAN3 made agreements relative to the the MIB-only scenario at RAN3#108-e for X2AP (R3-204528) and XnAP (R3-204387) as well as for stage 2 (R3-204389).

However focus for these discussions was mainly on X2 and Xn. In this paper we look into a F1 scenario which was not analysed in detail so far.

# 2	Discussion

As per current semantics description, the F1AP Served Cell Information IE (TS 38.473 clause 9.3.1.10) contains cell configuration information of a cell in the gNB-DU, and the initial intention of this IE was therefore to inform the gNB-CU about cell information configured by the OAM in the gNB-DU. Among this information is the Served PLMNs IE (enhanced with the Extended Served PLMNs List IE) and the Broadcast PLMN Identity Info List IE with semantics referring to SIB1 broadcast as shown here:

| Served PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB 1 associated to the NR Cell Identity in the NR CGI IE | - |  |
| --- | --- | --- | --- | --- | --- | --- |

| Broadcast PLMN Identity Info List |  | 0..<maxnoofBPLMNsNR> |  | This IE corresponds to the PLMN-IdentityInfoList IE in SIB1 as specified in TS 38.331 [8]. All PLMN Identities and associated information contained in the PLMN-IdentityInfoList IE are included and provided in the same order as broadcast in SIB1. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Absence of broadcast of PLMN information in the SN for the case where "the SN does not broadcast system information other than radio frame timing and SFN" was handled RAN3#108-e. However in the NG-RAN split architecture broadcast of PLMN info also depends on core network support of the PLMN (F1AP):

"For NG-RAN, the gNB-CU may include Available PLMN List IE, and optionally also Extended Available PLMN List IE in the F1 SETUP RESPONSE message, if the available PLMN(s) are different from what gNB-DU has provided in F1 SETUP REQUEST message, gNB-DU shall take this into account and only broadcast the PLMN(s) included in the received Available PLMN list(s)."

The available PLMNs as seen by the gNB-CU are typically the PLMNs served by the core networks to which the gNB-CU is connected, and the NG-RAN cell shall as per the text above not broadcast PLMNs not available in the gNB-CU. By extrapolation of earlier decision taken by RAN3 (TS 37.340 CR#0178 Correction of NG connection in MR DC), we believe this also applies to cells deployed for SCG-only operation.

Proposal 1: RAN3 to confirm that an NG-RAN cell deployed for SCG-only operation shall not broadcast PLMN info if the PLMN is not available in the gNB-CU.

If the condition in the procedural text cited above applies, and if we consider the current semantics descriptions for the Served PLMNs IE and the Broadcast PLMN Identity Info List IE, the current specification appears unclear relative to whether the gNB-DU should reflect the updated broadcast situation in these IEs during the following gNB-DU Configuration Update procedure. On one hand, the OAM configuration has not changed, but on the other hand the SIB1 content has changed.

Answer to this question can be found by analysing the role of these two IEs. The Served PLMNs IE was introduced to convey configured information, and for this IE the gNB-DU Configuration Update procedure should only be initiated if the configured information was changed. This will ensure that the gNB-CU can inform the gNB-DU when any temporarily unavailable PLMN again becomes available.

Proposal 2: RAN3 to confirm that the Served PLMNs IE conveys OAM configured information, and is not updated if a configured PLMN is not available in the gNB-CU (and hence not broadcast).

However the situation is different for the Broadcast PLMN Identity Info List IE which was introduced to inform the gNB-CU about information sent to the UE e.g. for the purpose of area specific SIB. The information is therefore intended to represent actual broadcast information, and we believe that the gNB-DU should therefore trigger a gNB-DU Configuration Update procedure including an updated Broadcast PLMN Identity Info List IE when the SIB1 content is changed.

Proposal 3: RAN3 to confirm that the Broadcast PLMN Identity Info List IE conveys actual broadcast information, and therefore should be updated by means of a gNB-DU Configuration Update procedure when the SIB1 content is changed.

In order to reflect the above in the specification and hence enable differentiated handling of the Served PLMNs IE and the Broadcast PLMN Identity Info List IE, we have submitted an F1AP CR to the present meeting in [1].

Proposal 4: RAN3 to discuss and agree the F1AP CRs submitted in [1] (Rel-15) and [2] (Rel-16).

# 3	Conclusion

We have made the following proposals:

Proposal 1: RAN3 to confirm that an NG-RAN cell deployed for SCG-only operation shall not broadcast PLMN info if the PLMN is not available in the gNB-CU.

Proposal 2: RAN3 to confirm that the Served PLMNs IE conveys OAM configured information, and is not updated if a configured PLMN is not available in the gNB-CU (and hence not broadcast).

Proposal 3: RAN3 to confirm that the Broadcast PLMN Identity Info List IE conveys actual broadcast information, and therefore should be updated by means of a gNB-DU Configuration Update procedure when the SIB1 content is changed.

Proposal 4: RAN3 to discuss and agree the F1AP CRs submitted in [1] (for Rel-15) and [2] (for Rel-16).

# References

[1]		R3-206200, Further correction on PLMN info in Served Cell Information IE, Rel-15 CR to TS 38.473, Nokia, Nokia Shanghai Bell

[2]		R3-206201, Further correction on PLMN info in Served Cell Information IE, Rel-16 CR to TS 38.473, Nokia, Nokia Shanghai Bell