3GPP TSG-RAN WG3 Meeting #110e	R3-206220

# Online Meeting, 2nd – 12th November 2020

Agenda Item:	9.3.3

Source:	Ericsson

Title: 	AS re-keying and Emergency Fallback in the UE Context Modification procedure

Document for:	Discussion and Approval

# 1	Information

At the early RAN3 meetings, we discussed the AS re-keying in the UE context modification procedure, and how to handle when the AS re-keying and the Emergency Fallback are requested in the same message [1].

A LS is sent to SA2 and SA3 in [2]. We got reply from SA3 in [3]. We also got rely from SA2 in [4].

This paper aims to bring the conclusions on this topic.

# Discussion

SA3 has clarifies their view:

Q1. RAN3 respectfully asks SA3 to feedback whether it is acceptable for the network to give up the AS re-keying procedure and only initiate the emergency fallback procedure when the two procedures collide. SA3 Answer: It is acceptable to SA3, for the network to give up the AS re-keying procedure and only initiate the emergency fallback procedure when the two procedures collide.

SA2 has clarified their view:

SA2 discussed the behaviour of NG-RAN node if it receives one NGAP UE CONTEXT MODIFICATION REQUEST message that includes both the Security Key IE and the Emergency Fallback Indicator IE. From SA2 point of view, the AS re-keying procedure and the Emergency Fallback procedure are two different procedures and may collide with a very low probability. The Emergency Fallback shall have the highest priority and also require a strict service delay, so to guarantee the success of Emergency Fallback, the Emergency Fallback shall take precedence over AS rekeying if they collide. Consequently, SA2 agrees with the feedback of SA3 to give up the AS re-keying procedure and only initiate the emergency feedback procedure when the two procedures collide. Accordingly, SA2 would like to provide the following feedback: To SA2: ACTION: RAN3 respectfully asks SA2 to feedback whether the case that both the Security Key IE and the Emergency Fallback Indicator IE are included within one NGAP UE CONTEXT MODIFICATION REQUEST message is abnormal or not. SA2 Answer: When the AS re-keying procedure and the Emergency Fallback procedure collides, the AMF gives up the AS re-keying procedure and only initiates the emergency fallback procedure. If AMF includes both the Security Key IE and the Emergency Fallback Indicator IE within one NGAP UE CONTEXT MODIFICATION REQUEST message, NG-RAN takes it as abnormal case. SA2 approved attached CR to clarify this scenario.

In our view, when Emergency Fallback is request, NG-RAN node could perform release and redirect, inter or intra system handover. If the AS-rekeying is requested in the same message, it would be very beneficial to allow NG-RAN node to skip the AS-rekeying handling and only handle the Emergency Fallback.

The SA2 and SA3 response are align with our view to prioritize the emergency fallback handling.

We propose to specify the handling in abnormal case. To clarify, in this case, NG-RAN node could ignore the AS Rekeying and only perform Emergency fallback handling as described in the successful operation.

Proposal 1: RAN3 to agree to the NG-RAN node shall only handle the Emergency Fallback in case it is requested together with AS Re-keying.

Proposal 2: RAN3 to agree to capture in the abnormal case the above;

The CR is submitted in [5] and [6].

# 3	Proposals

Proposal 1: RAN3 to agree to the NG-RAN node shall only handle the Emergency Fallback in case it is requested in together with AS Re-keying.

Proposal 2: RAN3 to agree to capture in the abnormal case the above;

# 4	Reference

[1]	R3-202112	AS re-keying and Emergency Fallback in the UE Context Modification procedure, Ericsson

[2]	R3-202833	LS on AS rekeying handling, RAN3

[3]	R3-203938	LS reply to RAN WG3 LS on AS rekeying handling, SA3

[4]	S2-2005909	Reply LS on AS rekeying handling, SA2

[5]	R3-206224	Clarification of AS re-keying in the UE Context Modification procedure, Ericsson

[6]	R3-206222	Clarification of AS re-keying in the UE Context Modification procedure, Ericsson