# 3GPP TSG-RAN WG3 #110-e						R3-206262

# 2 – 12 November 2020

# Title:	[DRAFT] Reply LS on SA WG2 assumptions from conclusion of study on architecture aspects for using satellite access in 5G

# Response to:	Reply LS on SA WG2 assumptions from conclusion of study on architecture aspects for using satellite access in 5G (R3-20xxxx/S2-2008307)

# Release:	Release 17

# Work Item:	NR_NTN_solutions, 5GSAT_ARCH

Source:	Qualcomm Incorporated [to be RAN3]

To:	SA2

Cc:	RAN2, SA3-LI, SA5

Contact Person:

Name:	Luis Lopes

Tel. Number:

E-mail Address:	llopes@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN3 thanks SA2 for their reply LS in R3-20xxxx/S2-2008307.

RAN3 understands that there are scenarios where it is important that the cell ID received by the CN should always correspond to a fixed geographical area. RAN3 has started considering how to associate the cell ID sent to the CN to a fixed geographical area and also whether it is possible to provide TN level granularity in some instances.

From an initial analysis, RAN3 thinks that suitable fixed-earth CGI definitions can be configured in RAN and CN, and that the required mapping for ULI (and other functionality involving RAN-CN interaction) may be performed by the RAN, without impacts on the CN. If this is done, the NGAP signalling should use mapped cell IDs in most or all procedures.

Regarding the mapping itself, further study is needed on the level of granularity that may be attained in RAN. Since the information available may not always be of the same precision, a possible option would be to define two (or more) cell layers with different granularity, such that the RAN can use a cell ID from the layer that represents the best granularity available at the time of the respective message.

RAN3 will also continue to study resulting impacts or requirements in RAN functions, procedures and signalling as applicable to this use case.

2. Actions:

To SA WG2.

ACTION: 	RAN3 kindly asks SA2 to take the above information into account.

3. Date of Next RAN3 Meetings:

RAN3#111-e	January 2020	Electronic meeting