3GPP TSG-RAN Meeting # 110-e	R3-206266

# 2 - 12 November 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.410 | CR | 0029 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of NAS Node Selection Function |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions |  |  |  |  |  | Date: |  |  | 2020-10-02 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to TS 38.300, it is not precluded that a cell served by a gNB does not broadcast the PLMN ID included in the Global gNB ID. With that statement, it seems possible that different cells in the same gNB broadcast different PLMNs, and the same logical gNB will have connectivity to different CNs (i.e. similar to MOCN at NG level, but there may be no sharing of the air interface resource). This scenario may happen for NTN where a single gNB provides coverage in multiple countries. Under this arrangement, it is important to ensure that a UE accessing the network via a cell of country A does not use the CN of country B, in any scenario (including when the UE provides a 5G-S-TMSI, or a GUAMI). This also addresses a requirement agreed by SA2 in S2-2008311 (RAN selects an AMF/AMF set when the UE attempts to establish an RRC connection with an AMF serving a different country to where the UE is located) with the exception of the case where a cell of country A has coverage in country B. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify that the AMF (as determined by NNSF) shall support the selected PLMN indicated by the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear that the RAN should check the selected PLMN even when the 5G-S-TMSI or GUAMI of the registered AMF are available, and the AMF can be identified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.7	NAS Node Selection function

The interconnection of NG-RAN nodes to multiple AMFs is supported in the 5GS architecture.

Therefore, a NAS node selection function is located in the NG-RAN node to determine the AMF association of the UE, based on the UE's temporary identifier, which was assigned to the UE by the AMF. When the UE's temporary identifier has not been yet assigned or is no longer valid the NG-RAN node may instead take into account slicing information to determine the AMF. In all cases, the AMF shall support the selected PLMN as indicated by the UE.

This functionality is located in the NG-RAN node and enables proper routing via the NG interface. On NG, no specific procedure corresponds to the NAS Node Selection Function.