3GPP TSG-RAN3 Meeting #110-e	R3-206298

E-meeting, 2-13 November 2020

Title: 	Consider on Multi-Hop Performance: QoS, Latency, Fairness

Source: 	CATT

Agenda item:	13.3.2

Document Type:	Discussion

1. Introduction

During the RAN3#109-e meeting, the issues on multi-hop performance, including QoS, latency and fairness have been discussed in [1]. The issues identified by companies may need pending RAN2 progress.

After the last RAN2#111-e meeting, there is an email discussion#902 to identify the technical issues to address multi-hop performance issues [2]. During the email discussion, companies identified the following issues which have RAN3 impact.

Topology-wide fairness:

- Issue 1: To discuss the definition of topology-wide fairness with QoS.

- Issue 2: To discuss how to achieve the topology-wide fairness.

Multi-hop latency:

- Issue 1: To discuss whether HbH PDB should be provided by donor-CU to IAB node.

In this document, we will further discuss the above issues which related with RAN3.

2. Discussion

  - 2.1 Topology-wide fairness

Based on the email discussion#902[1], the issues on topology-wide fairness have been discussed. Topology-wide fairness should be distinguished with load balancing. Load balancing focus on IAB-node that should schedule the child IAB-node and UE fairly. Topology-wide fairness requests that UE with different hops have the same QoS for the same service regardless of where a UE attaches to the IAB network.

In the email discussion#902[1], most of companies agree that topology-wide fairness cannot be met based on the existing R16 IAB baseline. Topology-wide fairness should be focused on UE experience rather than IAB-node. To define the Topology-wide fairness, in our point, any new end-user QoS metric is necessary to be defined based on the existing 5G QoS framework. Topology-wide fairness requires that UE experience the same QoS for the same service regardless ‎of the number of hops between the UE and the Donor CU‎.‎

Proposal 1: ‎Topology-wide fairness requires that UE experience the same QoS for the same service regardless of the number of hops between the UE and the Donor CU.

In R16, IAB-node doesn’t have the enough information to achieve the topology-wide fairness, such as the number of hops from donor-CU to UE. The latency will be different for the same service in the upstream or downstream for different UEs with different number of hops between the UE and the Donor CU. In R16, IAB-node doesn’t have the information, such as the number of remaining hops in the upstream or downstream. Thus, the traffic QoS for UE is difficult to be guaranteed.

For example, there are two UEs, i.e., UE1 and UE2. The number of hops between UE1 and donor-CU is 5, while the number of hops between UE1 and donor-CU is 3. Since each hop in the network introduces additional latency in the upstream or downstream. The latency in the upstream or downstream for UE1 is larger than UE2’s. In order to ensure the topology-wide fairness, the latency should be the same for the same service in the upstream or downstream for different UEs with different number of hops between the UE and the Donor CU. Thus IAB-node should be provided‎ with topology information‎, e.g., the number of remaining hops in the upstream or downstream, to prior scheduling the data transmission with larger number of remaining hops.

Proposal 2: ‎IAB-node is provided with topology information by donor-CU, e.g., the number of remaining hops in the upstream or downstream.

  - 2.2 Multi-hop latency

In R16, whether IAB-node can discard the packets which PDB cannot be met is left into implementation. If IAB-node does not discard the expired packets, it will have negative impact on the radio resource efficiency. Thus it’s necessary to discuss the mechanism on HbH PDB management in R17.

Observation 1: Discarding the expired packets at intermediate IAB-node can improve the radio resource efficiency.

If the expired packets continually to be sent, it would increase the possibility on congestion. Thus, discarding the expired packets also has benefits for congestion mitigation.

Observation 2: Discarding the expired packets also has benefits for congestion mitigation.

To achieve the expired packets discarding, the IAB node should have acknowledge of HbH PDB, so that the IAB node can discard the expired packets due to HbH PDB limitation. Thus, the HbH PDB can be provided by donor-CU.

Proposal 3: IAB-node is provided with HbH PDB by donor-CU.

3. Conclusion

Based on the discussion in this paper, we propose the following:

Observation 1: Discarding the expired packets at intermediate IAB-node can improve the radio resource efficiency.

Observation 2: Discarding the expired packets also has benefits for congestion mitigation.

Proposal 1: ‎Topology-wide fairness requires that UE experience the same QoS for the same service regardless of the number of hops between the UE and the Donor CU.

Proposal 2: ‎IAB-node is provided with topology information by donor-CU, e.g., the number of remaining hops in the upstream or downstream.

Proposal 3: IAB-node is provided with HbH PDB by donor-CU.

4. Reference

- R3-205742‎, CB: # 10_IAB_MultiHopPerf - Summary of email discussion, CATT.

- R2-2009073, [Post111-e][902][eIAB] Enhancements to improve topology-wide fairness, multi-hop latency and congestion mitigation, Samsung.