3GPP TSG-RAN WG3 Meeting #110-e	R3-206419

E-meeting, 2 – 12 Nov 2020

Title:	[DRAFT] LS on minimizing data loss during MBS mobility

Response to:

Release:	Release 17

Work Item:	NR_MBS-Core, FS_5MBS

Source:	Huawei [will be RAN3]

To:	SA2, RAN2

Cc:

Contact person:	Yan Wang

Wangyan7@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1	Overall description

RAN3 have some discussion on minimizing data loss during MBS mobility. It is agreed to use synchronized DL PDCP SN, and therefore it is needed for the gNB to receive the same SN for the same MBS data packet from the core network.

For IP Multicast method, gNBs are receiving the same packet from the same IP address, and the same SN will be received by the gNBs by default.

For shared GTP-U Tunnel method, it is needed for the UPF to put the same SN in the same MBS data packet when delivering them to different gNBs.

There are several alternatives to be used:

-	Reuse the Sequence Number in GTP-U header

-	Reuse the DL QFI Sequence Number in "GTP-U Container" in GTP-U Extension Header

-	Introduce a new SN in "GTP-U Container" in GTP-U Extension Header

RAN3 would appreciate feedback from SA2 and RAN2 feedback on the above.

2	Actions

To SA2 and RAN2 group:

ACTION:

SA2 respectfully asks SA2 and RAN2 to take the above information into account and provide feedback on the above alternatives.

3	Dates of next RAN3 meetings

TSG-RAN3 Meeting #111e	Jan 25 – Feb 02, 2021			E-Meeting

TSG-RAN3 Meeting #112e	May 15 – May 28, 2021			E-Meeting