3GPP TSG-RAN3 Meeting #110-e	R3-206432

E-meeting, 2 - 12 November 2020

Title: 	Solutions and Flow charts of network slice service continuity

Source: 	Huawei

Agenda item:	17.2

Document Type:	Discussion and Decision

1. Introduction

Last RAN3 meeting #109-e agreed the call flows of the potential solutions to maintain the network slice service continuity during handover.

This contribution describes potential solutions with or without CN involvement and continues to discuss the slice re-mapping call flows.

2. Discussion

2.1 Slice re-mapping with/without CN involvement

Fig. 1: Slice re-mapping solutions: (a) with CN; (b) without CN

- Given the agreed Scenario 2 in [1], we give two possible slice re-mapping mechanisms distinguished by whether the CN is involved. Fig. 1 shows the re-mapping scenario, where UE’s ongoing PDU session is re-mapped from Slice #1 in S-gNB to Slice #2 in T-gNB.

Fig. 1(a) shows the re-mapping solution where both the RAN and CN parts are involved. No matter which part makes the slice re-mapping decision, extra CN procedures will be introduced.

Fig. 1(b) shows the re-mapping solution where only the RAN part is involved. The anchor node of UE’s ongoing PDU session is still S-gNB, i.e., the CN part of the PDU session is still within Slice #1. The RAN part of the PDU session is re-mapped to Slice #2 in the T-gNB. The UL/DL CN traffic is relayed between the S-gNB and T-gNB via the Xn tunnel.

- Proposal 1: Capture the above solutions (i.e., with/without CN involvement) in TR 38.832 for inter-RA mobility.

2.2 Slice Remapping decision for NG based handover

- According to TS 23.502 Section 4.9.1.3[2], during the NG-based handover, the 5GC will do the initial admission control before sending the Handover Request message to the T-gNB. After receiving the Handover Request message from the AMF, the T-gNB will perform further admission control and feedback the admission results in the Handover Request Acknowledge message sent to the AMF. Therefore, the PDU sessions accepted to be established during the NG-based handover is jointly determined by the 5GC and T-gNB. Similar mechanisms can be applied to the slice re-mapping decision making.

- The following clause is the proposed flow chart of the joint slice re-mapping decision-making mechanism of the 5GC and T-gNB.

2.2.1 Slice Remapping decision in 5GC and target gNB at NG based handover

Fig. 2: Slice re-mapping/fallback determined by the AMF and T-gNB

- The S-gNB sends the HANDOVER REQUIRED message to the AMF.

- If the UE’s ongoing slice(s) is not supported by the T-gNB, the AMF may make the initial slice re-mapping/fallback decision and include the decision in the HANDOVER REQUEST message to the T-gNB.

- Considering the initial slice re-mapping/fallback decision made by the AMF, if the UE’s ongoing or re-mapped/fallback slice(s) is rejected in the target gNB, based on the slice re-mapping policy described in section 6.2.X, the T-gNB shall include the further re-mapped/fallback decision in the HANDOVER REQUEST ACKNOWLEDGE message to the AMF.

- The AMF may send the slice re-mapping/fallback decision to the S-gNB through the HANDOVER COMMAND message.

- Proposal 2: Capture the above flow chart for NG-based handover into the TR 38.832.

2.3 Slice Remapping decision for MR-DC case

The following clauses provide the flow charts of the solutions for the proposed DC scenarios[3], i.e., slice resource shortage and non-support slice. Generally, dependent on the specific use cases, slice re-mapping/fallback can be performed by the MN or/and SN.

2.2.1 Slice Remapping decision in SN

Fig. 3: Slice re-mapping/fallback determined by the SN

- The MN sends the SN Addition Request message to the SN.

- If the UE’s ongoing slice(s) is rejected by the SN, based on the slice re-mapping policy described in section 6.2.X, the SN makes the slice re-mapping/fallback decision. The SN shall include the slice re-mapping/fallback decision in the SN Addition Request Acknowledge message sent to the MN.

- The MN shall send the slice re-mapping/fallback decision to the AMF through the PDU Session Modification Indication message.

- The AMF responds the PDU Session Modification Confirmation message.

2.2.2 Slice Remapping decision in MN

Fig. 4: Slice re-mapping/fallback determined by the MN

- The MN may make the slice re-mapping/fallback decision and include the decision in the SN Addition Request message to the SN.

- The SN may confirm the slice re-mapping/fallback decision made by MN in the SN Addition Request Acknowledge message to the MN.

- The MN shall send the slice re-mapping/fallback decision to the AMF through the PDU Session Modification Indication message.

- The AMF responds the PDU Session Modification Confirmation message.

- Proposal 3: For dual connectivity scenarios, capture the above solutions into the TR 38.832.

3. Conclusion

Based on the discussion in this paper, we propose the following:

- Proposal 1: Capture the above solutions (i.e., with/without CN involvement) in TR 38.832 for inter-RA mobility.

- Proposal 2: Capture the above solution for NG-based handover into the TR 38.832.

- Proposal 3: For dual connectivity scenarios, capture the above solutions into the TR 38.832.

4. Reference

- R3-205783, TP for TR38.382 on service continuity for slicing.

- 3GPP TS 23.502 V16.6.0, Procedures for the 5G System (5GS).

- R3-206430, Further discussion on the scenarios for network slice service continuity.

Annex 1– TP

6.2	Solutions

Editor Note: Capture the solutions for the scenario and issue.

6.2.m Candidate solutions with/without CN involvement

Fig. 1: Slice re-mapping solutions: (a) with CN impact; (b) without CN impact

- For scenario 2, there are two possible slice re-mapping solutions depending on whether the CN is involved. Fig. 1 shows the re-mapping scenario, where UE’s ongoing PDU session is re-mapped from Slice #1 in S-gNB to Slice #2 in T-gNB.

Fig. 1(a) shows the re-mapping solution where both the RAN and CN parts are involved. No matter which part makes the slice re-mapping decision, extra CN procedures will be introduced.

Fig. 1(b) shows the re-mapping solution where only the RAN part is involved. The anchor node of UE’s ongoing PDU session is still S-gNB, i.e., the CN part of the PDU session is still within Slice #1. The RAN part of the PDU session is re-mapped to Slice #2 in the T-gNB. The UL/DL CN traffic is relayed between the S-gNB and T-gNB via the Xn tunnel.

6.2.Y Slice Re-mapping Message Sequence Charts

Editor’s note: Feasibility of this solution at system level requires further work including checking with SA2.

6.2.Y.1 Slice Remapping decision in target gNB at Xn based handover

Fig. 1: Slice re-mapping/fallback determined by the T-gNB

- The S-gNB sends the HANDOVER REQUEST message to the T-gNB.

- If the UE’s ongoing slice(s) is rejected in the target gNB, based on the slice re-mapping policy described in section 6.2.X, the T-gNB makes the slice re-mapping/fallback decision. The T-gNB may send the slice re-mapping/fallback decision in the HANDOVER REQUEST ACKNOWLEDGE message to the S-gNB.

- The T-gNB shall send the slice re-mapping/fallback decision to the AMF through the PATH SWITCH REQUEST message.

- The AMF responds the PATH SWITCH REQUEST ACKNOWLEDGE message. The AMF may reject the PDU sessions in the PDU Session Resource Released List IE.

6.2.Y.2 Slice Remapping decision in target gNB at NG based handover

Fig. 2: Slice re-mapping/fallback determined by the T-gNB

- The S-gNB sends the HANDOVER REQUIRED message to the AMF.

- The AMF sends the HANDOVER REQUEST message to the T-gNB.

- If the UE’s ongoing slice(s) is rejected in the target gNB, based on the slice re-mapping policy described in section 6.2.X, the T-gNB shall include the re-mapped/fallback decision in the HANDOVER REQUEST ACKNOWLEDGE message to the AMF.

- The AMF may send the slice re-mapping/fallback decision to the S-gNB through the HANDOVER COMMAND message.

6.2.Y.3 Slice Remapping decision in 5GC at NG based handover

Fig. 3: Slice re-mapping/fallback determined by the AMF

- The S-gNB sends the HANDOVER REQUIRED message to the AMF.

- If the UE’s ongoing slice(s) is not supported by the T-gNB, the AMF may make the slice re-mapping/fallback decision and include the decision in the HANDOVER REQUEST message to the T-gNB.

- The T-gNB responds to the AMF through the HANDOVER REQUEST ACKNOWLEDGE message.

- The AMF may send the slice re-mapping/fallback decision to the S-gNB through the HANDOVER COMMAND message.

6.2.Y.4 Slice Remapping decision in 5GC and target gNB at NG based handover

Fig. 4: Slice re-mapping/fallback determined by the AMF and T-gNB

- The S-gNB sends the HANDOVER REQUIRED message to the AMF.

- If the UE’s ongoing slice(s) is not supported by the T-gNB, the AMF may make the initial slice re-mapping/fallback decision and include the decision in the HANDOVER REQUEST message to the T-gNB.

- Considering the initial slice re-mapping/fallback decision made by the AMF, if the UE’s ongoing or re-mapped/fallback slice(s) is rejected in the target gNB, based on the slice re-mapping policy described in section 6.2.X, the T-gNB shall include the further re-mapped/fallback decision in the HANDOVER REQUEST ACKNOWLEDGE message to the AMF.

- The AMF may send the slice re-mapping/fallback decision to the S-gNB through the HANDOVER COMMAND message.

6.2.Y.5 Slice Remapping decision in SN for MR-DC case

Fig. 5: Slice re-mapping/fallback determined by the SN

- The MN sends the SN Addition Request message to the SN.

- If the UE’s ongoing slice(s) is rejected by the SN, based on the slice re-mapping policy described in section 6.2.X, the SN makes the slice re-mapping/fallback decision. The SN shall include the slice re-mapping/fallback decision in the SN Addition Request Acknowledge message sent to the MN.

- The MN shall send the slice re-mapping/fallback decision to the AMF through the PDU Session Modification Indication message.

- The AMF responds the PDU Session Modification Confirmation message.

6.2.Y.6 Slice Remapping decision in MN for MR-DC case

Fig. 6: Slice re-mapping/fallback determined by the MN

- The MN may make the slice re-mapping/fallback decision and include the decision in the SN Addition Request message to the SN.

- The SN may confirm the slice re-mapping/fallback decision made by MN in the SN Addition Request Acknowledge message to the MN.

- The MN shall send the slice re-mapping/fallback decision to the AMF through the PDU Session Modification Indication message.

- The AMF responds the PDU Session Modification Confirmation message.