3GPP TSG-RAN WG3 #110-e     					                                                    R3-206657

- 2 - 12 Nov 2020

Online

Agenda item:	10.2.1.5

Source: 	ZTE

Title: 	TP for BL CR 38.423 on Load Balancing Enhancements

Document for:	Approval

Introduction

This contribution provides a TP for BLCR to 38.423. i.e., add slice level  PRB and DRB usage as load metrics over Xn.

Text Proposal for 38.423

-------------------------------------------------Start of the first change-------------------------------------------------------------------

9.1.3.18	RESOURCE STATUS REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested measurement according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestStoporPartialStoporAdd |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, add, …) | Type of request for which the resource status is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BIT STRING (SIZE(32)) | Each position in the bitmap indicates measurement object the NG-RAN node2 is requested to report. First Bit = PRB Periodic, Second Bit = TNL Capacity Ind Periodic, Third Bit = Composite Available Capacity Periodic, Fourth Bit = Number of Active UEs, Fifth Bit = RRC connections.Sixth Bit = Slice PRB Periodic, Seventh Bit = Slice Level DRB Periodic. Other bits shall be ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. |  |  |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  |  |  |
| >>Cell ID | M |  | 9.2.3.25 |  |  |  |
| >>SSB To Report List |  | 0..1 |  | SSB list to which the request applies. | YES | Ignore |
| >>>SSB To Report Item |  | 1 .. < maxnoofSSBAreas> |  |  | EACH | Ignore |
| >>>>SSB-Index | M |  | INTEGER (0..,63..) |  |  |  |
| >>Slice To Report List |  | 0..1 |  | S-NSSAI list to which the request applies. | YES | ignore |
| >>>Slice To Report Item |  | 1 .. < maxnoofBPLMNs > |  |  | EACH | ignore |
| >>>>PLMN Identity | M |  | 9.3.1.14 | Broadcast PLMN |  |  |
| >>>>S-NSSAI List |  | 1 |  |  | EACH | ignore |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  |  |  |
| >>>>S-NSSAI | M |  | S-NSSAI 9.3.1.38 |  |  |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting of PRB Periodic, TNL Capacity Ind Periodic, Composite Available Capacity Periodic. Also used as the averaging window length for all measurement object if supported. |  |  |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop" or "add". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNs | Maximum no. of PLMN Ids.broadcast in a cell. Value is 12. |

-------------------------------------------------End of the first change-------------------------------------------------------------------

-----------------------------------------------Start of the second change-----------------------------------------------------------------

9.2.2.55	Slice Available Capacity

The Slice Available Capacity IE indicates the amount of resources that are available relative to the maximum pre-configured or reserved resource for a network slice or a shared resource block in a cell of the NG-RAN node.. The Slice Capacity Value Downlink IE and the Slice Capacity Value Uplink IE can be weighted according to the ratio of the corresponding cell capacity class values contained in the Composite Available Capacity Group IE, if available.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice Available Capacity |  | 1..< maxnoofBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.3.1.14 | Broadcast PLMN |
| >S-NSSAI Available Capacity List |  | 1 |  |  |
| >>S-NSSAI Available Capacity Item | M | 1 .. < maxnoofSliceItems> |  |  |
| >>>S-NSSAI |  |  | 9.3.1.38 |  |
| >>>Slice Available Capacity Value Downlink | O |  | INTEGER (0..100) | Value 0 shall indicate no available capacity, and 100 shall indicate maximum available capacity . Slice Capacity Value should be measured on a linear scale. |
| >>>Slice Available Capacity Value Uplink | O |  | INTEGER (0..100) | Value 0 indicates no available capacity, and 100 indicates maximum available capacity. Slice Capacity Value should be measured on a linear scale. |
| >>>Slice Radio Resource Status | O |  | 9.3.1.xx |  |
| >>>Slice DRB Usage | O |  | 9.3.1.yy |  |
| >Shared S-NSSAI Available Capacity List |  |  |  |  |
| >>S-NSSAI Available Capacity Item | 1 .. < maxnoofSharedSliceItems> |  |  |  |
| >>>Shared Slice List | O |  | 9.3.1.37 | S-NSSAI list per Shared Resource |
| >>>Slice Available Capacity Value | M |  | INTEGER (0..100) | Value 0 shall indicate no available capacity, and 100 shall indicate maximum available capacity . Slice Capacity Value should be measured on a linear scale. |
| >>>Slice Radio Resource Status | O |  | 9.3.1.xx |  |
| >>>Slice DRB Usage | O |  | 9.3.1.yy |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNs | Maximum no. of PLMN Ids.broadcast in a cell. Value is 12. |
| maxnoofSharedSliceItems [FFS] | Maximum no. of signalled slice support items. Value is 512. [FFS] |

-----------------------------------------------End of the second change-----------------------------------------------------------------

-----------------------------------------------Start of the third change-----------------------------------------------------------------

9.3.1.xx	Slice Radio Resource Status

The Radio Resource Status IE indicates the usage of the PRBs within a slice for all traffic in downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL GBR PRB usage | M |  | INTEGER (0..100) | DL GBR PRB usage |
| UL GBR PRB usage | M |  | INTEGER (0..100) | UL GBR PRB usage |
| DL non-GBR PRB usage | M |  | INTEGER (0..100) | DL non-GBR PRB usage |
| UL non-GBR PRB usage | M |  | INTEGER (0..100) | UL non-GBR PRB usage |

-----------------------------------------------End of the third change-----------------------------------------------------------------

-----------------------------------------------Start of the fourth change-----------------------------------------------------------------

9.3.1.yy	Slice DRB Usage

The Slice DRB Usage IE indicates the usage of the DRBs within a slice.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Number of DRB | O |  | BIT STRING (SIZE(24)) |  |
| DRB Composite Available Capacity | O |  |  | This IE indicates the overall available resource level of DRB. |

-----------------------------------------------End of the fourth change-----------------------------------------------------------------