3GPP TSG-RAN WG3 #110-e     					                                                    R3-206746

- 2 - 12 Nov 2020

Online

Agenda item:	9.3.7.1

Source: 	ZTE, China Telecom

Title: 	Discussion on RAN Sharing Enhancement for MLB in NR

Document for:	Discussion and Approval

Introduction

In LTE, the WI RAN Aspects of RAN Sharing Enhancements for LTE [1] was agreed in RAN#65 meeting. In [1], the X2 Resource Status reporting procedure shall be enhanced to support network sharing:

----------------------------------------------------------------------------------------------------------------------------------------------

The objective of this work item is to make appropriate changes according to the study item conclusion as follows:

- Enhance relevant S1 overload procedure(s),

- Enhance X2 Resource Status reporting procedure,

- Introduce means to quantify and monitor data volume per PLMN and per QoS profile parameters (e.g. QCI).

Co-operation with other groups may be required in case some of the requirements would impact parts of the overall system which are out of scope of RAN3.

----------------------------------------------------------------------------------------------------------------------------------------------

In the Study Item of this topic, it was widely accepted that load reporting among neighbouring Shared eNBs has to be per PLMN-ID granularity. Load balancing among Shared neighbour cells has to take current Load per PLMN ID and agreed metrics per PLMN ID into consideration. Resource Status Update message should enhanced on per-PLMN basis.

Although this MLB enhancement was not introduced in LTE, in this paper, we give analysis and discuss whether this enhancement could be introduced in NR in order to support network sharing.

Discussion

In the case of RAN sharing enhancement, load reporting among neighbouring Shared gNBs could be per PLMN ID granularity. For example, the load of PLMN A in Cell 1 is highly overloaded, it is necessary for Cell 1 to know the load status of neighbouring cells of neighbouring gNBs with PLMN A for load balancing. The gNB which Cell 1 belongs to shall initiate the Resource Status Reporting Initiation procedure to the neighbouring gNBs and trigger the measurement report of the resource status sent by neighbouring gNBs in RESOURCE STATUS UPDATE message under PLMN A. If the load of PLMN A in the target neighbouring gNB is not overloaded, the source gNB could offload some UEs within PLMN A to the target gNB.

In order to complete RAN sharing enhancement solution, RESOURCE STATUS UPDATE message shall include PLMN list corresponding to the resource status report per cell. Furthermore, the Resource Status Reporting Initiation procedure shall be enhanced as well, i.e. RESOURCE STATUS REQUEST message.

Proposal : The Resource Status Reporting Initiation and Resource Status Reporting procedures could be enhanced on per-PLMN basis.

Conclusion

It is proposed to agree on the relative proposal and the corresponding CRs [2][3].

Proposal : The Resource Status Reporting Initiation and Resource Status Reporting procedures could be enhanced on per-PLMN basis.

References

- RP-141671 New WI proposal: RAN Aspects of RAN Sharing Enhancements for LTE   NEC

[2] R3-206750 CR for 38.300 on RAN Sharing Enhancement for MLB   ZTE, China Telecom

[3] R3-206753 CR for TS38.423 on RAN Sharing Enhancement for MLB   ZTE, China Telecom