3GPP TSG-RAN WG3 Meeting #110-e	R3-206983

# E-meeting, 2 – 12 Nov 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0681 | rev | 1 | Current version: | 16.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of alternative QoS profile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Orange, Vodafone, Ericsson, Nokia, Nokia Shanghai Bell, InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL |  |  |  |  |  | Date: |  |  | 2020-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN3 #108e meeting, the TP R3-204194 was agreed to support DRB level Alternative QoS Profile over F1. However, the changes were only performed on the Notify procedure. According to the existing specification, the DU is not able to perform selection on the Alternative QoS profile during the UE Context Setup procedure and UE Context Modification procedures. It means during these procedures, only the CU, which does not well know the Uu condition, makes the decision on the Alternative QoS procedure without any DU report. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enable the DU to feedback the DRB level current QoS parameters set index during the UE Context Setup procedure and UE Context Modification procedures. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR introduces the support of alternative QoS profile coordination during the UE Context Setup procedure and UE Context Modification procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AQP feature is not supported during UE Context Setup procedure and UE Context Modification procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.2, 8.3.4.2, 9.2.2.2, 9.2.2.8, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.423 CR0494 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Simplified the procedure text, corrected typos |  |  |  |  |  |  |  |  |

<CHANGES START>

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure.

If the UE-CapabilityRAT-ContainerList IE is included in the UE CONTEXT SETUP REQUEST, the gNB-DU shall take this information into account for UE specific configurations.

If the servingCellMO IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SpCell accordingly.

If the SpCell UL Configured IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure UL for the indicated SpCell accordingly.

If the SCell To Be Setup List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall consider it as a list of candidate SCells to be set up. If the SCell UL Configured IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure UL for the indicated SCell accordingly. If the servingCellMO IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SCell accordingly.

If the DRX Cycle IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall use the provided value from the gNB-CU.

If the UL Configuration IE in DRB to Be Setup Item IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall take it into account for UL scheduling.

If the SRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup two RLC entities for the indicated SRB. If the Additional Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup the indicated RLC entities for the indicated SRB.

If the DRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the QoS Flow Mapping Indication IE is included in the DRB To Be Setup List IE for a QoS flow, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the indicated DRB.

For each GBR DRB, if the Alternative QoS Parameters Sets IE is included in the GBR QoS Flow Information IE in the UE CONTEXT SETUP REQUEST message, gNB-DU shall, if supported, behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [3].

If the BH Information IE is included in the UL UP TNL Information to be setup List IE for a DRB, the gNB-DU shall, if supported, use the indicated BAP Routing ID and BH RLC channel for transmission of the corresponding GTP-U packets to the IAB-donor, as specified in TS 38.340 [30].

<Unchanged Text Omitted>

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

<Unchanged Text Omitted>

The gNB-DU shall report to the gNB-CU, in the UE CONTEXT MODIFICATION RESPONSE message, the result for all the requested or modified DRBs, SRBs and BH RLC Channels in the following way:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed to be Setup List IE;

-	A list of DRBs which are successfully modified shall be included in the DRB Modified List IE;

-	A list of DRBs which failed to be modified shall be included in the DRB Failed to be Modified List IE;

-	A list of SRBs which failed to be established shall be included in the SRB Failed to be Setup List IE.

-	A list of successfully established SRBs with logical channel identities for primary path shall be included in the SRB Setup List IE only if CA based PDCP duplication is initiated for the concerned SRBs.

-	A list of successfully modified SRBs with logical channel identities for primary path shall be included in the SRB Modified List IE only if CA based PDCP duplication is initiated for the concerned SRBs.

-	A list of BH RLC channels which are successfully established shall be included in the BH RLC Channel Setup List IE;

-	A list of BH RLC channels which failed to be established shall be included in the BH RLC Channel Failed to be Setup List IE;

-	A list of BH RLC channels which are successfully modified shall be included in the BH RLC Channel Modified List IE;

-	A list of BH RLC channels which failed to be modified shall be included in the BH RLC Channel Failed to be Modified List IE;

-	A list of SL DRBs which are successfully established shall be included in the SL DRB Setup List IE;

-	A list of SL DRBs which failed to be established shall be included in the SL DRB Failed to be Setup List IE;

-	A list of SL DRBs which are successfully modified shall be included in the SL DRB Modified List IE;

-	A list of SL DRBs which failed to be modified shall be included in the SL DRB Failed to be Modified List IE.

For each GBR DRB, if the Alternative QoS Parameters Sets IE is included in the GBR QoS Flow Information IE in the UE CONTEXT MODIFICATION REQUEST message, gNB-DU shall, if supported, behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [3].

If the BAP Control PDU Channel IE is included in the BH RLC Channel to be Setup List IE, the gNB-DU shall, if supported, consider that the configured BH RLC channel can be used to transmit BAP Control PDUs, and use this BH RLC channel as specified in TS 38.340 [30].

<Unchanged Text Omitted>

<NEXT CHANGES>

#### 9.2.2.2	UE CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DU To CU RRC Information | M |  | 9.3.1.26 |  | YES | reject |
| C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the gNB-DU | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the SgNB Resource Coordination Information IE as defined in subclause 9.2.117 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| DRB Setup List |  | 0..1 |  | The List of DRBs which are successfully established. | YES | ignore |
| >DRB Setup Item Iist |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>LCID | O |  | 9.3.1.35 | LCID for the primary path or for the split secondary path for fallback to split bearer if PDCP duplication is applied. | - |  |
| >>DL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>> DL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.123 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| SRB Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >SRB Failed to Setup Item |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| DRB Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >DRB Failed to Setup Item |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SCell Failed To Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell Failed to Setup Item |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Inactivity Monitoring Response | O |  | ENUMERATED (not-supported, ...) |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| SRB Setup List |  | 0..1 |  |  | YES | ignore |
| >SRB Setup Item |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>LCID | M |  | 9.3.1.35 | LCID for the primary path if PDCP duplication is applied | - |  |
| BH RLC Channel Setup List |  | 0..1 |  | The list of BH RLC channels which are successfully established. | YES | ignore |
| >BH RLC Channel Setup Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| BH RLC Channel Failed to be Setup List |  | 0..1 |  | The list of BH RLC channels whose setup has failed. | YES | ignore |
| >BH RLC Channel Failed to be Setup Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SL DRB Setup List |  | 0..1 |  | The List of SL DRBs which are successfully established. | YES | ignore |
| >SL DRB Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| SL DRB Failed To Setup List |  | 0..1 |  |  | EACH | ignore |
| >SL DRB Failed To Setup Item IE |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Requested Target Cell ID | O |  | NR CGI 9.3.1.12 | Special Cell indicated in the UE CONTEXT SETUP REQUEST message. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofDLUPTNLInformation | Maximum no. of DL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |

<NEXT CHANGES>

#### 9.2.2.8	UE CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the SgNB Resource Coordination Information IE as defined in subclause 9.2.117 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| DU To CU RRC Information | O |  | 9.3.1.26 |  | YES | reject |
| DRB Setup List |  | 0..1 |  | The List of DRBs which are successfully established. | YES | ignore |
| >DRB Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>LCID | O |  | 9.3.1.35 | LCID for the primary path or for the split secondary path for fallback to split bearer if PDCP duplication is applied. | - |  |
| >>DL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>DL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. < maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.123 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| DRB Modified List |  | 0..1 |  | The List of DRBs which are successfully modified. | YES | ignore |
| >DRB Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>LCID | O |  | 9.3.1.35 | LCID for the primary path or for the split secondary path for fallback to split bearer if PDCP duplication is applied. | - |  |
| >>DL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>DL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>RLC Status | O |  | 9.3.1.69 | Indicates the RLC has been re-established at the gNB-DU. | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. < maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.123 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| SRB Failed to be Setup List |  | 0..1 |  | The List of SRBs which are failed to be established. | YES | ignore |
| >SRB Failed to be Setup Item IEs |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| DRB Failed to be Setup List |  | 0..1 |  | The List of DRBs which are failed to be setup. | YES | ignore |
| >DRB Failed to be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SCell Failed To Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell Failed to Setup Item |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| DRB Failed to be Modified List |  | 0..1 |  | The List of DRBs which are failed to be modified. | YES | ignore |
| >DRB Failed to be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Inactivity Monitoring Response | O |  | ENUMERATED (Not-supported, ...) |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the gNB-DU | YES | ignore |
| Associated SCell List | O |  | 9.3.1.77 |  | YES | ignore |
| SRB Setup List |  | 0..1 |  |  | YES | ignore |
| >SRB Setup Item |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>LCID | M |  | 9.3.1.35 | LCID for the primary path if PDCP duplication is applied | - |  |
| SRB Modified List |  | 0..1 |  |  | YES | ignore |
| >SRB Modified Item |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>LCID | M |  | 9.3.1.35 | LCID for the primary path if PDCP duplication is applied | - |  |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| BH RLC Channel Setup List |  | 0..1 |  | The list of BH RLC channels which are successfully established. | YES | ignore |
| >BH RLC Channel Setup Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| BH RLC Channel Failed to be Setup List |  | 0..1 |  | The list of BH RLC channels whose setup has failed. | YES | ignore |
| >BH RLC Channel Failed to be Setup Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| BH RLC Channel Modified List |  | 0..1 |  | The list of BH RLC channels which are successfully modified. | YES | ignore |
| >BH RLC Channel Modified Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| BH RLC Channel Failed to be Modified List |  | 0..1 |  | The list of BH RLC channels whose modification has failed. | YES | ignore |
| >BH RLC Channel Failed to be Modified Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SL DRB Setup List |  | 0..1 |  | The List of SL DRBs which are successfully established. | YES | ignore |
| >SL DRB Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| SL DRB Modified List |  | 0..1 |  | The List of SL DRBs which are successfully modified. | YES | ignore |
| >SL DRB Modified Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| SL DRB Failed To Setup List |  | 0..1 |  | The List of SL DRBs which are failed to be setup. | YES | ignore |
| >SL DRB Failed To Setup Item |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>cause | O |  | 9.3.1.2 |  | - |  |
| SL DRB Failed To be Modified List |  | 0..1 |  | The List of SL DRBs which are failed to be modified. | YES | ignore |
| >SL DRB Failed To be Modified Item |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>cause | O |  | 9.3.1.2 |  | - |  |
| Requested Target Cell ID | O |  | NR CGI 9.3.1.12 | Special Cell indicated in the UE CONTEXT MODIFICATION REQUEST message. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofDLUPTNLInformation | Maximum no. of DL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |

<NEXT CHANGES>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<Unchanged Text Omitted>

id-ExtendedTAISliceSupportList,

id-E-CID-MeasurementQuantities-Item,

id-ConfiguredTACIndication,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofBPLMNsNR,

maxnoofDLUPTNLInformation,

maxnoofNrCellBands,

maxnoofULUPTNLInformation,

<Unchanged Text Omitted>

DRBs-Modified-Item	::= SEQUENCE {

dRBID							DRBID,

lCID								LCID		OPTIONAL,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Modified-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-Modified-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-RLC-Status							CRITICALITY ignore	EXTENSION RLC-Status									PRESENCE optional }|

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-List		PRESENCE optional }|

{ ID id-CurrentQoSParaSetIndex				CRITICALITY ignore	EXTENSION QoSParaSetIndex								PRESENCE optional },

...

}

DRBs-ModifiedConf-Item	::= SEQUENCE {

dRBID							DRBID,

uLUPTNLInformation-ToBeSetup-List		ULUPTNLInformation-ToBeSetup-List	,

iE-Extensions	ProtocolExtensionContainer { { DRBs-ModifiedConf-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-ModifiedConf-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-List			PRESENCE optional },

...

}

DRB-Notify-Item ::= SEQUENCE {

dRBID			DRBID,

notification-Cause	Notification-Cause,

iE-Extensions	ProtocolExtensionContainer { { DRB-Notify-ItemExtIEs } }	OPTIONAL,

...

}

DRB-Notify-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-CurrentQoSParaSetIndex	CRITICALITY ignore	EXTENSION QoSParaSetNotifyIndex	PRESENCE optional	},

...

}

DRBs-Required-ToBeModified-Item	::= SEQUENCE {

dRBID							DRBID,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List	,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Required-ToBeModified-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-Required-ToBeModified-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-RLC-Status			CRITICALITY ignore			EXTENSION RLC-Status				PRESENCE optional }|

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-List			PRESENCE optional },

...

}

DRBs-Required-ToBeReleased-Item	::= SEQUENCE {

dRBID		DRBID,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Required-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-Required-ToBeReleased-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-Setup-Item ::= SEQUENCE {

dRBID							DRBID,

lCID								LCID		OPTIONAL,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List	,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Setup-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-Setup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-List			PRESENCE optional }|

{ ID id-RLCDuplicationInformation			CRITICALITY ignore	EXTENSION RLCDuplicationInformation						PRESENCE optional}|

{ ID id-CurrentQoSParaSetIndex				CRITICALITY ignore	EXTENSION QoSParaSetIndex									PRESENCE optional },

...

}

DRBs-SetupMod-Item	::= SEQUENCE {

dRBID							DRBID,

lCID								LCID		OPTIONAL,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List	,

iE-Extensions	ProtocolExtensionContainer { { DRBs-SetupMod-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-SetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-List			PRESENCE optional }|

{ ID id-CurrentQoSParaSetIndex				CRITICALITY ignore	EXTENSION QoSParaSetIndex									PRESENCE optional },

...

}

DRBs-ToBeModified-Item	::= SEQUENCE {

dRBID						DRBID,

qoSInformation				QoSInformation	OPTIONAL,

uLUPTNLInformation-ToBeSetup-List	ULUPTNLInformation-ToBeSetup-List	,

uLConfiguration				ULConfiguration	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-ToBeModified-ItemExtIEs } }	OPTIONAL,

...

}

<Unchanged Text Omitted>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<Unchanged Text Omitted>

id-SlotNumber										ProtocolIE-ID ::= 432

id-TRP-MeasurementRequestList						ProtocolIE-ID ::= 433

id-MeasurementBeamInfoRequest						ProtocolIE-ID ::= 434

id-E-CID-ReportCharacteristics						ProtocolIE-ID ::= 435

id-ConfiguredTACIndication							ProtocolIE-ID ::= 436

id-Extended-GNB-DU-Name								ProtocolIE-ID ::= 437

id-Extended-GNB-CU-Name								ProtocolIE-ID ::= 438

END

-- ASN1STOP

<CHANGES END>