3GPP TSG-RAN WG3 Meeting #110-e                                                            	R3-20xxxx

2-12 November 2020

Title:	LS on inter-donor topology redundancy

Response to:

Release:	Rel-17

Work Item:	          NR_IAB_enh-Core

Source:	Samsung

To:	RAN1

Cc:	RAN2

Contact Person:

#### Name:	Weiwei Wang

E-mail Address:	ww1016.wang@samsung.com

- Overall Description:

RAN3 discussed the inter-donor topology redundancy for load balancing, and agreed to support the following two scenarios (as shown in the following figure) with the principle that an IAB-DU only has F1 interface with one donor-CU:

- Scenario 1: the IAB is multi-connected with 2 Donors.

- Scenario 2: the IAB’s parent/ancestor node is multi-connected with 2 Donors.

In both scenarios, the boundary IAB node, i.e., IAB3 in the figure, is simultaneously connected to the two parent nodes (i.e., IAB1 and IAB2) belonging to two different donors. RAN3 would like to ask RAN1 to take these scenarios into account when discussing duplexing enhancements, and provide feedback on the coordination needed between the two donors.

.

## 2. Actions:

To RAN2:

ACTION: RAN3 respectfully asks RAN1 to take the above into account when discussion duplexing enhancements and to provide feedback on the coordination needed between the two donors.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #111-e	    25th. Jan -  5th   Feb.   2021