3GPP TSG-RAN WG3 #110-e	R3-206932

Online, 2-12 November 2020

Agenda Item:	8.1

Source:	Qualcomm (moderator)

Title:	Summary of Offline Discussion on PNI-NPN release

Document for:	Approval

# Introduction

CB: # 78_PNI-NPN

- no st2 CR needed

- need for new cause value?

- check details

(QC - moderator)

Summary of offline disc R3-206932

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Interim status

Based on the discussion and proposed conclusions in section 4, the interim status is as follows

A majority of companies prefer not to expand the context release text (purpose of procedure).

A majority of companies support reusing / expanding the existing cause value.

Further discussion seems needed to reach a final consensus.

# Discussion

From the online discussion, it is assumed From the online discussion, it is assumed that no stage 2 is required, and so the scope of the discussion is about the proposed stage 3 changes as a consequence of the SA2 CR and agreement. The relevant CRs are in [1,2,3]. The changes in these CRs are summarized as follows:

- Add new condition to the statement of when context release is used (section 8.3.3.1) [1]

- Add new cause value OR extend existing PNI-NPN cause value in NGAP [1,2]

- Extend existing NPN cause value in XnAP to mention also release, and include mention of “appropriate cause value” in procedural text for completeness [3]

As an alternative, it was also mentioned online that an existing generic cause value could be reused (“5GC generated cause”).

## NGAP changes

Please provide your view on items 1 and 2 above (from refs [1,2])

| Company | Comment |
| --- | --- |
| Huawei | Item1: “nice to have” update. Maybe it can be briefly described as “PNI-NPN Mobility Information is updated” Item2: we would prefer to extend the existing cause value, if something is really needed. |
| Nokia | Item 1: prefer to avoid adding text every time there is new case, otherwise make believe this is an exhaustive list. Item 2: extend existing cause. |
| ZTE | Item1: “nice to have” update. . Item 2: Existing cause value “NPN access denied” is so generic that it doesn’t cover the release reason of allowed PNI-NPN list changed. We prefer to add new cause value. |
| Qualcomm | Item 1: tend to no because as Nokia said this is not exhaustive, and we should not suggest that this should get updated any more in future. Also stage 2 is quite clear. Item 2: extend existing cause value – in fact this helps to make clear that release is anyway used for initial access failure (not just MRL change). |
| Ericsson | 1) agree with Nokia, where does this end? 2) would really (sorry about that) go for “do nothing”. I would expand on 1) and ask: for how many special cases would we need a new cause value? For RAN KPIs it is sufficient to know “wasn’t because of me, it was the CN”. And to add on RNL a cause that actually stems from a (CN generated) NAS action, why would RAN / RNL care? And finally, using a cause that is originally thought to be used for NPN access problems doesn’t seem right. |

Conclusions for NGAP:

- On text for release procedure, 3 companies think this is not needed or are not convinced it is needed, and 2 companies say this is nice to have.

- On cause value, 3 companies are fine with extending the existing cause value, 1 company prefers to add a new cause value, and one company thinks no change is needed

## XnAP changes

Please provide your view on item 3 (from ref [3])

| Company | Comment |
| --- | --- |
| Huawei | We support this update for inactive UE mobility. |
| Nokia | CR is OK. We’d like to cosign. |
| ZTE | We are fine to update cause value for release, but current existing cause value is too generic. New cause value for release reason needs to be added. |
| Qualcomm | Still ok with CR. |
| Ericsson | Don’t think this is needed. The Cause is a mandatory IE in a response message, we normally don’t specify such IEs in the procedure text. |

Conclusions for XnAP:

- On cause value, 3 companies are fine with extending the existing cause value, 1 company prefers to add a new cause value, and 1 company thinks no need for change in procedural text (presumably also no change in the cause value)

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-206365, CR38.413 for allowed PNI-NPN changed (ZTE Corporation)

- R3-206166, Support of release on CAG subscription change (Qualcomm Incorporated)

- R3-206167, Support of release on CAG subscription change (Qualcomm Incorporated)