3GPP TSG-RAN WG3 Meeting #110e	draft-R3-206957

# Online Meeting, 2nd – 12th November 2020

Agenda Item:	9.3.7.1

Source:	Ericsson, ZTE, NEC, CATT, Nokia, Nokia Shanghai Bell, Huawei, Samsung

Title: 	NGAP correction on Global eNB ID in Target ID IE

Document for:	Discussion and Approval

# 1	Background

Among the interested companies, we have discussed during offline that in NGAP the Global eNB ID in Target ID refers to the Global ng-eNB ID IE Type, which is specified as “globally identify an ng-eNB (see TS 38.300). “

This paper summarize the discussion so far and aiming to make a conclusion on the matter.

# 2	Discussion

In TS 38.300, it is defined that “ng-eNB” as “node providing E-UTRA user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC”.

Two issues are discussed offline:

- Does it bring confusion that the Global eNB ID refers to the Global ng-eNB ID?

Views:

- The reference causes confusion and we better to fix it, so that the Global eNB ID does not refer to Global ng-eNB ID. If we decide to correct it, there are a couple of solutions.

- Global eNB ID reuses the encoding for Global ng-eNB ID described in 9.3.1.8. This is allowed and not mistake.

- The Global eNB ID IE is defined in NGAP in Rel 16, which includes home eNB ID. Do we need to align the Global eNB ID in the Target ID?

Views:

- we don't have specific discussion on inter-system HO to HeNB, we also don't consider how to route to HeNB via a HeNB GW. Maybe no need.

- Yes, we need to align. There are at least 3 solutions:

-          Option A: Add a new choice for “Home eNB ID” in the Global ng-eNB ID IE  => This does not seem preferable, since Home eNB is not supported per-se in 5GS.

-          Option B: Add a new choice for “Global eNB ID” in the Target ID IE, that points to Global eNB ID defined in 9.3.1.165 => This does not seem preferable, since it overlaps with an existing choice.

-          Option C: Add a new choice for “Global Home eNB ID” in the Target ID IE, that points to a newly defined IE in 9.3.1.x. Option C seems to be the best option, illustrated below.

9.3.1.25 Target ID This IE identifies the target for the handover. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CHOICE Target ID; M. >NG-RAN. >>Global RAN Node ID; M; 9.3.1.5. >>Selected TAI; M; TAI 9.3.3.11. >E-UTRAN. >>Global eNB ID; M; Global ng-eNB ID 9.3.1.8. >>Selected EPS TAI; M; EPS TAI 9.3.3.17. >Target RNC-ID. >>LAI; M; 9.3.3.30. >>RNC-ID; M; 9.3.1.123; This IE is ignored if the Extended RNC-ID IE is included in the Target ID IE.. >>Extended RNC-ID; O; 9.3.1.124; The Extended RNC-ID IE is used if the RNC identity has a value larger than 4095.. >Target Home eNB ID. >>Global Home eNB ID; M; 9.3.1.x. >>Selected EPS TAI; M; EPS TAI 9.3.3.17.

It is proposed that RAN3 to discuss this issue and make a conclusion. If CR is needed, it can be produced during the meeting.

Proposal 1: RAN3 to discuss the issue related to the Global eNB ID in Target ID IE and make a conclusion.

# 3	Summary of the offline discussion

CB: # 86_GlobaleNBIDinTgtIDIE

- should inter-system HO to E-UTRAN be allowed when tgt node is a HeNB? Check w.r.t. routing (TAI used for HeNBs)

- continue checking IE encoding

- should we introduce an IE which is not used?

- go for a simple description in semantics?

(E///)

rev in R3-206957

Question 1: should inter-system HO to E-UTRAN be allowed when the target node is a HeNB? If yes, how to solve routing when TAI is used for HeNBs?

| Company | Yes/No | Comment |
| --- | --- | --- |

Question 2: should we introduce an IE which is not used? (i.e. the HeNB in the target ID)

| Company | Yes/No | Comment |
| --- | --- | --- |

Question 3: do you consider Global eNB ID refers to Global ng_eNB ID an issue and need to be fixed?

| Company | Yes/No | Comment |
| --- | --- | --- |

Question 4: if Yes to Question 3, provide your view how to fix, i.e. a simple description in semantics or other way?

| Company | Comment |
| --- | --- |