3GPP TSG-RAN WG3 Meeting #110-e	R3-206988

E-meeting, 2 – 12 November, 2020

Agenda item:	9.3.7.1

Source:	Ericsson (moderator)

Title:		CB: # 98_MeasGapConfig - Summary of email discussion

Document for:	Approval

# 1	Introduction

This paper provides summary of discussions at RAN#110-e on:

CB: # 98_MeasGapConfig

- need to clarify behavior on reception of MeasGapConfig IE? Same mechanism as Cell Group Config?

- maintain backwards compatibility!

- merge disc from 6773

- any delta w.r.t. previous discussions?

(E/// - moderator)

# 2	For the Chairman’s Notes

[To be completed]

# 3	Discussion

## 3.1 	Analysis of transparent Delivery of MeasGapConfig

In R3-206809 it is proposed to add the following sentence to the F1: UE Context Setup, UE Context Modification (gNB-CU initiated) and UE Context Modification (gNB-DU initiated):

If the MeasGapConfig IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall perform RRC Reconfiguration as described in TS 38.331 [8]. The MeasGapConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8].

To understand the rationale behind this sentence let’s analyse what TS38.471 describes with regards to the use of the MeasGapConfig IE.

In TS38.473, section “8.3.1	UE Context Setup” it is mentioned that

If the gNB-CU includes the SMTC information of the measured frequency(ies) in the MeasurementTimingConfiguration IE of the CU to DU RRC Information IE that is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall generate the measurement gaps based on the received SMTC information. Then the gNB-DU shall send the measurement gaps information to the gNB-CU in the MeasGapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message.

If the MeasConfig IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall deduce that changes to the measurements configuration need to be applied. If the measObjectToAddModList IE is included in the MeasConfig IE, then the frequencies added in such IE are to be activated. Then the gNB-DU shall decide if measurement gaps are needed or not and, if needed, the gNB-DU shall send the measurement gaps information to the gNB-CU in the MeasGapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message. If the measObjectToRemoveList IE is included in the MeasConfig IE, the gNB-DU shall ignore it.

In TS38.473, section “8.3.4	UE Context Modification (gNB-CU initiated)” it is mentioned that

If the gNB-CU includes the SMTC information of the measured frequency(ies) in the MeasurementTimingConfiguration IE of the CU to DU RRC Information IE that is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall generate the measurement gaps based on the received SMTC information. Then the gNB-DU shall send the measurement gaps information to the gNB-CU in the MeasGapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message.

If the MeasConfig IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall deduce that changes to the measurements’ configuration need to be applied. The gNB-DU shall take the received info, e.g. the measObjectToAddModList IE, and/or the measObjectToRemoveList IE into account, when generating measurement gap and when deciding if a measurement gap is needed or not.

From the text mentioned above, it can be deduced the following:

- The gNB-DU is responsible to generate measurement gaps. Namely, the measurement gap configuration is decided by the gNB-DU and shall not be changed by the gNB-CU. This is confirmed by the text above stating that 
Then the gNB-DU shall send the measurement gaps information to the gNB-CU in the MeasGapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message. 
And
The gNB-DU shall take the received info, e.g. the measObjectToAddModList IE, and/or the measObjectToRemoveList IE into account, when generating measurement gap and when deciding if a measurement gap is needed or not.

- The gNB-DU is the node deciding whether measurement gaps are needed or not. Namely, when a gNB-DU receives from the gNB-CU a MeasConfig IE indicating the need for measurement gaps on certain frequency bands, the gNB-DU decides if such measurement gaps are needed or not and the gNB-DU produces a MeasGapConfig IE as a consequence of such decision.

  - If a gNB-DU decides that no measurement gaps are needed, gNB-DU does not generate a MeasGapConfig IE

  - If a gNB-DU decides, at any point in time, that measurement gaps are needed it generates a  MeasGapConfig IE

The understanding above is also confirmed by the fact that the gNB-CU signals to the gNB-DU a MeasConfig IE which does not include the details about the Measurement Gap configuration. Namely, the MeasConfig IE signalled from gNB-CU to gNB-DU does not contain the MEasGapConfig IE, which is under the gNB-DU responsibility. This is confirmed by the following semantics in 38.473:

#### 9.3.1.25	CU to DU RRC Information

This IE contains the RRC Information that are sent from gNB-CU to gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CG-ConfigInfo | O |  | OCTET STRING | CG-ConfigInfo, as defined in TS 38.331 [8]. | - |  |
| UE-CapabilityRAT-ContainerList | O |  | OCTET STRING | This IE is used in the NG-RAN and it consists of the UE-CapabilityRAT-ContainerList, as defined in TS 38.331 [8]. | - |  |
| MeasConfig | O |  | OCTET STRING | MeasConfig, as defined in TS 38.331 [8] (without MeasGapConfig). For EN-DC/NGEN-DC operation, includes the list of FR2 frequencies for which the gNB-CU requests the gNB-DU to generate gaps. For NG-RAN,NE-DC and MN for NR-NR DC, includes the list of FR1 and/or FR2 frequencies for which the gNB-CU requests the gNB-DU to generate gaps and the gap type (per-UE or per-FR). | - |  |

From the above, it is clear that, once the gNB-DU generates the MeasGapConfig IE, containing the measurement gap configuration, the gNB-CU shall signal it to the UE so to configure the measurement gaps. Failure to do so would generate an out of synch situation between measurement gaps configured at gNB-DU and those configured at the UE.

From the above it can also be understood that the content of the MeasGapConfig IE should be transferred transparently by the gNB-CU to the UE. Failure to do so would again create an out of synch situation between measurement gaps configured at gNB-.DU and those configured at the UE.

In light of the analysis above, companies are invited to provide their comments on whether R3-206809 can be agreed

| Company | Comment |
| --- | --- |
| Ericsson | Yes, R3-206809 can be agreed as it clarifies some principles that are implicitly supported in the specifications. |

If companies believe that R3-206809 cannot be agreed, please explain

- Why a new MeasGapConfig IE signalled from a gNB-DU to a gNB-CU should not trigger a UE RRC reconfiguration aimed at configuring the measurement gaps

- Why a new MeasGapConfig IE signalled from a gNB-DU to a gNB-CU should not be signalled to the UE transparently

| Company | Comment |
| --- | --- |

## 3.2 	Analysis of MeasGapConfig IE and the Configuration Query IE

In R3-206774 the following change is proposed for section “8.3.4	UE Context Modification (gNB-CU initiated)” of TS38.473:

If the GNB-DU Configuration Query IE is contained in the UE CONTEXT MODIFICATION REQUEST message, gNB-DU shall include the CellGroupConfig IE and the MeasGapConfig IE in the DU To CU RRC Information IE in the UE CONTEXT MODIFICATION RESPONSE message.

To understand the rationale behind this change one needs to understand the purpose of the GNB-DU Configuration Query IE. This IE is described as follows in TS38.473:

| GNB-DU Configuration Query | O |  | ENUMERATED (true, ...) | Used to request the gNB-DU to provide its configuration. | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |

Therefore, the GNB-DU Configuration Query IE is a tool for the gNB-CU to check on the gNB-DU configuration and eventually identify that there is no out of synch between the gNB-DU configuration known at the gNB-CU and that in place at the gNB-DU.

As per current specifications, the GNB-DU Configuration Query IE is used only to request that the gNB-DU reports the CellGroupConfig IE. However, as seen in the analysis from section 3.1, the gNB-DU is also responsible for generation of the MeasGapConfig IE. Hence it is plausible that the gNB-CU can query the gNB-DU configuration in terms of MeasGapConfig IE.

Conclusion 1: The the GNB-DU Configuration Query IE is a tool for the gNB-CU to request the latest gNB-DU configuration information. As gNB-DU is responsible for MeasGapConfig, it is plausible that the the GNB-DU Configuration Query IE triggers reporting of the MeasGapConfig IE too

Of course, it is obvious that if the gNB-CU requests from the gNB-DU reporting of the CellGroupConfig and MeasGapConfig, via the GNB-DU Configuration Query IE, the gNB-CU shall not signal these parameters towards the UE (by means of RRC reconfiguration) unless these parameters are different from those currently configured at the UE.

Conclusion 2: IF the gNB-CU receives CellGroupConfig and MeasGapConfig as a response to signalling of the the GNB-DU Configuration Query IE, the gNB-CU shall not signal these parameters to the UE via RRC reconfiguration unless they are different from those last configured at the UE

In light of the analysis above, companies are invited to provide their comments on whether R3-206774 can be agreed

| Company | Comment |
| --- | --- |
| Ericsson | Yes, R3-206774 can be agreed as it enalbes the gNB-DU to report its configuration information when the GNB-DU Configuration Query IE is received |

If companies believe that R3-206774 cannot be agreed, please explain

- How can a gNB-CU check the latest gNB-DU MeasGapConfig configuration and how to correct eventual out of synch MeasGapConfig configurations between gNB-DU and UE

| Company | Comment |
| --- | --- |

# 4	Conclusion, Recommendations [if needed]

If needed

# 5	References

[1] R3-206032, Discussion on the MDT for inactive UE (Samsung)

[2] R3-206701, Management based MDT should not overwrite signaling based MDT (ZTE)

[3] R3-206094, Further discussion on MDT configuration overriding issue (Huawei)

[4] R3-206549, Discussion on Signalling and Management based Logged MDT (Ericsson)