3GPP TSG-RAN WG3 #110-e	R3-206900

Online, 2-12 November 2020

Agenda Item:	15.2

Source:	China Unicom

Title:	Summary of Offline Discussion on per slice QoE measurement

Document for:	Approval

# Introduction

This contribution provides the summary of offline discussion for the following:

CB: # NRQoE6-Slice

- Slice QoE is needed for SLA maintenance and enforcement in OAM, slice service experience analysis and prediction in NWDAF and network slice selection in NSSF ?

- How to inform the 5GC the mapping between a QOE report and the specific slice?

- Network slice scope information should be added in NR QoE measurement configuration to support slice QoE measurement?

- Network slice information should be added in NR QoE report to support slice QoE reporting?

- Capture agreements as TP for TR

(CU - moderator)

Summary of offline disc R3-206900

# For the Chairman’s Notes

Propose the following:

Propose to capture the following:

# Discussion

The following contributions are captured in this section.

| R3-206036 | Discussion on NR QoE solutions (Samsung) | discussion |
| --- | --- | --- |
| R3-206493 | Discussion on requirements and mechanisms for per slice QoE measurement (China Unicom, ZTE) | discussion |
| R3-206715 | Consideration on slice QoE measurement (ZTE) | discussion |

## Issues to be discussed

According to the above description and related discussion papers [1-3], the issues of per slice QoE measurement can be listed as follows.

- Whether per slice QoE measurement should be supported

- If yes,  the scenarios of per slice QoE measurement

- If yes, the mechanism of per slice QoE measurement

  - configuration

  - collection and mapping

  - reporting

## The demand of per slice QoE measurement

### Requirements of slice QoE in NR QoE

Network slicing is a key feature for 5G, it should be considered anywhere in 5G. Regarding NR QoE, collecting QoE per slice is helpful for operators to be aware of actual user experience of a specific slice. The slice QoE is useful no matter for management system (e.g. OAM) or for network functions (e.g. UDM, NWDAF, etc.).

The requirements of slice QoE in 5G are:

- for slice SLA maintenance and enforcement in OAM

- for slice experience analysis and prediction in NWDAF

- for better slice selection decision in NSSF

Proposal 1: NR QoE should support per slice QoE measurement.

Moderator’s note: The answer could be agree/not agree, and comments/reasoning of the answer is welcome.

| Company | Answer | Comment |
| --- | --- | --- |
| China Unicom | agree | From operators’ point of view, it’s quite essential to confirm that the SLA is fulfilled and to statistics the QoE of services for different users with the same service type running on different slices to show the significantly superiority of customized slice. |
| Huawei | Yes |  |

## The scenarios of per slice QoE measurement

Slice awareness in NG-RAN is introduced at PDU session level, by indicating the S-NSSAI corresponding to the PDU Session. Therefore, gNB can get a one-to-one mapping between the the PDU session and slice.

After permutation and combination according to different applications, service types and slice, the possible scenarios are as follows:

Table 1

| Scenario 1 | APP 1 –service type 1 –slice 1 APP 2 –service type 2 –slice 2 | Scenario 5 | APP 1 | –service type 1 –slice 1 –service type 2 –slice 2 |
| --- | --- | --- | --- | --- |
| Scenario 2 | APP 1 –service type 1 –slice 1 APP 2 –service type 2 –slice 1 | Scenario 6 | APP 1 | –service type 1–slice 1 –service type 2–slice 1 |
| Scenario 3 | APP 1 –service type 1 –slice 1 APP 2 –service type 1 –slice 2 | Scenario 7 | APP 1 | –service type 1–slice 1 –service type 1–slice 2 |
| Scenario 4 | APP 1 –service type 1 –slice 1 APP 2 –service type 1 –slice 1 | Scenario 8 | APP 1 | –service type 1 –slice 1 |

Proposal 2: Per slice QoE measurement should cover all scenarios in table 1.

Moderator’s note: The answer could be a list of scenarios (e.g. scenario 1/2/3 or all), and comments/reasoning of the answer is welcome.

| Company | Answer | Comment |
| --- | --- | --- |
| China Unicom | All if possible | For scenario 7, APP1 is allowed to setup PDU sessions correlated to slice 1 and slice 2. During the PDU Session Setup procedure, two PDU sessions are setup and the same service type is served by both slices. In this scenario, the application layer may not be able to create QoE report of service running on specific slice (i.e. slice 1 or slice 2). From our point of view, in actual deployment, one APP will select one single slice according to NSSP (Network slice selection Policy) in URSP (UE Routing Selection Policy). Thus, the scenario 5 and 7 can be low priority to be considered if they have no feasibility or practical use. |
| Huawei | Yes | We could study these scenarios, and see if some scenarios could be prioritized |

Proposal 2.1: If there are other scenarios, please describe them below.

| Company | Other scenario |
| --- | --- |

## The mechanism of per slice QoE measurement

### Configuration

In LTE QMC, the QoE can be collected from UEs in a specific area which realized by a parameter in the QoE configuration IEs, the parameter can be either Area Scope (including list of PLMNs, TACs or Cells) out of the QoE configuration file checked by eNB or cell list in the QoE configuration file checked by UE.

For NR, the method of checking Area Scope by the RAN node or checking cell list by UE can be reused in slice QoE. To be specific, identification of slice (i.e. S-NSSAI) can be included in QoE measurement configuration to support mechanism for per slice QoE measurement.

For both management-based and signaling-based solution, the Network Slice Scope can be included in the QoE measurement collection configuration, just the same as Area Scope.

Proposal 3: RAN3 to consider area scope method for slice QoE measurement . Network slice scope information should be added in NR QoE measurement configuration to support slice QoE measurement.

Moderator’s note: The answer could be agree/not agree, and comments/reasoning of the answer is welcome.

| Company | Answer | Comment |
| --- | --- | --- |
| China Unicom | Agree | This is a similar implementation with the Area Scope |
| Huawei | ok |  |

### Collection and mapping

During the PDU session setup procedures, a PDU session is associated to an S-NSSAI, which identifies a network slice. If the S-NSSAI list is included in QoE measurement configuration to specify the target slice(s), mechanism to realize the mapping between a QoE report and the specific slice should be introduced.

Proposal 4: RAN3 to study how to realize the mapping between a QoE report and the specific slice?

Moderator’s note: The answer is whether the suggested potential solution is valid for all the considered scenarios in 3.3.

| Company | Potential solution | Answer |
| --- | --- | --- |
| China Unicom | The procedure can be as follows: 1. OAM/CN transmit the QoE measurement configuration to NG-RAN, which includes the target slice list; 2. NG-RAN can map the target slice list to PDU session list and attach the PDU session list with the QoE measurement configuration to UE; 3. UE will check the PDU session list and map the PDU session to APP according NSSP and collect the QoE measurement based on the configuration; 4. UE will feedback the QoE measurement report with the corresponding PDU session ID; 5. NG-RAN can remap the PDU session ID back to slice ID and attach it in the QoE report. | scenario 7 need FFS if supported |
| Huawei | The procedures described above could be taken as a starting point, not sure if this is a common scenario that one service type would be cross-slice. |  |

### Reporting

Proposal 5: Network slice information should be added in NR QoE report to support slice QoE reporting?

Moderator’s note: The answer could be yes/no, and comments/reasoning of the answer is welcome.

| Company | Answer | Comment |
| --- | --- | --- |
| China Unicom | Yes | See Proposal 4 |
| Huawei | Yes |  |

## TP to be captured in TR

If the conclusion of the above discussion is in support of per slice QoE measurement, what should be captured in the TR? According to the discussion pagers, there are two options as follows.

- Introducing a separate section that describes the per slice QoE measurement

- Capture the description related to slice in the corresponding procedures, e.g. configuration, reporting

Proposal 6: To introduce separate section for QoE slice is necessary in TR .

Moderator’s note: The answer could be yes/no, and comments/reasoning of the answer is welcome.

| Company | Answer | Comment |
| --- | --- | --- |
| China Unicom | yes | A separate section to specify the scenario and mechanism is necessary. |
| Huawei | yes |  |

## Others

Anything else needs to be discussed, please list here.

# Conclusion, Recommendations

# References

- R3-206036: Discussion on NR QoE solutions (Samsung)	discussion

- R3-206493: Discussion on requirements and mechanisms for per slice QoE measurement (China Unicom, ZTE)	discussion

- R3-206715: Consideration on slice QoE measurement (ZTE)	discussion