3GPP TSG-RAN3 Meeting #110-e	R3-207107

E-meeting, 2 - 12 November 2020

Title: 	Evaluation of Slice re-mapping policy solutions

Source: 	Huawei, LGU+

Agenda item:	17.2

Document Type:	Discussion and Decision

Annex – TP

6	Study necessity and mechanisms to support service continuity

6.1	Scenario and issue description

Editor Note: capture the description of scenario and issue.

6.2	Solutions

Editor Note: Capture the solutions for the scenario and issue.

<Unchanged Text Omitted>

6.2.x	Configuration Based Solution

The following analysis is provided for the scenario 1 ad scenario 2 respectively:

- Scenario 1: Slice resource shortage in case of Intra-RA mobility and Inter-RA mobility

As specified in TS 28.541, the slice re-mapping between different S-NSSAIs can be achieved via the prioritized resource modeling. For example, suppose UE’s ongoing slice is S-NSSAI 1 configured with rRMPolicyMaxRatio policy, which can use at least one of the shared resources, prioritized resources and dedicated resources. If the dedicated resources are not available, it can use other un-used prioritized and shared resources.

But the following needs to be further studied, e.g., for the S-NSSAI 1,

  - it can explicitly use resources belonging to which S-NSSAIs;

  - it can use the dedicated but not used resources of other S-NSSAIs;

  - it can preempt the used prioritized and/or shared resources from other S-NSSAIs.

In this case, further involvement with SA5 is required.

- Scenario 2: Non-supported slice in case of Inter-RA mobility

In this case, if the T-gNB does not support certain S-NSSAIs, these S-NSSAIs will not be included in the RRMPolicyMemberList, thus no resource will be planned by the T-gNB, as specified in TS 28.541.

For example, suppose UE’s ongoing slice is S-NSSAI 1, it will not be included in the RRMPolicyMemberList of the T-gNB. Thus the re-mapping of S-NSSAI 1 to the supported S-NSSAI(s) of T-gNB is not supported.

In this case, slice re-mapping is not supported yet by the prioritized resource modelling defined in SA5. And further involvement with SA5 is required.