3GPP TSG-RAN WG3 #110-e							 R3-206899

2-12 November 2020

Online

Agenda Item:	15.2

Source:	ZTE (moderator)

Title:	Summary of Offline Discussion on NRQoE5-Signalling_flow

Document for:	Approval

Introduction

CB: # NRQoE5-Signalling_flow

- Reuse Trace mechanism for NR QoE is preferred, enhancements can refer MDT solution in NR?

- Management-based and signalling-based QoE signalling flow?

- The RAN node should know the service type related to QoE measurement configuration and QoE reporting which it performs?

- Send an LS to SA5 (cc RAN2) relative to framework used for LTE QMC?

- Send an LS to SA4 (cc RAN2) to inquire about their plans concerning application client requirements for NR services with respect to mandatory vs. optional support of OMA-DM and QMC?

- Capture agreements as TP for TR

- Send LSout to other groups, if needed

(ZTE- moderator)

Summary of offline disc

Note: The first round email discussion plan to be end before the end of first week (Friday 18:00 UTC 2020-11-6).

For the Chairman’s Notes

Propose the following:

Agree the TP on NR QoE for 38.890_Addition of general Signaling-based signaling for NR QoE in R3-207073.

Agree the TP on NR QoE for 38.890_Addition of general Management-based signaling for NR QoE in R3-207072.

Agree Send an LS to SA5 (cc RAN2) relative to framework used for LTE QMC in R3-207112.

Propose to capture the following:

Phase 2 Discussion

TP for addition general signalling based QOE procedure

Based on the input in phase 1 discussion, the general signalling based QOE procedure can be accepted based on modification.

We would like Samsung to provide TP for addition of Signaling-based signaling for NR QoE.

Proposal_phase 2_1: Agree the TP on NR QoE for 38.890_Addition of general Signaling-based signaling for NR QoE in R3-207073.

Please provide your further comments on the revised CR.

| Company | Comment |
| --- | --- |

TP for general management based QOE procedure

Based on the input in phase 1 discussion, the general management based QOE procedure can be accepted based on modification.

We would like ZTE to provide the TP for addition of management-based signaling for NR QoE.

Proposal_phase 2_2: Agree the TP on NR QoE for 38.890_Addition of general Management-based signaling for NR QoE in R3-207072.

Please provide your further comments on the revised CR

| Company | Comment |
| --- | --- |
| Nokia | 6.z.x: clarifications needed (in title and text) that the section concerns activation of measurement collection job. Concerning "UE selected by OAM": Such selection is possible and already exists for LTE QMC, however in that case the OAM triggers activation via the CN. So "UE selected by OAM" should not be covered in a section for m-based activation. It is covered in the other TP (7073). 6.z.y: clarifications needed (in title and text) that the section concerns deactivation of measurement collection job. So specific UE is not targeted. |
| ZTE | 6.z.x 1: OK for clarifications on “the section concerns activation of measurement collection job”. 2: For “UE selected by OAM” Actually, the “UE selected by OAM” come from one companies comments in the phase 1 discussion. Which is “We think that m-based QoE should have the possibility to target individual UEs” . In CB: # NRQoE3-RANConfig_Report There is a potential agreement list below - For this case ,as individual Both Management-based and signaling-based QoE measurement could be configured in a certain area and towards an individual specific UE; Based on the potential agreement, the update can be : From: Either NG-RAN finds qualified UEs that meet the criteria (e.g. application layer capability, service type, etc.) or NG-RAN node checks the criteria for the UE selected by OAM (FFS) To NG-RAN finds a target individual UE(FFS) or qualified UEs that meet the criteria (e.g. application layer capability, service type, etc.) 6.z.y: 1: OK to add clarification. Hope this will be fine. |

Send an LS to SA5 (cc RAN2) relative to framework used for LTE QMC

Based on the input in phase 1 discussion, the LS based on R3-206370 can be accepted based on  modification.

We would like Nokia to provide the LS.

Proposal_phase 2_3: Agree the Send an LS to SA5 (cc RAN2) relative to framework used for LTE QMC in R3-207112.

Please provide your further comments on the revised LS.

| Company | Comment |
| --- | --- |
| Nokia | An updated draft has been provided, with simplifications and included suggestions from the first discussions. |

Discussion

Signalling procedure

Last RAN3 meeting achieved agreement on solutions.

Both the management-based and signalling-based solutions for NR QoE management are supported.

While the correspond signalling is missing. In [2][6][8][16][23], companies share the view that MDT-based signalling can be baseline for NR QMC. Based on MDT/Trace, in this meeting one company provides detail signalling in [3] for TP.

It is noting mobility and RAN triggered aspects covered by other CBs, in this CB only cover general activation and deactivation procedure.

Management-based signalling

Management-based activation

One company provides detail signalling in [3] for TP. The detail description is copied below:

Management-based activation The procedure is used for activating the QoE measurement triggered by OAM shown in figure 1. The OAM sends the QoE measurement configuration to gNB. gNB finds the qualified UEs that meet the criteria (e.g. application layer capability, service type, etc.). gNB sends the QoE measurement configuration to the UE. When a session starts, the application layer in UE checks the criteria (e.g. cell list, service type, etc.), if meets, start QoE measurement and reporting.

Proposal 1: Can we agree to capture into TR38.890 the Management-based activation signalling in [3] ?

| Company | Yes/NO | Comment |
| --- | --- | --- |
| ZTE | Yes with modification | 1:The step 0 is not necessary and should be removed. 2: Replace “gNB” with “NG-RAN node”. |
| Samsung | Yes | Step 0 is based on the common understanding in previous meeting “The UE Application layer measurement capability is indicated in the UE Radio Capability Info Indication message.” So it’s ok to capture step 0. As the objectives of QoE SI said that “In order to support 5G existing services (e.g. streaming services) and new emerging services in the future, the potential mechanisms to support for the following aspects should be studied in NR.”, gNB is more appropriate. So no need to change. |
| Huawei |  | This figure is just for management-based procedure, we think maybe we just need a general figure covering both management-based and signaling-based, e.g. in [20] |
| CATT |  | Agree to capture |
| LGE | Yes |  |
| China Unicom | Yes with intergration | Compared with the figure of signaling-based procedure, the only difference is the source to initiate, i.e. OAM and CN. If no further modification for both of them, agree with Huawei to integrate them as a general figure. Removing the Step 0 is OK since it’s one implicit aspect in the following criteria judgment. If ng-eNB shall be supported as well in NR QoE management, then “NG-RAN node” is preferred |
| Nokia | Yes | should add measurement reporting to TCE |
| Ericsson | We cannot the finalize the figures yet | We think that m-based QoE should have the possibility to target individual UEs, so the 2nd sentence in the text as well as the text in the rectangles below the gNB and UE in the figure need to be modified accordingly. In particular, we should clarify that either the exact intended UE is being found by RAN or the UE that satisfies the criteria. Alternatively, if we want to align with the s-based QoE figure below, then the rectangle below the gNB can say ‘Check criteria’. The text needs to state that an m-based QMC shall not overwrite a corresponding s-based existing configuration. The figure also needs to show: - Both the RRC and app layer of the UE; - AT commands between the RRC and app layer of the UE. The final versions of the figures cannot be agreed at this time because the figures should also align with all agreements from other comebacks. So, the figures are FFS. |
| Qualcomm |  | Step 2 can be rename to “RRC Message” similar to step 3. We are not sure whether RAN2 will use RRC Reconfiguration for QoE configuration. So, the RAN2 message name should be generic. |

Management-based deactivation

One company provides detail signalling in [3] for TP. The detail description is copied below:

The procedure is used for deactivating the QoE measurement triggered by OAM shown in figure. The OAM sends the deactivation indication to gNB to indicate which QoE measurement should be deactivated, then gNB sends it to UE, and UE stops corresponding measurement and reporting.

Proposal 2: Can we agree to capture into TR38.890 the Management-based deactivation signalling in [3] ?

| Company | Yes/NO | Comment |
| --- | --- | --- |
| ZTE | Yes with modification | Replace “gNB” with “NG-RAN node”. |
| Samsung | Yes | No need change, as the scope of this SI is only NR. |
| Huawei |  | See our comments for last proposal. |
| CATT | Yes |  |
| LGE | Yes |  |
| China Unicom | Yes with intergration | If the conclusion is that it’s similar with the figure of signaling-based procedure, agree with Huawei to integrate them as a general figure. |
| Nokia |  | Need to clarify what is deactivated (in NW / in UE AS (RRC) /in UE APP layer) |
| Ericsson | We, cannot finalize the figures yet | In the bottom right rectangle, it should say that there should be a check of whether there is an ongoing session pertaining to the QMC (according to the SA4 requirements). The measurement can be stopped only if there is no such ongoing session. Deactivation of measurements due to RAN overload should be discussed separately. The final versions of the figures cannot be agreed at this time because the figures should also align with all agreements from other comebacks. So, the figures are FFS. |
| Qualcomm |  | Similar to answer to last question. The RRC message name should be generic. |

Signalling-based signalling

Signalling-based activation

One company provides detail signalling in [3] for TP. The detail description is copied below:

The procedure is used for activating the QoE measurement triggered by CN shown in figure. The CN sends the QoE measurement configuration to gNB. gNB checks the criteria (e.g. application layer measurement capability, service type, etc.) for the UE. gNB sends the QoE measurement configuration to the UE. When a session starts, the application layer in UE checks the criateria (e.g. cell list, service type, etc.), if meets, start QoE measurement and reporting.

Proposal 3: Can we agree to capture into TR38.890 the Signalling-based activation signalling in [3] ?

| Company | Yes/NO | Comment |
| --- | --- | --- |
| ZTE | Yes with modification | 1:The step 0 is not necessary and should be removed. 2: Replace “gNB” with “NG-RAN node”. |
| Samsung | Yes | No need change, please refer to the Samsung’s view in previous proposals. |
| Huawei |  | Similar comments to last two proposal2, as a SI, maybe we just need a general figure to illustrate the deactivate or activate operation for both types of QoE measurements |
| CATT | Yes |  |
| LGE | Yes |  |
| China Unicom |  | See proposal 1 |
| Ericsson | We, cannot finalize the figures yet | OAM must be in the figure. OAM is always involved. Also, in the following sentence we should add the yellow part: “The procedure is used for activating the QoE measurement configured by OAM and triggered by CN”. The text needs to state that an m-based QMC shall not overwrite a corresponding s-based existing configuration. What is the purpose of having the “Check criteria” box in the s-based flavor? The figure also needs to show: - Both the RRC and app layer of the UE; - AT commands between the RRC and app layer of the UE. The final versions of the figures cannot be agreed at this time because the figures should also align with all agreements from other comebacks. So, the figures are FFS. |
| Qualcomm |  | Change gNB into NG-RAN Add OAM as Ericsson indicated Change RRC name to be generic. |

signalling-based deactivation

One company provides detail signalling in [3] for TP. The detail description is copied below:

The procedure is used for deactivating the QoE measurement triggered by CN . There are two options to trigger deactivation in this case. Option 1a is that the CN sends the deactivation indication to gNB to indicate which QoE measurement should be deactivated, then gNB sends it to UE, and UE stops corresponding measurement and reporting. Option 1b is that the CN sends UE PDU session or context related message to gNB. gNB finds that the criteria are no longer met for a specific QoE measurement after receiving the message. gNB sends the deactivation indication to indicate which QoE measurement should be deactivated, then UE stops corresponding measurement and reporting.

Proposal 4: Can we agree to capture into TR38.890 the Signalling-based deactivation signalling in [3] ?

| Company | Yes/NO | Comment |
| --- | --- | --- |
| ZTE | Yes with modification | 1: Replace “gNB” with “NG-RAN node”. 2: There is no description in LTE QoE that RAN need to consider PDU session/ UE context modification when deactive the QoE task. In order to take LTE solution as baseline, it is proposed to remove option 1b from the figure and description part. |
| Samsung | Yes, OK to remove option 1b | - no need change “gNB”, as the scope of this SI is NR. - Option 1b considers there may be new scopes in NR related to PDU sessions, which is discussed in other CB. OK to remove for this CB. |
| Huawei |  | See comments above. |
| CATT | Yes | Ok after remove 1b |
| LGE | Yes, but | Remove option 1b |
| China Unicom |  | See proposal 2 |
| Ericsson | We, cannot finalize the figures yet | OAM must be in the figure. OAM is always involved. Also, in the following sentence we should add the yellow part: “The procedure is used for deactivating the QoE measurement configured by OAM and triggered by CN”. In the bottom right rectangle, it should say that there should be a check of whether there is an ongoing session pertaining to the QMC (according to the SA4 requirements). The measurement can be stopped only if there is no such an ongoing session. Deactivation of measurements due to RAN overload should be discussed separately. Option 1b should be removed – it contradicts the SA4 requirements. The final versions of the figures cannot be agreed at this time because the figures should also align with all agreements from other comebacks. So, the figures are FFS. |
| Qualcomm |  | Step 1b seems not needed RRC message name should be generic |

Send an LS to SA5 (cc RAN2) relative to framework used for LTE QMC ?

In [8], the company observed that SA5 has developed LTE QoE measurements in TS 28.405 without dependency on MDT/Trace. The company observed that Coexistence between NR QMC and LTE QMC will have to be ensured independently of the framework chosen for NR QMC. Therefore, the LTE QMC framework needs to be clarified as part of the Rel-17 NR QoE. A draft LS is prepared for this purpose.

Proposal 5: Whether the draft LS in [9] is needed?

| Company | Yes/NO | Comment |
| --- | --- | --- |
| ZTE | Yes |  |
| Samsung | Pending until the LS content is clear and correct. | In TS38.300, it is said that “The feature is activated by Trace Function from the MDT framework”, meaning this feature uses the Trace Function, but not saying it’s based on MDT. If NR QoE is based on LTE QMC solution, the coexistence is ensured. We also observed that in the recent version of TS 28.405, the signaling-based solution is missed, if that’s the thing paper [9] wants to confirm with SA5, we are fine with the LS with a more accurate description. |
| Huawei | Yes | We could send LS to other groups updating the progress in RAN3 |
| CATT | Yes |  |
| LGE | Yes |  |
| China Unicom | Yes | Agree with Samsung, LS is needed when LS content is clear. |
| Ericsson | Yes, but | The text should be greatly simplified |
| Qualcomm | Yes |  |
| Nokia | Yes |  |

Send an LS to SA4 (cc RAN2) to inquire about their plans concerning application client requirements for NR services with respect to mandatory vs. optional support of OMA-DM and QMC

Based on observation from [8], the contribution raised a concern on CP based MDT mechanism due to OAM-DM mechanism defined in SA4.

However, SA4 specification mandates application clients in the UEs, or at least the MTSI client, to support QoE report configuration and reporting based on OMA-DM. OMA-DM is based on user plane transport and QoE reports based on such transport will not be visible at the RAN. OMA-DM enables activation of QoE reports for individual UEs (similar to MDT subscriber-based activation) but would not to our knowledge be suitable for area-based activation

Therefore , a draft LS can be found in [8] to ask SA4 provide clarification.

RAN3 would like to inform SA4 about discussions relative to visibility of QoE reports in the RAN and potential access stratum level actions relying on these QoE reports. RAN3 also observes that TS 26.114 specifies that an MTSI client that supports the QoE metrics feature shall support OMA-DM for activation/deactivation and gathering of QoE metrics in the MTSI client. The OMA-DM mechanism relies on user plane, and corresponding reports are therefore not visible in the RAN. LTE QMC functionality, based on control plane, is optional in the MTSI client. The UE provides its LTE QMC capability to the E-UTRAN, and the E-UTRAN further forwards this capability information to the EPC. RAN3 would therefore like to ask SA4 about current and planned status concerning application client requirements for NR services with respect to mandatory vs. optional support of QoE activation/deactivation and reporting based on OMA-DM and QMC.

Proposal 6: Whether the draft LS in [8] to SA4 is needed?

| Company | Yes/NO | Comment |
| --- | --- | --- |
| ZTE | Probably not. | Pending to the discussion in CB#2. |
| Samsung | Probably not. | Agree with ZTE |
| Huawei | Pending | Not sure the intention, maybe we need to reach a consensus before sending LS, since this OMA-DM is transparent to RAN? |
| CATT | Maybe | The SI will be closed soon, we may need to get confirm for some concern |
| China Unicom | Probably not. | Pending to the discussion in CB#2. |
| Ericsson | No | In our understanding, OMA-DM is likely obsolete. For instance, for 5GMS (5G Media Streaming) work SA4 have removed the OMA-DM way of handling QoE. |
| Qualcomm | No | As I know, OAM-DM based configuration is not really used. This is one reason that we use RRC for the configuration and reporting. |
| Nokia | yes | Agree this is related to the discussion in CB#2, but in order to get feedback before closure of the SI we believe the LS has to be sent from this meeting. |

Conclusion, Recommendations [if needed]

If needed

References

R3-206491 time plan update for NR QoE SI (China Unicom)

R3-206036 Discussion on NR QoE solutions (Samsung)

R3-206037 pCR for TR 38.890 NR QoE solutions, procedures and interface impacts (Samsung)

R3-206173 QoE metrics correlation with radio (Qualcomm Incorporated)

R3-206174 QoE aware by gNB (Qualcomm Incorporated)

R3-206360 Discussion on NR QoE solutions (CATT)

R3-206361 Discussion on RAN involved NR QoE (CATT)

R3-206369 Open points on QMC framework (Nokia, Nokia Shanghai Bell)

R3-206370 [Draft] LS on Framework for QoE Measurement Collection (Nokia, Nokia Shanghai Bell)

R3-206397 pCR for TR 38.890 Radio-related Measurements and Information in NR QoE Management (Ericsson)

R3-206398 pCR for TR 38.890 Mobility Support for NR QoE Management (Ericsson)

R3-206399 pCR for TR 38.890 Way Forward on Remaining Issues in NR QoE SI (Ericsson)

R3-206492 Further discussion on NR QoE solution (China Unicom)

R3-206493 Discussion on requirements and mechanisms for per slice QoE measurement (China Unicom, ZTE)

R3-206535 Further Consideration on QOE Configuration and Report (China Telecommunications)

R3-206713 Further Consideration On Study of NR QoE (ZTE)

R3-206714 TP for TR 38890 (ZTE)

R3-206732 Discussions on QoE report visibility at the RAN (Huawei)

R3-206733 Discussions on the radio related measurements and information as assistance to the NR QoE management functionality (Huawei, China Unicom, China Mobile)

R3-206734 Discussions on potential RAN3 impacts about the QoE measurement configuration, reporting and releasing under SA, NSA and MR-DC operation (Huawei, China Unicom)

R3-206787 Further discussion on NR QoE management (CMCC)

R3-206795 TP to 38.890 for NR QoE management (CMCC)

R3-206834 Discussion on signaling-based NR QoE management (LG Electronics)

R3-206175 Interworking with LTE QoE (Qualcomm Incorporated)

R3-206715 Consideration on slice QoE measurement (ZTE)