3GPP TSG-RAN WG3 Meeting #110-e	R3-207069

# 2 - 12 November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0645 | rev | 2 | Current version: | 16.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Uniqueness of BH RLC channel ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei, Samsung, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2020-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The IE definition of BH RLC Channel ID says “This IE uniquely identifies a BH RLC channel for an IAB-node and IAB-donor-DU”. On the other hand, in the definition of BH Information, the listed Egress BH RLC Channel IDs are coupled with Next-hop BAP Addresses to identify the link where the listed Egress BH RLC Channel ID applies, which suggests contrary to this claimed uniqueness of the BH RLC channel ID within a node. When the scope of uniqueness of the RLC Channel ID was discussed, the conclusion was to leave it up to network implementation, meaning that only uniqueness on a given link is guaranteed. Therefore, of the IE definitions of BH RLC Channel ID and BH Information, the former is erroneous. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the description for the RLC Channel ID IE to align with TS 38.331. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it clarifies the definition of an IE to align with other specification, e.g. TS 38.331. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incoherent specification wrt. the scope of uniqueness of BH RLC channel ID, and not align with other specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.113 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: resubmit to RAN3#110-e. Rev 2: updated during RAN3#110-e to align with TS 38.331. |  |  |  |  |  |  |  |  |

Start of change

#### 9.3.1.113	BH RLC Channel ID

This IE uniquely identifies a BH RLC channel in the link between IAB-MT of the IAB-node and IAB-DU of the parent IAB-node or IAB-donor-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BH RLC CH ID | M |  | BIT STRING (SIZE(16)) |  |

End of change