3GPP TSG RAN WG3#110-e	R3-207166

2nd November-12th November, 2020

Online

Title: 	(TP for SON BLCR 38.473) Introducing the successful HO report in F1AP

Source: 	China Telecom

Agenda item:	10.2.1.3

Document for:	Approval

Introduction

During the e-mail discussion in the CB: # 1003_SONMDT_SuccessHO, we have reached a consensus on reuse ACCESS AND MOBILITY INDICATION message to transmit Successful Handover report over F1 interface。

In this document, we provide the TP on Successful Handover Report for F1AP.

Text Proposal

Following changes depend on the 3GPP TS38.473 V16.3.0:

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

8.11.1	Access and Mobility Indication

8.11.1.1	General

This procedure is initiated by gNB-CU to send the Access and Mobility related Information to gNB-DU.

The procedure uses non-UE-associated signalling.

8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Access and Mobility Indication procedure. Successful operation

The Access and Mobility Indication procedure is initiated by ACCESS AND MOBILITY INDICATION message sent from gNB-CU to gNB-DU.

If the ACCESS AND MOBILITY INDICATION message contains the RACH Report Information List IE the gNB-DU shall take it into account for optimisation of RACH access procedures.

If the ACCESS AND MOBILITY INDICATION message contains the RLF Report Information List IE the gNB-DU shall take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful HO Report Information List IE the gNB-DU may take it into account for optimisation of mobility parameters.

8.11.1.3	Abnormal Conditions

Not applicable.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RACH Report Information List |  | 0..1 |  |  | YES | ignore |
| >RACH Report Information Item |  | 1 .. <maxnoofRACHReports> |  |  | - |  |
| >>RACH Report Container | M |  | OCTET STRING | RACH-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoof SuccessfulHOReports> |  |  |  | ignore |
| >>Successful HO Report Container | M |  | OCTET STRING | FFS on the definition | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHReports | Maximum no. of RACH Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64.FFS |

/////////////////////////////////////////////////////////////////////////change end////////////////////////////////////////////////////////////////////////////////////

Reference

- TR37.816, Study on RAN-centric data collection and utilization for LTE and NR, (Release 16)

- TS38.473, F1AP, (Release 16)

- TS38.401, Architecture description, (Release 16)

- R3-205510, Summary of Offline Discussion on Successful Handover Report, Samsung, 2020-08.

- R3-206879, Summary of Offline Discussion on Successful Handover Report, Samsung (moderator), 2020.11.