3GPP TSG-RAN WG3#111-e                                             	                     R3-210155

E-Meeting, 25 January – 04 February 2021

Agenda item:	9.3.4.1

Source:	Nokia, Nokia Shanghai Bell

Title: 	Solution for 4g-5g Direct Data Forwarding

Document for:	Approval

# Introduction

At the last RAN3 the case of 5g to 4g was solved and the case of 4g to 5g pending.

At this meeting, the 4g to 5g case is continued with the following two options

Open issue (To be continued next meeting):

Open issue1: Which solution should be adopted to support direct data forwarding from EPS to 5GS in case one DRB in target gNB contains QoS flows mapped to different E-RABs in the source eNB.

- Solution 1 [2]: The target CU-CP transmits the old flow to E-RAB mapping and new DRB configuration/mapping to the target CU-UP in the BEARER CONTEXT SETUP REQUEST message.

- Solution 2 [3]: The target CU-CP request for per QoS flow list data forwarding information towards target CU-UP and the target CU-UP provide corresponding data forwarding tunnel information during Bearer Context Setup procedure.

Open issue2: Further discuss on stage 2/stage 3 impact on how to support inter-system HO with shared SgNB

This paper addresses open issue 1.

# Discussion

The solution 1 is described in [2] with the following details:

The same as intra-system handover discussed in R3-206005, the QoS Flows Information To Be Setup IE in DRB To Setup Item in E1AP BEARER CONTEXT SETUP REQUEST messge can be used to transmit the new mapping. QoS Flows forwarded on the forwarding tunnel(s) in the DRB Data forwarding information Request IE can be used to transmit the old mapping. The CU-UP use the old mapping to handle the forwarded data and use the new mapping to handle the data received from the 5GC. This can also support the scenario if one DRB contains QoS flows mapped to different E-RABs.

For one DRB which is only used for handling forwarded packets, Source DRB Indication proposed in R3-206005 can help to indicate this to the gNB-CU-UP.

The above description means that for a same DRB 1 two QoS flows lists would be provided:

- One indicating the new mapping

- One indicating the old mapping

The idea is to set up at target side both old DRB and new DRB configuration, like for intra-system handover.

However, in case of inter-system handover, the PDCP is reset and a complete new 5g PDCP is used, different than 4g PDCP. There is no need of synchronization of PDCP between source and target sides, and therefore no need to have a useless transient DRB configuration which would correspond to “old E-RAB”.

Instead, the target gNB can directly remap the forwarded packets to the new DRB configuration. In case of different E-RAB/flow and DRB/flow mappings, the target gNB can apply a mapping to guarantee the QoS i.e. map the forwarded packets over an E-RAB tunnel to the DRB corresponding to the QoS flow of highest QoS among the forwarded QoS flows.

To make a clean design, solution 2 can be used to request the per E-RAB forwarding tunnel information.

Moreover, Solution 2 presented in tdoc [3] at last meeting can reuse the naming convention already agreed for the 5g to 4g direction.

For handover from 5GS to EPS, to provide data forwarding information, the name Data Forwarding Tunnel to E-UTRAN list is used. Similarly, for the case of EPS to 5GS the names Data Forwarding from E-UTRAN Request list IE and Data Forwarding  from E-UTRAN Response list IE could be introduced in Bearer Context setup Request message and Bearer Context setup Response message.

# Conclusion and Proposal

This paper has compared solutions 1 and 2 and proposed solution 2.

Proposal 1: select solution 2 and agree the CR in [4].

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO, Inc.

- R3-206015, Direct Data Forwarding for Inter-system Handover from 4g to 5g

- R3-206330, Support of Direct Data Forwarding for Inter-system Handover

- R3-210156, Support of Direct Data Forwarding for Inter-system Handover, CR TS 38.463