3GPP TSG-RAN WG3 Meeting #111 e	R3-210176

# Online, Jan 25 th - Feb 4th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.473 | CR |  | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BLCR37473 for Multi-USIM devices support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_MUSIM-Core |  |  |  |  |  | Date: |  |  | 2021-01-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Feature addition for Multi-USIM devices support |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new IE “Paging Cause” description in in successful operation of paging procedure. - Add a new IE “Paging Cause” in S1 PAGING message. - Add a new clause for “Paging Cause” description |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multi-USIM devices operation cannot be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.2 Successful Operation 9.1.6 Paging 9.2.1.x Paging Cause |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

## 8.6	Paging procedures

### 8.6.1	Paging

#### 8.6.1.1	General

The purpose of the Paging procedure is used to provide the paging information to enable the ng-eNB-DU to page a UE. The procedure uses non-UE associated signalling.

#### 8.6.1.2	Successful Operation

Figure 8.6.1.2-1: Paging procedure. Successful operation.

The ng-eNB-CU initiates the procedure by sending a PAGING message.

The Paging DRX IE may be included in the PAGING message, and if present the ng-eNB-DU may use it to determine the final paging cycle for the UE.

At the reception of the PAGING message, the ng-eNB-DU shall perform paging of the UE in cells which belong to cells as indicated in the Paging Cell List IE.

The Paging Cause IE may be included in the PAGING message, and if present the ng-eNB-DU may use it according to FFS[x].

#### 8.6.1.3	Abnormal Conditions

Not applicable.

NEXT CHANGE

### 9.2.5	Paging messages

#### 9.2.5.1	PAGING

This message is sent by the ng-eNB-CU and is used to request the ng-eNB-DU to page UEs.

Direction: ng-eNB-CU  ng-eNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| UE Identity Index value | M |  | 9.3.1.63 |  | YES | reject |
| CHOICE Paging Identity | M |  |  |  | YES | reject |
| >RAN UE Paging identity | M |  | 9.3.1.64 |  | - |  |
| >CN UE paging identity | M |  | 9.3.1.65 |  | - |  |
| Paging DRX | O |  | 9.3.1.66 | It is defined as the minimum between the RAN UE Paging DRX and CN UE Paging DRX | YES | ignore |
| Paging Cell List |  | 1 |  |  | YES | ignore |
| >Paging Cell Item IEs |  | 1 .. <maxnoofPagingCells> |  |  | EACH | ignore |
| >> E-UTRAN CGI | M |  | 9.3.1.12 |  | - |  |
| Paging Cause | O |  | 9.3.1.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPagingCells | Maximum no. of paging cells, the maximum value is 512. |

NEXT CHANGE

#### 9.3.1.x	Paging Cause

This element indicates the paging cause value for paging a UE as defined in FFS[x].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Cause | O |  | ENUMERATED (Voice, …) | If this IE is present, UE can decide to respond to the paging message or not, based on the paging cause value. |

END OF CHANGES