3GPP TSG-RAN WG3 #111-e	R3-210308

E-meeting, 25st January – 4th February 2021

Source:	CATT

Title:	(TP on SON for 38.473) Support of SUL PRB usage

Agenda Item:	10.2.1.5

Document for:	Discussion and decision

- Introduction

This is a TP based on R3-210306 “(TP on SON for 36.423) Discussion on PSCell MLB and SUL PRB usage”.

- TP for TS 38.473

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

9.3.1.129	Radio Resource Status

The Radio Resource Status IE indicates the usage of the PRBs per cell and per SSB area for all traffic in Downlink and Uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| SSB Area Radio Resource Status List |  | 1 |  |  | – |  |
| >SSB Area Radio Resource Status Item |  | 1..<maxnoofSSBAreas> |  |  | – |  |
| >>SSB Index | M |  | INTEGER (0..63) |  | – |  |
| >>SSB Area DL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL GBR PRB usage | – |  |
| >>SSB Area UL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL GBR PRB usage | – |  |
| >>SSB Area DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL non-GBR PRB usage | – |  |
| >>SSB Area UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL non-GBR PRB usage | – |  |
| >>SSB Area DL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area DL Total PRB usage | – |  |
| >>SSB Area UL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area UL Total PRB usage | – |  |
| >>DL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  | – |  |
| >>UL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  | – |  |
| >>SSB Area SUL GBR PRB usage | O |  | INTEGER (0..100) | Per SSB area SUL GBR PRB usage | YES | ignore |
| >>SSB Area SUL non-GBR PRB usage | O |  | INTEGER (0..100) | Per SSB area SUL non-GBR PRB usage | YES | ignore |
| >>SSB Area SUL Total PRB usage | O |  | INTEGER (0..100) | Per SSB area SUL Total PRB usage | YES | ignore |
| >>SUL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

//////////////////////////////////////////////////////////////end//////////////////////////////////////////////////////////////