3GPP TSG-RAN WG3 Meeting #111-e	R3-210402

E-meeting, 25 Jan – 5 Feb 2021

Title:	[draft] LS on Mobility Enhancement Optimization

Response to:

Release:	Rel-17

Work Item:	          NR_ENDC_SON_MDT_enh-Core

Source:	Huawei [will be RAN3]

To:	RAN2

Cc:	-

Contact Person:

#### Name:	Henrik Olofsson

E-mail Address:	henrik(dot)olofsson@huawei.com

- Overall Description:

Regarding SON enhancements for CHO and DAPS HO, RAN3 agreed that the UE can report the following information in the RLF report:

- the  reestablishmentCellId to indicate the successful CHO or DAPS HO cell;

- explicit HO type to indicate CHO or DAPS HO;

- a new cell information to indicate the selected CHO cell after the first connection failure and before the reestablishment;

- the time since the first connection failure until the second one;

- the time elapsed since the CHO execution until the (first) connection failure

- the time elapsed since the first connection failure until the successful RACH with the target DAPS HO cell;

- a new indicator to indicate whether the failure between the UE and the source cell occurred before the one with the target cell.

Besides, the first failure related information for CHO MRO and the failure related information in DAPS HO target cell in case of DAPS HO should be reported at least in case of two consecutive failures.

## 2. Actions:

To RAN2:

ACTION: RAN3 respectfully asks RAN2 to take the above into account and specify the corresponding UE behavior.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #112-e	    17th – 28th May 2021

TSG-RAN3 Meeting #113-e	    23th – 27th Aug 2021