3GPP TSG-RAN WG3 Meeting #111-e	R3-210428

# Online, January 25th – February 4th 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.415 | CR | 0019 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE support for D1 part of UL Delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh |  |  |  |  |  | Date: |  |  | 2021-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In current specification it is unclear if the UL Delay result includes the D1 part or not |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of one flag in UL PDU SESSION INFORMATION to signal if UL delay includes or not includes the D1 part |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear if the UL Delay Result includes or not the D1 part. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.2, 5.5.3.xx (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

//////////////////////////////////////// Change Start ////////////////////////////////////////////////

#### 5.5.2.2	UL PDU SESSION INFORMATION (PDU Type 1)

This frame format is defined to allow the UPF to receive some control information elements which are associated with the transfer of a packet over the interface.

The following shows the respective UL PDU SESSION INFORMATION frame.

| Bits |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6 | 5 | 4 | 3 |  | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  |  | QMP | DL Delay Ind. |  | UL Delay Ind. | SNP | 1 |
| N3/N9 Delay Ind. | UL Delay Result D1 Ind |  | QoS Flow Identifier |  |  |  |  |  |  | 1 |
| DL Sending Time Stamp Repeated |  |  |  |  |  |  |  |  |  | 0 or 8 |
| DL Received Time Stamp |  |  |  |  |  |  |  |  |  | 0 or 8 |
| UL Sending Time Stamp |  |  |  |  |  |  |  |  |  | 0 or 8 |
| DL Delay Result |  |  |  |  |  |  |  |  |  | 0 or 4 |
| UL Delay Result |  |  |  |  |  |  |  |  |  | 0 or 4 |
| UL QFI Sequence Number |  |  |  |  |  |  |  |  |  | 0 or 3 |
| N3/N9 Delay Result |  |  |  |  |  |  |  |  |  | 0 or 4 |
| Padding |  |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.2-1: UL PDU SESSION INFORMATION (PDU Type 1) Format

//////////////////////////////////////// Next change ////////////////////////////////////////////////

#### 5.5.3.xx	UL Delay Result D1 Ind

Description: This parameter indicates if the UL Delay Result includes or not includes D1 measurement.

Value range: {0= D1 measurement is not included, 1= D1 measurement is included}.

Field length: 1 bit.

//////////////////////////////////////// Change End ////////////////////////////////////////////////