3GPP TSG-RAN3 Meeting #111-e	R3-210483

E-meeting, 25 Jan – 5 Feb 2021

Title: 	Introduction of the survival Time

Source: 	Huawei

Agenda item:	21.3

Document Type:	Discussion

1. Introduction

The RAN2#112-e meeting replied to SA2 on the preference of survival time definition in [1] .

- On the issue of choosing whether Survival Time should be expressed as i) a maximum time in units of “time” where each unit corresponds to the data burst periodicity defined in TSCAI in Rel-16, or ii) a maximum number of consecutive data burst transmission failures, where a data burst corresponds to a single application message, RAN2 prefers option i) – to express Survival time in units of “time”. The exact wording of the relevant RAN2 agreement reached at the ongoing meeting is provided here for your convenience: Agreements => Time period during which “message loss” can be tolerated is adopted as the preferred format for Survival time. FFS how this will be achieved and what message loss means in RAN2.

In this document, we discuss this issue and propose the corresponding CRs.

2. Discussion

2.1 Survival time

2.1.1 Introduction of survival time

TS 22.104 [4] provides the requirements of survival time for different categories of vertical applications. According to the following Table-1, which is partly extracted from Table 5.2-1 in clause 5.2 of TS 22.104, the survival time of a motion control application equals to one transfer interval of the traffic. Thus, a motion control system is still working if only one message is lost, while the system will transition into a down state if two consecutive messages are lost. In order to guarantee the service availability, 5GS shall try its best to avoid the system enters down state, by resuming successful message transmission before the survival time expires.

Table-1 Periodic deterministic communication service performance requirements

| Influence quantity |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Message size [byte] (note 12a) | Transfer interval: target value (note 12a) | Survival time (note 12a) | UE speed (note 13) | # of UEs | Service area (note 3) | Remarks |
| 50 | 500 μs | 500 μs | ≤ 75 km/h | ≤ 20 | 50 m x 10 m x 10 m | Motion control (A.2.2.1) |
| 40 | 1 ms | 1 ms | ≤ 75 km/h | ≤ 50 | 50 m x 10 m x 10 m | Motion control (A.2.2.1) |
| 20 | 2 ms | 2 ms | ≤ 75 km/h | ≤ 100 | 50 m x 10 m x 10 m | Motion control (A.2.2.1) |

In SA2 #136AH meeting [5], it was agreed to add a key issue to TR 23.700-20 [5], which is on the use of survival time for deterministic applications in 5GS. This key issue is about how to transfer the survival time to RAN, i.e., how 5GS acquires the survival time. SA2 has studies whether 5GS can acquire survival time via interworking with CNC. In SA2 #139e meeting, SA2 sent a LS [6] to IEEE to enquire whether CNC is able to provide survival time that the 5GS can use for traffic processing. A LS [7] has been replied by IEEE. In the response, IEEE has clarified that the term “survival time” is not used in IEEE 802.1 TSN standards. A related application-centric communication characteristics is “tolerance to loss”. But the CNC does not communicate characteristics of applications or TSN streams towards the bridges, and may not even be aware of such information. Thus, IEEE TSN does not provide survival time via CNC.

However, based on SA2’s further evaluation as depicted in clause 7.3 of TS 23.700-20, SA2 agreed that AF can acquire survival time information, e.g. survival time can be pre-configured in the TSN AF, or survival time can be received by any AF from an application via direct provisioning or via other means which would be out-of-scope for 3GPP.

Observation 1: Survival time can be pre-configured in the TSN AF, or can be obtained by any AF from an application.

2.1.2 Survival time impact

SA2#142-e meeting has agreed the use of Survival Time for Deterministic Applications in 5GS, which is captured in TR 23.700-20:

- Survival Time information is conveyed together with TSCAI Periodicity parameter (the time between periodic TSC bursts) and burst size (e.g. MDBV).

- If the Survival Time information is specified by the AF in units of "time" with respect to TSN working domain burst periodicity, Survival Time needs to be mapped to the 5GS time domain by the SMF based on latest cumulative rateRatio between the TSN time and 5G time.

- Survival time specified in unit of “time” will be supported over NGAP.

Therefore, the corresponding RAN3 impact should be captured in TS 38.413. Given that SA2 just begins its WI phase, it is too early to propose the detailed IE format.

One example of the signalling format is given in the Annex.

- Add the survival time per QoS Flow, in units of "time" with respect to burst periodicity.

2.2 Communication service availability

In the SA2 LS [2], SA Request RAN2 and RAN3 to inform SA2 whether receiving survival time is sufficient for NG-RAN to address the performance targets (same Survival Time but different communication service availability for different services) laid out by SA1 in Table 5.2.1 in TS 22.104. In TS 22.104, there is a following description about communication service availability:

The availability of the communication service is calculated using the accumulated down time. For instance, in case the communication service is expected to run for a time T, the unavailability U of the communication service can be calculated as U=i∆tiT Where Δti is the length of the i-th downtime interval of the communication service within the time period T. The communication service availability A can then be calculated as A = 1–U.

Based on above text, we think the communication service availability has a relationship with the traffic periodicity, the frequency that the application enters down state, and the average downtime interval. The frequency that the application enters down state is further decided by the service reliability which indicates the probability of a packet loss, and survival time which indicates the number of consecutive lost packets leading to the application’s down. In order to guarantee the communication service availability of an application, the SMF shall derive the appropriate service reliability according to its communication service availability, survival time, traffic periodicity, as well as its average downtime interval. The service reliability will be indicated to the RAN node in the form of 5G QoS characteristics, which is enough for RAN node for configuration and scheduling determination.

Therefore, instead of the communication service availability, we think some additional QoS parameters similar to that in 5G QoS characteristics might be more useful for RAN. For example, the 5G QoS characteristics of the QoS flow can be used by the RAN node to fulfil a moderate/acceptable service reliability. If packet loss is detected, the additional QoS parameters can assist the RAN node to improve the service reliability to a reasonably higher level for survival time expiry avoidance. Such a scheme can guarantee the communication service availability with efficient resource utilization. The additional QoS parameters shall be able to reflect the level of service reliability requirement, and the details of the additional QoS parameters can be further studied.

- Instead of the communication service availability, additional QoS parameters reflecting level of service reliability requirement can be used by RAN for survival time expiry avoidance if RAN detects a message loss.

3. Conclusion

Based on the discussion in this paper, we propose the following:

Observation 1: Survival time can be pre-configured in the TSN AF, or can be obtained by any AF from an application.

- Add the survival time per QoS Flow, in units of "time" with respect to burst periodicity.

- Instead of the communication service availability, additional QoS parameters reflecting level of service reliability requirement can be used by RAN for survival time expiry avoidance if RAN detects a message loss.

4. Reference

- R2-2010838, LS response to SA2, Samsung

- R3-206839, LS on Use of Survival Time for Deterministic Applications in 5GS (S2-2007880; contact: Nokia).

- RAN3#110-e Chairman notes.

- 3GPP TS 22.104 V17.3.0, Service requirements for cyber-physical control applications in vertical domains; Stage 1 (Release 17).

- Draft Report of SA WG2 meeting #136AH, SA WG2 Meeting #136AH, 13 - 17 January, 2020, Incheon, South Korea.

- S2-2004653, LS on need for survival time for IEEE TSN, 3GPP TSG-WG SA2 Meeting #139E e-meeting, June 1 – 12, 2020, Electronic.

- S2-2004934, Response to LS on the need for survival time, IEEE 802.1.

5. Annex

9.3.1.131	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.3.1.132 |  |
| Burst Arrival Time | O |  | 9.3.1.133 |  |
| Survival Time | O |  | 9.3.1.aaa |  |

<Unchanged Text Omitted>

9.3.1.aaa	Survival Time

This IE indicates the Survival Time of the TSC QoS flow as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Survival Time | M |  | INTEGER (0..256, …) | Survival Time expressed in units of burst periodicity. |