3GPP TSG-RAN3 Meeting #111-e	R3-210507

Electronic meeting, 25 January – 4 February 2021

Agenda item:	15.2

Source: 	Samsung

Title: 	Open issues in NR QoE solutions

Document for:	Discussion & Approval

Introduction

In RAN3 110E Meeting, a lot of agreements had been captured in TR, however, some definitions of NR QoE management are still not clear, since this will be the last meeting for this study item, the ambiguous words should be defined for better understanding the content in the TR.

In addition, although some issues (e.g. multiple QMC support, RAN overload handling) are pending to RAN2 decision, RAN2 will discuss those issues at the same time during RAN3 111E meeting, in this paper, we would like to discuss the RAN3 impact on those issues.

Discussion

2.1 Definitions clarification

In the latest TR, some descriptions are ambiguous, for example, what’s the difference between “deactivate QoE measurement”, “stop QoE measurement” and “release measurement configuration”, according to the LTE QMC, all of them are the same, which means when a deactivation is sent to UE, UE release configuration and then stop measurement based on SA4 defined mechanism in application layer. The same thing happens in “QoE measurement reporting” and “QoE measurement collection”

Observation 1: the descriptions related to “deactivation/stop/release”, “collection/reporting” are not clear and ambiguous in the TR.

Proposal 1: definitions of “activation of QoE measurement”, “deactivation of QoE measurement”, “QoE measurement collection” and “QoE measurement reporting” should be clarified.

2.2 (De)activate multiple QoE measurements for a UE

Unlike LTE QMC in which only two services are considered, diverse services and new emerging services should be considered in NR QoE. A single UE may have one or more QoE measurement(s) configured at the same time, which means activate and deactivate one or more QoE measurement(s) simultaneously for a UE should be supported in NR QoE management.

To support activate one or more QoE measurement(s) configuration simultaneously, it is proposed to include QoE measurement list in QoE measurement configuration.

To support deactivate one of the multiple QoE measurements, it is proposed to include QoE reference list in deactivation configuration to notify UE which QoE measurement job should be deactivated.

Both of them should be supported in management based and signaling based QoE measurement collection, which means support multiple QoE measurements for a single UE has impact on both NG and Xn from RAN3 perspective.

Proposal 2: To support activate one or more QoE measurement configuration(s) simultaneously, it is proposed to include QoE measurement configuration list in one QoE measurement configuration related message.

Proposal 3: To support deactivate one of the multiple QoE measurements, it is proposed to include QoE reference list in deactivation configuration to notify UE which QoE measurement collection job should be deactivated.

Proposal 4: multiple QoE measurement collection for a single UE should be supported for both management-based and signaling-based QoE measurement collection.

2.3 RAN overload handling during mobility

When RAN overload, the on-gonging QoE reporting will be stopped. However, the QoE when RAN overload is more important for operators than any other time. To keep the integrity of QoE reports even in RAN overload, two solutions can be considered:

- Suspend QoE reporting in UE AS layer, the QoE measurement in UE application layer is not change, the QoE reports will be buffered in UE AS layer, which will be sent to gNB when overload ends.

- Prolong QoE report interval in UE AS layer, the QoE measurement in UE application layer is not change, the QoE reports will be buffered in UE AS layer, which will be sent to gNB with a longer report interval

Both of the solutions will have impact during mobility, if the reporting is modified in the source gNB because of RAN overload, the target gNB should know that and may modify it according to its own load situation, and vice versa.

So during the handover, the reporting configuration should be sent to the target gNB, the target gNB can decide whether change the reporting or not based on the load situation.

Proposal 5: besides suspend QoE reporting, modifying the QoE report interval (e.g. prolong the QoE reporting period) should also be supported.

Proposal 6: reporting configuration should be included in QoE measurement configuration during mobility.

According to our analysis, we propose a TP for TR38.890 in the appendix.

Proposal 7: It is proposed RAN3 to agree the TP for TR 38.890-v0.2.0 in the appendix.

Conclusion

In this paper, we analyse the potential solutions and specification impact, the following are proposals:

Observation 1: the descriptions related to “deactivation/stop/release”, “collection/reporting” are not clear and ambiguous in the TR.

Proposal 1: definitions of “activation of QoE measurement”, “deactivation of QoE measurement”, “QoE measurement collection” and “QoE measurement reporting” should be clarified.

Proposal 2: To support activate one or more QoE measurement configuration(s) simultaneously, it is proposed to include QoE measurement configuration list in one QoE measurement configuration related message.

Proposal 3: To support deactivate one of the multiple QoE measurements, it is proposed to include QoE reference list in deactivation configuration to notify UE which QoE measurement collection job should be deactivated.

Proposal 4: multiple QoE measurement collection for a single UE should be supported for both management-based and signaling-based QoE measurement collection.

Proposal 5: besides suspend QoE reporting, modifying the QoE report interval (e.g. prolong the QoE reporting period) should also be supported.

Proposal 6: reporting configuration should be included in QoE measurement configuration during mobility.

Proposal 7: It is proposed RAN3 to agree the TP for TR 38.890-v0.2.0 in the appendix.

References

- TR 38.890

Appendix: Text Proposal

Start of 1st change

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

QoE measurement: An application layer measurement configured by OAM, see details in [3] [4] [5] [6] for different service type.

QoE report: The result of a QoE measurement.

Radio-related measurements: Measurements on the radio layer, whose purpose is to help network to further evaluate and improve the QoE.

Radio-related information: Information other than “radio-related measurements”, e.g. feature info, mobility history info or dual connectivity status. FFS on Radio related information only from UE or RAN node or both.

QoE measurement collection activation: activation of measurement collection job associated with QoE reference, the QoE measurement configuration is transmitted to UE.

QoE measurement collection deactivation: deactivation of measurement collection job associated with QoE reference, the QoE measurement configuration is released in UE.

QoE measurement collection: collection of QoE measurements in UE application layer.

QoE measurement reporting: the reporting of the QoE report, which is different from QoE measurement collection

Editor’s NOTE: The above three definitions may subject to further refinements once further consensus are reached.

End of 1st change

Start of 2nd change

6.3 	QoE measurement triggering and stopping

QoE measurement triggering and stopping can be realized using time-based and/or threshold-based criteria, configured by the OAM. Time-based QoE measurement triggering and stopping in NR is achieved by reusing mechanisms specified in LTE for the start and stop of QoE measurements. Threshold-based QoE measurement triggering and stopping allows to start and stop QoE measurement when given thresholds are passed.

Meanwhile, RAN is not allowed to intervene, i.e. pause, activate or de-activate an ongoing QoE measurement collection unless instructed otherwise by the OAM.

Activate multiple QoE measurement collection jobs simultaneously for different service types for a single UE should be supported both management-based and signaling-based QoE measurement collection.

6.4 	Release of QoE measurement configuration

An NG-RAN node can issue a release of QoE measurement configuration for UEs previously configured for QoE measurement reporting, provided that the session for which the QoE measurements are reported is completed.

Deactivate one or more of the multiple QoE measurement collection jobs simultaneously for a single UE should be supported both management-based and signaling-based QoE measurement collection.

Editor’s NOTE: FFS whether RAN is allowed to release an ongoing QoE measurement reporting; to be confirmed by RAN2.

6.5 	QoE measurement handling at RAN overload

In case of RAN overload in standalone connectivity, RAN can stop new QoE measurement configurations, release existing QoE measurement configurations, pause QoE measurement reporting and modifying the reporting period configuration. FFS for details under EN-DC/MR-DC operation.

6.5.X 	Reporting configuration handling during mobility

If QoE measurement reporting is paused or modified, it should be indicated to the target gNB during handover, and the target gNB can notify UE to continue QoE measurement reporting in case there is no overload in the target gNB and vice versa.

end of change