3GPP TSG-RAN3 #111		R3-210525

25th Jan– 4th Feb 2021

Online

Agenda Item:	17.2

Source:	Ericsson

Title:	TP for 38.823 – structure of slice re-mapping solutions chapter

Document for:	Discussion, TP

# Introduction

The structure of chapter 6.2 in TR 23.823 is not consistant and would need to be better organized.

In this contribution we propose to re-organize the paper so that related solutions are placed in same sub-chapter, and it is clear what sequence diagram corresponds to the different solutions.

# Discussion

In RAN3 #109, the chapter 6.2 was divided into two chapters: 6.2.1“Re-mapping Policy in target NG-RAN node” and 6.2.2 – “Slice Re-mapping Message Sequence Charts”. However, in RAN3#110, new solutions without sequenc charts were added to section 6.2.2, and new chapters, 6.2.3 – 6.2.8 were added with solutions, some corresponding to sequencye charts in section 6.2.2, and some complete with sequence charts.

There are some issues with this structure:

Some solutions can not be referenced since they are not placed in a section with individual numbering. It is unclear what solutions the sequence charts corresponds to, and is is unclear if the subsections in chapter 6.2.2 curresponds to full solutions, or if there are solution descriptions in other chapters.

Also, some solutions are grouped in same section, while some are own sections, not even all solutions with re-mapping policy in target RAN are placed in the corresponding chapter.

To mitigate these issues, we propose:

- All solutions are placed in own sub-section with section numbering, so that they can be referenced.

- Sequence charts are placed in the same section as the corresponding solution. If same sequence chart is used for several solutions, they are included in the section of the first solution, and referenced in following solutions.

- The solutions are divided into 4 chapters: 6.2.1 Legacy solutions, 6.2.2 Remapping Policy in target NG-RAN node; 6.2.3 Remapping Policy in 5GC; and 6.2.4 Other solutions.

In section 6.3 “Configuration Based Solution”, both scenario 1 and 2 are discussed. For scenario 1 a solution is described, but for scenario 2, it is only explained why re-mapping is not possible in legacy system, but no solution is described.

Observation 1	The discussion on scenario 2 in section 6.2.3 is not a solution and should not be in the solution chapter.

- The discussion on Scenario 2 is removed from section 6.2.3.

- RAN 3 should discuss whether more text should be added for scenario 2 in section 6.1 capturing the discussion on scenario 2 in section 6.2.3.

In the following, we provide a TP for 23.823, based on the discussion above.

# Conclusion

Observation 1	The discussion on scenario 2 in section 6.2.3 is not a solution and should not be in the solution chapter.

- All solutions are placed in own sub-section with section numbering, so that they can be referenced.

- Sequence charts are placed in the same section as the corresponding solution. If same sequence chart is used for several solutions, they are included in the section of the first solution, and referenced in following solutions.

- The solutions are divided into 4 chapters: 6.2.1 Legacy solutions, 6.2.2 Remapping Policy in target NG-RAN node; 6.2.3 Remapping Policy in 5GC; and 6.2.4 Other solutions.

- The discussion on Scenario 2 is removed from section 6.2.3.

- RAN 3 should discuss whether more text should be added for scenario 2 in section 6.1 capturing the discussion on scenario 2 in section 6.2.3.

# Text proposal for TS 38.832

## 6.2	Solutions

Editor Note: Capture the solutions for the scenario and issue.

### 6.2.1	Legacy Solutions

#### 6.2.1.1	Configuration Based Solution

- Scenario 1: Slice resource shortage in case of Intra-RA mobility and Inter-RA mobility

As specified in TS 28.541, the slice re-mapping between different S-NSSAIs can be achieved via the prioritized resource modeling. For example, suppose UE’s ongoing slice is S-NSSAI 1 configured with rRMPolicyMaxRatio policy, which can use at least one of the shared resources, prioritized resources and dedicated resources. If the dedicated resources are not available, it can use other un-used prioritized and shared resources.

But the following needs to be further studied, e.g., for the S-NSSAI 1,

  - it can explicitly use resources belonging to which S-NSSAIs;

  - it can use the dedicated but not used resources of other S-NSSAIs;

  - it can preempt the used prioritized and/or shared resources from other S-NSSAIs.

In this case, further involvement with SA5 is required.

#### 6.2.1.2 Multi-carrier radio resource sharing

This solution is applicable to scenario 1. In this solution, it is assumed that radio resources are primarily assigned to a slice (or slice sets) on a frequency, or cell, basis. For example, a RAN node may host two layers as shown below:

Figure 6.2.6-1: RAN node supporting two layers

The solution addresses temporary resource shortage in one cell as per scenario 1, and where the RAN node hosts another cell with different frequency and overlapping coverage where the same slice is available.

In above, this could be the case for slice 1 and cell 1/F2 (or also slice 1 and cell 2/F1).

The solution consists of setting up DC or CA using user plane resources of F1 (or F2), for some or all UEs with slice1 PDU sessions. This action can be wholly decided by the RAN node, without referring to the CN or other nodes. This solution can be seen as fallback planning in the RAN.

### 6.2.2	Re-mapping Policy in target NG-RAN node

Editor note: Feasibility of this solution at system level requires further work including checking with SA2.

In solutions where the target NG-RAN node decides the re-mapping at incoming handover, the target NG-RAN node should be aware of the re-mapping policy for the involved PDU session. The following options are available:

#### 6.2.2.1	Configuration in target NG-RAN node

General Description

This option assumes that the remapping policy is rather static because it should have been validated by the tenant or the operator.

Therefore, the NG-RAN node is configured in advance with the re-mapping policy by the OAM.

In this option, the granularity of the re-mapping policy is per slice i.e. for each supported S-NSSAI, the target NG-RAN node is configured with a list of possible re-mapped S-NSSAI(s) as follows:

- S-NSSAI 1 <> re-mapped list (S-NSSAI 10, S-NSSAI 11)

- S-NSSAI 2 <> re-mapped list (S-NSSAI 12, S-NSSAI 13)

Sequence Chart, Xn based handover

Figure 6.2.2.1-1: Slice re-mapping/fallback determined by the T-gNB

- The S-gNB sends the HANDOVER REQUEST message to the T-gNB.

- If the UE’s ongoing slice(s) is rejected in the target gNB, based on the slice re-mapping policy described in section 6.2.1, the T-gNB makes the slice re-mapping/fallback decision. The T-gNB may send the slice re-mapping/fallback decision in the HANDOVER REQUEST ACKNOWLEDGE message to the S-gNB.

- The T-gNB shall send the slice re-mapping/fallback decision to the AMF through the PATH SWITCH REQUEST message.

- The AMF responds the PATH SWITCH REQUEST ACKNOWLEDGE message. The AMF may reject the PDU sessions in the PDU Session Resource Released List IE.

Editor Note:  It is FFS whether and how the UE is aware of slice remapping.

Sequence Chart, NG based handover

Figure 6.2.2.2-1: Slice re-mapping/fallback determined by the T-gNB

- The S-gNB sends the HANDOVER REQUIRED message to the AMF.

- The AMF sends the HANDOVER REQUEST message to the T-gNB.

- If the UE’s ongoing slice(s) is rejected in the target gNB, based on the slice re-mapping policy described in section 6.2.1, the T-gNB shall include the re-mapped/fallback decision in the HANDOVER REQUEST ACKNOWLEDGE message to the AMF.

- The AMF may send the slice re-mapping/fallback decision to the S-gNB through the HANDOVER COMMAND message.

Editor Note:  It is FFS whether and how the UE is aware of slice remapping.

Evaluation

#### 6.2.2.2	Signaling in NG Setup Response

General Description

The NG-RAN node has received in advance the re-mapping policy in the NG Setup Response message (or any update in the AMF configuration Update message) from the CN.

In this option the granularity of the re-mapping policy is the slice i.e. for each S-NSSAI supported by the target NG-RAN node, the CN includes in the NG Setup Response (respectively AMF Configuration Update) message an associated list of possible re-mapped S-NSSAI(s).

Sequence Charts

For signaling during HO, see section 6.2.1.1

Evaluation

#### 6.2.2.3	Signaling in NG Handover Request

#### General Description

At the time of handover, the CN includes in the NG Handover Request message the current PDU Session, the associated S-NSSAI and also the list of S-NSSAI(s) to which this PDU session can be re-mapped.

In this option the granularity of the re-mapping policy can be either:

- Per PDU session (using same principles as slice association in PDU Session Setup)

- Per UE: even though signaled for the involved PDU session, the choice of possible re-mapped slices for a given slice is a general policy for the UE.

Example of per UE policy:

UE 1, any PDU session of S-NSSAI 1 <> re-mapped list (S-NSSAI 10, S-NSSAI 11)

UE 2, any PDU session of S-NSSAI 1 <> re-mapped list (S-NSSAI 12, S-NSSAI 13)

Example of per PDU session policy:

UE 1, PDU Session 1, S-NSSAI 1 <> re-mapped list (S-NSSAI 10, S-NSSAI 11)

UE 1, PDU Session 2, S-NSSAI 1 <> re-mapped list (S-NSSAI 12, S-NSSAI 13)

UE 2, PDU Session 3, S-NSSAI 1 <> re-mapped list (S-NSSAI 14, S-NSSAI 15)

Sequence Charts

Evaluation

6.2

#### .2.	Signaling from Source NG-RAN node

General Description

When the PDU session is created in the source NG-RAN node, the CN includes in the NGAP PDU Session Resource Setup Request message (or the Initial Context Setup Request message or the NG Handover Request message) the S-NSSAI associated with the PDU session and also the list of S-NSSAI(s) to which this PDU session can be re-mapped.

At the time of subsequent Xn handover, the source NG-RAN node includes in the Xn Handover Request message the current PDU Session, the associated S-NSSAI and also the list of S-NSSAI(s) to which this PDU session can be mapped.

In this option the granularity of the re-mapping policy can be either:

- Per PDU session (using same principles as slice association in PDU Session Setup)

- Per UE: even though signaled for the involved PDU session, the choice of possible re-mapped slices for a given slice is a general policy for the UE.

### Sequence Charts

For signaling during HO, see section 6.2.1.1

Evaluation

#### 6.2.2.5	Slice Remapping Solution for Scenario 6

At the same time the NG-RAN node may notice that another slice 2 which is not overloaded has resources available and is still compatible with the SLA of slice 1.

In short, there is a potential that some unloaded but "good enough or better" alternative slices in the RAN could be used for the subscriber to continue to receive service.

#### 6.2.2.6 Slice resource re-partitioning

Editor note: Feasibility of this solution at system level requires further work including checking with SA5.

This solution is applicable to scenario 1. In this solution, the resource limits for a particular slice in the RAN are relaxed (possibly for a limited time period). This is applicable for resource types which have been hard-partitioned between slices, or where a limit per slice has been defined according to the SLA. For example, such an approach could be applied individually (or jointly) to the following:

- spectrum resource (e.g. slots, beams, carriers etc)

- transport resources (e.g. backhaul capacity)

- hardware resources (e.g. specific processors, processing load, intra-RAN logical nodes such as a gNB-CU-UP)

To solve this problem, the system can allow a slice to use another slice’s resources on a temporary basis i.e. making the partition soft. The RAN may allow such temporary overflow while keeping some form of accounting of resources used which may be used to modify the existing SLA, or provide reporting.

Re-partitioning policy may be configured in the RAN.

The solution may have impacts in metric collection and OAM requirements, but does not impact the core network or the UE.

### 6.2.3	Slice re-mapping decision in 5GC

### 6.2.3.1	5GC Solution based on SSC-mode 3

The call flow below uses SSC mode 3 in 5GC as the service continuity solution:

Source NG-RAN node 1

5GC

Step 2: UE has ongoing PDU session 1 of slice 10

UE

Target NG-RAN node 2

Step 3: HO request (pdu session 1)

Handover command (PDU session 1 temporary accepted)

Step 6: NAS PDU Session Modification Command (end slice 10, new slice 11)

Handover complete

Step 7: Establish resources for PDU session 2 on slice 11

Step 8: Release PDU session 1 of slice 10

SSC mode 3 Timer expiry

Step 8: UE Configuration Update (new Allowed NSSAI= slice 11 only)

Step 4: PSR (end slice 10, new slice 11)

Register Accept (Allowed NSSAI (slices 10, 11)

Step 5: Register Request

Step 1: UE Allowed NSSAI (slices 10, 11)

Figure 6.2.7-1 Re-mapping based on SSC mode 3

Step 0: NG-RAN nodes have been configured with slice re-mapping slice 10 to 11.

Step1: 5GC has sent the UE Allowed NSSAI to the serving NG-RAN node and to the UE per existing procedures

Step 2: UE has ongoing PDU session 1 of slice 10.

Step 3: Source NG-RAN triggers Handover to target NG-RAN. The target NG-RAN node 2 informs during the HO procedure the source NG-RAN node 1 that it accepts the PDU session 1 of slice 10 temporarily due to slice re-mapping action.

#### Step 4: at handover

Figure 6.2.2.1-1: Slice re-mapping/fallback determined by the T-gNB

- The S-gNB sends the HANDOVER REQUEST message to the T-gNB.

- If the UE’s ongoing slice(s) is rejected in the target gNB, based on the slice re-mapping policy described in section 6.2.1, the T-gNB makes the slice re-mapping/fallback decision. The T-gNB may send the slice re-mapping/fallback decision in the HANDOVER REQUEST ACKNOWLEDGE message to the S-gNB.

- The T-gNB shall send the slice re-mapping/fallback decision to the AMF through the PATH SWITCH REQUEST message.

- The AMF responds the PATH SWITCH REQUEST ACKNOWLEDGE message. The AMF may reject the PDU sessions in the PDU Session Resource Released List IE.

Editor Note:  It is FFS whether and how the UE is aware of slice remapping.

completion, the target NG-RAN indicates to 5GC in Path Switch Request that PDU session 1 of slice 10 needs to be terminated and a new PDU session is to be setup with slice 11.

Step 5: The UE performs the post-handover registration (as Source and Target NG RAN nodes have different slice support, they don’t belong to the same registration area for the UE). Because 5GC received (end slice 10) at step 4, the 5GC still includes the slice 10 in the Allowed NSSAI towards the UE at this step (the slice is indeed still temporarily available until it receives from 5GC notification of the final release of PDU session 1 of slice 10 at step 9).

Step 6: In reaction to step 4, the 5GC triggers towards the UE the NAS PDU Session Modification Command to invoke SSC mode 3. The (end slice 10, new 11) may be included towards the UE to prompt the UE to setup the new PDU session 2 with slice 11 even if the URSP in the UE would indicate slice 10 as higher priority.

Step 7: the UE triggers the setup of PDU session 2 with slice 11 according to SSC mode 3 procedure as per existing procedures described in 23.502 § 4.3.2.2.1.

Step 8: at the expiry of SSC mode 3 timer, the 5GC triggers the release of the PDU session 1 of slice 10 according to SSC mode 3 procedures (existing procedures described in 23.502 § 4.3.2.2.1). The 5GC sends a final the UCU (UE Configuration Update) message in order to update the Allowed NSSAI towards the NG-RAN and the UE. In this example, the new Allowed NSSAI is slice 11.

Editor note: This solution is CN-centric and requires confirmation from SA2.

### 6.2.3.2 Slice Remapping decision in 5GC

General Description

#### This solution is applicable for scenario 2, when a UE with bearers associated to a given slice, e.g. S-NSSAI1, wants to be handed over to a target cell and where S-NSSAI1 is not supported in the target cell. At NG based

Figure 6.2.2.2-1: Slice re-mapping/fallback determined by the T-gNB

- The S-gNB sends the HANDOVER REQUIRED message to the AMF.

- The AMF sends the HANDOVER REQUEST message to the T-gNB.

- If the UE’s ongoing slice(s) is rejected in the target gNB, based on the slice re-mapping policy described in section 6.2.1, the T-gNB shall include the re-mapped/fallback decision in the HANDOVER REQUEST ACKNOWLEDGE message to the AMF.

- The AMF may send the slice re-mapping/fallback decision to the S-gNB through the HANDOVER COMMAND message.

Editor Note:  It is FFS whether and how the UE is aware of slice remapping.

HO, the AMF will detect that the target cell is not supporting S-NSSAI1 or that the Allowed NSSAI in the target cell for the UE does not include S-NSSAI1. The 5GC will then decide if the PDU sessions associated to S-NSSAI1 can be re-mapped to another slice. The new S-NSSAI is signalled with the HO Request, using legacy signalling, and there is no impact to the target gNB.

When Xn HO can be used, but the target gNB does not support all slices of an UE, the source gNB will use NG based HO instead, so that 5GC may re-map the slice.

At the end of the HO the UE will be updated with the new Allowed NSSAI through legacy NAS procedures. The original slice will be included in the Rejected NSSAI, and the UE will not be allowed to access it as long as it stays in the current RA. Once the UE enters a new RA, it may request to add the slice to the Allowed NSSAI, and the PDU sessions may be re-assigned to the original S-NSSAI1.

The granularity of slice remapping in this solution is per PDU session. The re-mapping decision can be based on slice awareness in registration area, operator policy for slice re-mapping as well as the subscription of the UE.

Sequence Chart

Figure 6.2.2.3-1: Slice re-mapping/fallback determined by the AMF

- The S-gNB sends the HANDOVER REQUIRED message to the AMF.

- If the UE’s ongoing slice(s) is not supported by the T-gNB, the AMF may make the slice re-mapping/fallback decision and include the decision in the HANDOVER REQUEST message to the T-gNB.

- The T-gNB responds to the AMF through the HANDOVER REQUEST ACKNOWLEDGE message.

- The AMF may send the slice re-mapping/fallback decision to the S-gNB through the HANDOVER COMMAND message.

Editor Note:  It is FFS whether and how the UE is aware of slice remapping.

Evaluation

System Impact

No impact on signalling protocols. gNB should be aware that re-mapping may be used, and select NG based HO when needed.

### 6.2.4	Other Solutions

#### 6.2.4

At the same time the NG-RAN node may notice that another slice 2 which is not overloaded has resources available and is still compatible with the SLA of slice 1.

In short, there is a potential that some unloaded but "good enough or better" alternative slices in the RAN could be used for the subscriber to continue to receive service.

#### .1 Slice Remapping decision in 5GC and target gNB at NG based handover

Figure 6.2.2.5-1: Slice re-mapping/fallback determined by the AMF and T-gNB

- The S-gNB sends the HANDOVER REQUIRED message to the AMF.

- If the UE’s ongoing slice(s) is not supported by the T-gNB, the AMF may make the initial slice re-mapping/fallback decision and include the decision in the HANDOVER REQUEST message to the T-gNB.

- If the UE’s ongoing or re-mapped/fallback slice(s) is rejected in the target gNB, based on the slice re-mapping policy described in section 6.2.1, the T-gNB shall include the further re-mapped/fallback decision in the HANDOVER REQUEST ACKNOWLEDGE message to the AMF.

- The AMF may send the slice re-mapping/fallback decision to the S-gNB through the HANDOVER COMMAND message.

Editor’s note: The efficiency of the solution needs to be further evaluated.

#### 6.2.4.2 Slice Remapping decision in SN for MR-DC case

Figure 6.2.2.6-1: Slice re-mapping/fallback determined by the SN

This flow chart applies to the scenario of resource shortage only.

- The MN sends the SN Addition Request message to the SN.

- If the UE’s ongoing slice(s) is rejected by the SN, based on the slice re-mapping policy described in section 6.2.1, the SN makes the slice re-mapping/fallback decision. The SN shall include the slice re-mapping/fallback decision in the SN Addition Request Acknowledge message to the MN.

- The MN may send the slice re-mapping/fallback decision to the AMF through the PDU Session Modification Indication message.

- The AMF responds the PDU Session Modification Confirmation message.

#### 6.2..3 Slice Remapping decision in MN for MR-DC case

Figure 6.2.2.7-1: Slice re-mapping/fallback determined by the MN

This flow chart applies to the scenario of resource shortage only.

- The MN makes the slice re-mapping/fallback decision and include the decision in the SN Addition Request message to the SN.

- The SN confirms the slice re-mapping/fallback decision made by the MN in the SN Addition Request Acknowledge message.

- The MN may send the slice re-mapping/fallback decision to the AMF through the PDU Session Modification Indication message.

- The AMF responds the PDU Session Modification Confirmation message.

- 6.2.4Scenario 1: Slice resource shortage in case of Intra-RA mobility and Inter-RA mobility

As specified in TS 28.541, the slice re-mapping between different S-NSSAIs can be achieved via the prioritized resource modeling. For example, suppose UE’s ongoing slice is S-NSSAI 1 configured with rRMPolicyMaxRatio policy, which can use at least one of the shared resources, prioritized resources and dedicated resources. If the dedicated resources are not available, it can use other un-used prioritized and shared resources.

But the following needs to be further studied, e.g., for the S-NSSAI 1,

  - it can explicitly use resources belonging to which S-NSSAIs;

  - it can use the dedicated but not used resources of other S-NSSAIs;

  - it can preempt the used prioritized and/or shared resources from other S-NSSAIs.

In this case, further involvement with SA5 is required.

For

### 6.2.4 Candidate solutions with/without CN involvement

Figure 6.2.4-1: Slice re-mapping solutions: (a) with CN impact; (b) without CN impact

This solution is applicable to scenario 2, where there are two possible slice re-mapping solutions depending on whether the CN is involved.

Figure 6.2.4-1 (a) shows the re-mapping solution where both the RAN and CN parts are involved. In this case, the CN procedure is involved.

Figure 6.2.4-1 (b) shows the re-mapping solution where the CN pat of the slice is not changed while the RAN part of the slice is remapped. The UL/DL traffics are relayed between the S-gNB and the T-gNB via the Xn tunnel.

Editor’s note: The handling of the UE at the target node needs to be clarified.

Editor’s note: Whether CN involvement is required, needs to be evaluated.

Editor’s note: Slice remapping needs to be defined in line with SA2 definitions.

#### Slice resource re-partitioning

Editor note: Feasibility of this solution at system level requires further work including checking with SA5.

This solution is applicable to scenario 1. In this solution, the resource limits for a particular slice in the RAN are relaxed (possibly for a limited time period). This is applicable for resource types which have been hard-partitioned between slices, or where a limit per slice has been defined according to the SLA. For example, such an approach could be applied individually (or jointly) to the following:

- spectrum resource (e.g. slots, beams, carriers etc)

- transport resources (e.g. backhaul capacity)

- hardware resources (e.g. specific processors, processing load, intra-RAN logical nodes such as a gNB-CU-UP)

To solve this problem, the system can allow a slice to use another slice’s resources on a temporary basis i.e. making the partition soft. The RAN may allow such temporary overflow while keeping some form of accounting of resources used which may be used to modify the existing SLA, or provide reporting.

Re-partitioning policy may be configured in the RAN.

The solution may have impacts in metric collection and OAM requirements, but does not impact the core network or the UE.

#### Multi-carrier radio resource sharing

This solution is applicable to scenario 1. In this solution, it is assumed that radio resources are primarily assigned to a slice (or slice sets) on a frequency, or cell, basis. For example, a RAN node may host two layers as shown below:

Figure 6.2.6-1: RAN node supporting two layers

The solution addresses temporary resource shortage in one cell as per scenario 1, and where the RAN node hosts another cell with different frequency and overlapping coverage where the same slice is available.

In above, this could be the case for slice 1 and cell 1/F2 (or also slice 1 and cell 2/F1).

The solution consists of setting up DC or CA using user plane resources of F1 (or F2), for some or all UEs with slice1 PDU sessions. This action can be wholly decided by the RAN node, without referring to the CN or other nodes. This solution can be seen as fallback planning in the RAN.

### 6.2.7	5GC Solution based on SSC-mode 3

The call flow below uses SSC mode 3 in 5GC as the service continuity solution:

Source NG-RAN node 1

5GC

Step 2: UE has ongoing PDU session 1 of slice 10

UE

Target NG-RAN node 2

Step 3: HO request (pdu session 1)

Handover command (PDU session 1 temporary accepted)

Step 6: NAS PDU Session Modification Command (end slice 10, new slice 11)

Handover complete

Step 7: Establish resources for PDU session 2 on slice 11

Step 8: Release PDU session 1 of slice 10

SSC mode 3 Timer expiry

Step 8: UE Configuration Update (new Allowed NSSAI= slice 11 only)

Step 4: PSR (end slice 10, new slice 11)

Register Accept (Allowed NSSAI (slices 10, 11)

Step 5: Register Request

Step 1: UE Allowed NSSAI (slices 10, 11)

Figure 6.2.7-1 Re-mapping based on SSC mode 3

Step 0: NG-RAN nodes have been configured with slice re-mapping slice 10 to 11.

Step1: 5GC has sent the UE Allowed NSSAI to the serving NG-RAN node and to the UE per existing procedures

Step 2: UE has ongoing PDU session 1 of slice 10.

Step 3: Source NG-RAN triggers Handover to target NG-RAN. The target NG-RAN node 2 informs during the HO procedure the source NG-RAN node 1 that it accepts the PDU session 1 of slice 10 temporarily due to slice re-mapping action.

Step 4: at handover completion, the target NG-RAN indicates to 5GC in Path Switch Request that PDU session 1 of slice 10 needs to be terminated and a new PDU session is to be setup with slice 11.

Step 5: The UE performs the post-handover registration (as Source and Target NG RAN nodes have different slice support, they don’t belong to the same registration area for the UE). Because 5GC received (end slice 10) at step 4, the 5GC still includes the slice 10 in the Allowed NSSAI towards the UE at this step (the slice is indeed still temporarily available until it receives from 5GC notification of the final release of PDU session 1 of slice 10 at step 9).

Step 6: In reaction to step 4, the 5GC triggers towards the UE the NAS PDU Session Modification Command to invoke SSC mode 3. The (end slice 10, new 11) may be included towards the UE to prompt the UE to setup the new PDU session 2 with slice 11 even if the URSP in the UE would indicate slice 10 as higher priority.

Step 7: the UE triggers the setup of PDU session 2 with slice 11 according to SSC mode 3 procedure as per existing procedures described in 23.502 § 4.3.2.2.1.

Step 8: at the expiry of SSC mode 3 timer, the 5GC triggers the release of the PDU session 1 of slice 10 according to SSC mode 3 procedures (existing procedures described in 23.502 § 4.3.2.2.1). The 5GC sends a final the UCU (UE Configuration Update) message in order to update the Allowed NSSAI towards the NG-RAN and the UE. In this example, the new Allowed NSSAI is slice 11.

Editor note: This solution is CN-centric and requires confirmation from SA2.

### 6.2.8 Slice Remapping decision in 5GC

General Description

This solution is applicable for scenario 2, when a UE with bearers associated to a given slice, e.g. S-NSSAI1, wants to be handed over to a target cell and where S-NSSAI1 is not supported in the target cell. At NG based HO, the AMF will detect that the target cell is not supporting S-NSSAI1 or that the Allowed NSSAI in the target cell for the UE does not include S-NSSAI1. The 5GC will then decide if the PDU sessions associated to S-NSSAI1 can be re-mapped to another slice. The new S-NSSAI is signalled with the HO Request, using legacy signalling, and there is no impact to the target gNB.

When Xn HO can be used, but the target gNB does not support all slices of an UE, the source gNB will use NG based HO instead, so that 5GC may re-map the slice.

At the end of the HO the UE will be updated with the new Allowed NSSAI through legacy NAS procedures. The original slice will be included in the Rejected NSSAI, and the UE will not be allowed to access it as long as it stays in the current RA. Once the UE enters a new RA, it may request to add the slice to the Allowed NSSAI, and the PDU sessions may be re-assigned to the original S-NSSAI1.

The granularity of slice remapping in this solution is per PDU session. The re-mapping decision can be based on slice awareness in registration area, operator policy for slice re-mapping as well as the subscription of the UE.

Sequence Chart

Figure 6.2.2.3-1: Slice re-mapping/fallback determined by the AMF

- The S-gNB sends the HANDOVER REQUIRED message to the AMF.

- If the UE’s ongoing slice(s) is not supported by the T-gNB, the AMF may make the slice re-mapping/fallback decision and include the decision in the HANDOVER REQUEST message to the T-gNB.

- The T-gNB responds to the AMF through the HANDOVER REQUEST ACKNOWLEDGE message.

- The AMF may send the slice re-mapping/fallback decision to the S-gNB through the HANDOVER COMMAND message.

Editor Note:  It is FFS whether and how the UE is aware of slice remapping.

Evaluation

System Impact

No impact on signalling protocols. gNB should be aware that re-mapping may be used, and select NG based HO when needed.