3GPP TSG-RAN WG3 Meeting #110-e	R3-210588

# E-meeting, 25 Jan – 5 Feb 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0523 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing Maximum Integrity Protected Data Rate after EPC to 5GC handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently the Maximum Integrity Protected Data Rate IE included in the Security Indication IE, is only transferred in the PDU session resource setup procedure, path switch request and Handover Resource Allocation procedure. In section 5.17.2.2.2 of TS 23.501, it is described that: For both idle mode and connected mode mobility from EPC to 5GC: …… - If this is the first mobility event for a PDU Session that was established while being connected to EPC, the UE shall trigger the PDU Session Modification procedure and: …… - should provide the UE Integrity Protection Maximum Data Rate to the network -i.e. SMF). The network shall consider that the maximum data rate per UE for user-plane integrity protection supported by the UE is valid for the lifetime of the PDU session.. - It can be observed after the EPC to 5GC handover, the UE shall initiate the PDU session modification procedure to provide the maximum IP data rate to the SMF, in case these PDU sessions are established at the EPC side. When the SMF acquires the UP IP maximum data rate, it makes sense to provide the updated value to the NG-RAN for UP Integrity protection. - This procedure is also described in the section 4.3.3 of TS 23.502 for PDU session modification procedure. 1. The procedure may be triggered by following events: 1a. (UE initiated modification) The UE initiates the PDU Session Modification procedure by the transmission of an NAS message (N1 SM container (PDU Session Modification Request (PDU session ID, Packet Filters, Operation, Requested QoS, Segregation, 5GSM Core Network Capability, Number Of Packet Filters, [Always-on PDU Session Requested])), PDU Session ID, UE Integrity Protection Maximum Data Rate….. - 6. The (R)AN may acknowledge N2 PDU Session Request by sending a N2 PDU Session Ack - …… - The NG-RAN may reject QFI(s) if it cannot fulfil the User Plane Security Enforcement information for a corresponding QoS Profile, e.g. due to the UE Integrity Protection Maximum Data Rate being exceeded. - Then it comes to the following questions: - During the handover, whether the Security Indication IE (as optional IE) should be included in the Handover Request message, when the SMF has not acquired the maximum IP data rate from the UE yet. - After the handover, how the SMF provides the updated Security Indication (including the acquired UE IP maximum data rate) to the NG-RAN node. - This first can be accomplished by the SMF in either way. But no matter whether the answer to the first question is, the SMF can provide the acquired UP IP maximum data rate value included in the Security Indication IE in the PDU session modify procedure. Further, when the NG-RAN receives the updated value in the PDU session modifiy procedure, it should feedback the Integrity Protection result included in the Security Result IE back to the CN. |
| Summary of change: |  | Include the security indication in PDU SESSION RESOURCE MODIFY REQUEST message, and include Security Result in the PDU SESSION RESOURCE MODIFY RESPONSE message Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the change only affects the UP integrity protection after EPC to 5GC handover. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UP security indication may not be updated when the SMF acquires the UE UP IP maximum data rate after the handover from EPC to 5GC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3, 9.3.4.3, 9.3.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: R3-206640 Rev1: R3-210588 Update the proposal to include the Security Indication in the PDU session resource modify request message, and the Security Result in the PDU session resource modify response message. Update based on the latest specification |  |  |  |  |  |  |  |  |

Change Begins

8.2.3	PDU Session Resource Modify

8.2.3.1	General

The purpose of the PDU Session Resource Modify procedure is to enable configuration modifications of already established PDU session(s) for a given UE. It is also to enable the setup, modification and release of the QoS flow for already established PDU session(s). The procedure uses UE-associated signalling.

8.2.3.2	Successful Operation

Figure 8.2.3.2-1: PDU session resource modify: successful operation

<Unchanged Text Omitted>

For each PDU session included in the PDU Session Resource Modify Request List IE:

-	For each QoS flow included in the QoS Flow Add or Modify Request List IE, based on the QoS Flow Level QoS Parameters IE, the NG-RAN node may establish, modify or release the DRB configuration and may change allocation of resources on NG or Uu accordingly. The NG-RAN node shall associate each QoS flow accepted to setup or modify with a DRB of the PDU session. The associated DRB for the QoS flow accepted to modify may not change.

-	For each QoS flow, if the Redundant QoS Flow Indicator IE is included, the NG-RAN node shall, if supported, store it and consider it for the redundant transmission as specified in TS 23.501 [9].

-	For each QoS flow included in the QoS Flow Add or Modify Request List IE, if the QoS Flow Add or Modify Request Item IE is included for an existing QoS Flow Identifier IE, the NG-RAN node shall overwrite the content of the full QoS Flow Add or Modify Request Item IE.

-	For each QoS flow included in the QoS Flow to Release List IE, the NG-RAN node shall de-associate the QoS flow with the previously associated DRB.

-	If the NAS-PDU IE is received for the PDU session, the NG-RAN node shall pass it to the UE when modifying the Data Radio Bearer configuration. The NG-RAN node does not send the NAS PDU received for the PDU session when all the QoS flows to be added or modified are failed and no QoS flow was requested to be released, even if e.g. the NG-U UP TNL modification is successful.

-	The NG-RAN node may change allocation of resources on NG according to the requested target configuration.

-	If the PDU Session Aggregate Maximum Bit Rate IE is included in the PDU Session Resource Modify Request Transfer IE, the NG-RAN node shall store and use the received PDU Session Aggregate Maximum Bit Rate value when enforcing traffic policing for Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [9].

-	If the UL NG-U UP TNL Modify List IE is included in the PDU Session Resource Modify Request Transfer IE, the NG-RAN node shall update the transport layer information for the uplink data accordingly for the concerned transport bearers identified by the DL NG-U UP TNL Information IE included in the PDU Session Resource Modify Request Transfer IE for the concerned PDU session.

-	If the Additional UL NG-U UP TNL Information IE is included in the PDU Session Resource Modify Request Transfer IE, the NG-RAN node may allocate resources for an additional NG-U transport bearer for some or all of the QoS flows present in the QoS Flow Add or Modify Request List IE and it shall indicate these QoS flows in the Additional DL QoS Flow per TNL Information IE in the PDU Session Resource Modify Response Transfer IE. In case the Additional DL QoS Flow per TNL Information IE is not included the SMF shall consider the proposed additional UL NG-U UP TNL information as available again.

-	In case more than one NG-U transport bearers have been set up for the PDU session, if all the QoS flows associated to one existing NG-U transport bearer are included in the QoS Flow to Release List IE in the PDU Session Resource Modify Request Transfer IE, the NG-RAN node and 5GC shall consider that the concerned NG-U transport bearer is removed for the PDU session, and both NG-RAN node and 5GC shall therefore consider the related NG-U UP TNL information as available again.

-	If the Redundant UL NG-U UP TNL Information IE is included in the PDU Session Resource Modify Request Transfer IE, the NG-RAN node shall, if supported, update the transport layer information for the uplink data accordingly for the concerned transport bearer identified by the Redundant DL NG-U UP TNL Information IE included in the PDU Session Resource Modify Request Transfer IE for the concerned PDU session.

-	If the Additional Redundant UL NG-U UP TNL Information IE is included in the PDU Session Resource Modify Request Transfer IE, the NG-RAN node may allocate resources for an additional redundant NG-U transport bearer for some or all of the QoS flows present in the QoS Flow Add or Modify Request List IE and it shall, if supported, indicate these QoS flows in the Additional Redundant DL QoS Flow per TNL Information IE in the PDU Session Resource Modify Response Transfer IE. In case the Additional Redundant DL QoS Flow per TNL Information IE is not included the SMF shall consider the proposed additional Redundant UL NG-U UP TNL information as available again.

-	If the Security Indication IE is included in the PDU Session Resource Modify Request Transfer IE, the NG-RAN node shall behave the same as defined in the PDU Session Resource Setup procedure. If the Security Result IE is included in the PDU Session Resource Modify Response Transfer IE in the PDU SESSION RESOURCE MODIFY RESPONSE message, it may be considered by the SMF as the new security status of the PDU session.

<Unchanged Text Omitted>

9.3.4.3	PDU Session Resource Modify Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Aggregate Maximum Bit Rate | O |  | 9.3.1.102 |  | YES | reject |
| UL NG-U UP TNL Modify List |  | 0..1 |  |  | YES | reject |
| >UL NG-U UP TNL Modify Item |  | 1..<maxnoofMultiConnectivity> |  |  | - |  |
| >>UL NG-U UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs. | - |  |
| >>DL NG-U UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 | Identifies the NG-U transport bearer at the NG-RAN node. | - |  |
| >>Redundant UL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs for the redundant transmission. | YES | ignore |
| >>Redundant DL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | Identifies the NG-U transport bearer at the NG-RAN node for the redundant transmission. | YES | ignore |
| Network Instance | O |  | 9.3.1.113 | This IE is ignored if the Common Network Instance IE is included. | YES | reject |
| QoS Flow Add or Modify Request List |  | 0..1 |  |  | YES | reject |
| >QoS Flow Add or Modify Request Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>QoS Flow Level QoS Parameters | O |  | 9.3.1.12 |  | - |  |
| >>E-RAB ID | O |  | 9.3.2.3 |  | - |  |
| >>TSC Traffic Characteristics | O |  | 9.3.1.130 | This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |
| >>Redundant QoS Flow Indicator | O |  | 9.3.1.134 | This IE indicates whether this QoS flow is requested for the redundant transmission. | YES | ignore |
| QoS Flow to Release List | O |  | QoS Flow List with Cause 9.3.1.13 |  | YES | reject |
| Additional UL NG-U UP TNL Information | O |  | UP Transport Layer Information List 9.3.2.12 | UPF endpoint of the additional NG-U transport bearer(s) proposed for delivery of UL PDUs for split PDU session. | YES | reject |
| Common Network Instance | O |  | 9.3.1.120 |  | YES | ignore |
| Additional Redundant UL NG-U UP TNL Information | O |  | UP Transport Layer Information List 9.3.2.12 | UPF endpoint of the additional NG-U transport bearer(s) proposed for delivery of redundant UL PDUs for split PDU session. | YES | ignore |
| Redundant Common Network Instance | O |  | Common Network Instance 9.3.1.120 |  | YES | ignore |
| Security Indication | O |  | 9.3.1.27 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |
| maxnoofMultiConnectivity | Maximum no. of connectivity allowed for a UE. Value is 4. The current version of the specification supports up to 2 connectivity. |

9.3.4.4	PDU Session Resource Modify Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs. | - |  |
| UL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | Identifies the NG-U transport bearer at the 5GC node. | - |  |
| QoS Flow Add or Modify Response List |  | 0..1 |  |  | - |  |
| >QoS Flow Add or Modify Response Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.152 | Index to the currently fulfilled alternative QoS parameters set | YES | Ignore |
| Additional DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information List 9.3.2.1 | NG-RAN node endpoint of the additional NG-U transport bearer(s) for delivery of DL PDUs for split PDU session, together with associated QoS flows. | - |  |
| QoS Flow Failed to Add or Modify List | O |  | QoS Flow List with Cause 9.3.1.13 |  | - |  |
| Additional NG-U UP TNL Information | O |  | UP Transport Layer Information Pair List 9.3.2.11 | NG-RAN node endpoint of the NG-U transport bearer corresponding to the modified UPF endpoint received in the PDU Session Resource Modify Request Transfer IE in case of PDU session split. | YES | ignore |
| Redundant DL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs for the redundant transmission. | YES | ignore |
| Redundant UL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | Identifies the NG-U transport bearer at the 5GC node for the redundant transmission. | YES | ignore |
| Additional Redundant DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information List 9.3.2.1 | NG-RAN node endpoint of the additional NG-U transport bearer(s) for delivery of redundant DL PDUs for split PDU session, together with associated QoS flows. | YES | ignore |
| Additional Redundant NG-U UP TNL Information | O |  | UP Transport Layer Information Pair List 9.3.2.11 | NG-RAN node endpoint of the NG-U transport bearer for delivery of redundant DL PDUs corresponding to the modified UPF endpoint received in the PDU Session Resource Modify Request Transfer IE in case of PDU session split. | YES | ignore |
| Security Result | O |  | 9.3.1.59 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

Next Change

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

<Unchanged Text Omitted>

<Unchanged Text Omitted>

PDUSessionResourceModifyRequestTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceModifyRequestTransferIEs} },

...

}

PDUSessionResourceModifyRequestTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-PDUSessionAggregateMaximumBitRate			CRITICALITY reject	TYPE PDUSessionAggregateMaximumBitRate		PRESENCE	optional		}|

{ ID id-UL-NGU-UP-TNLModifyList						CRITICALITY reject	TYPE UL-NGU-UP-TNLModifyList					PRESENCE	optional		}|

{ ID id-NetworkInstance								CRITICALITY reject	TYPE NetworkInstance								PRESENCE optional		}|

{ ID id-QosFlowAddOrModifyRequestList				CRITICALITY reject	TYPE QosFlowAddOrModifyRequestList			PRESENCE optional		}|

{ ID id-QosFlowToReleaseList						CRITICALITY reject	TYPE QosFlowListWithCause						PRESENCE optional		}|

{ ID id-AdditionalUL-NGU-UP-TNLInformation			CRITICALITY reject	TYPE UPTransportLayerInformationList			PRESENCE optional		}|

{ ID id-CommonNetworkInstance						CRITICALITY ignore	TYPE CommonNetworkInstance						PRESENCE optional		}|

{ ID id-AdditionalRedundantUL-NGU-UP-TNLInformation	CRITICALITY ignore	TYPE UPTransportLayerInformationList			PRESENCE optional		}|

{ ID id-RedundantCommonNetworkInstance				CRITICALITY ignore	TYPE CommonNetworkInstance						PRESENCE optional		}|

{ ID id-SecurityIndication					CRITICALITY ignore	TYPE SecurityIndication							PRESENCE optional		},

...

}

PDUSessionResourceModifyResponseTransfer ::= SEQUENCE {

dL-NGU-UP-TNLInformation				UPTransportLayerInformation											OPTIONAL,

uL-NGU-UP-TNLInformation				UPTransportLayerInformation											OPTIONAL,

qosFlowAddOrModifyResponseList			QosFlowAddOrModifyResponseList										OPTIONAL,

additionalDLQosFlowPerTNLInformation	QosFlowPerTNLInformationList										OPTIONAL,

qosFlowFailedToAddOrModifyList			QosFlowListWithCause												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyResponseTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceModifyResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalNGU-UP-TNLInformation					CRITICALITY ignore	EXTENSION UPTransportLayerInformationPairList	PRESENCE optional	}|

{ ID id-RedundantDL-NGU-UP-TNLInformation				CRITICALITY ignore	EXTENSION UPTransportLayerInformation				PRESENCE optional	}|

{ ID id-RedundantUL-NGU-UP-TNLInformation				CRITICALITY ignore	EXTENSION UPTransportLayerInformation				PRESENCE optional	}|

{ ID id-AdditionalRedundantDLQosFlowPerTNLInformation	CRITICALITY ignore	EXTENSION QosFlowPerTNLInformationList				PRESENCE optional	}|

{ ID id-AdditionalRedundantNGU-UP-TNLInformation		CRITICALITY ignore	EXTENSION UPTransportLayerInformationPairList	PRESENCE optional	}|

{ ID id-SecurityResult									CRITICALITY ignore	EXTENSION SecurityResult								PRESENCE optional	},

...

}

<Unchanged Text Omitted>

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<Unchanged Text Omitted>

id-QosFlowParametersList									ProtocolIE-ID ::= 277

id-integrityProtectionResult								ProtocolIE-ID ::= yyy

id-MaximumIntegrityProtectedDataRate-UL						ProtocolIE-ID ::= zzz

END

-- ASN1STOP

Change Ends