3GPP TSG-RAN WG3 Meeting #110e	R3-210619

Online, 25 January – 4 February 2021

Agenda item:	  22.2.4

Source:		Lenovo, Motorola Mobility

Title:		Open Issues on MBS Bearer Management over F1/E1

Document for:		Discussion and Decision

1	Introduction

In RAN3#110e, the MBS bearer management over F1/E1 was discussed, and the following agreement were made [1]:

- Use a shared F1-U tunnel for PTM transmission of an MBS radio bearer for an MBS Session

- It is FFS whether a shared F1-U tunnel can be used for the same MBS Session established in multiple cells of the same DU. To be continued...

- Support the method that gNB-DU assigns the DL F1-U GTP-U tunnel info, provides it to gNB-CU-CP and then gNB-CU-CP forwards it to gNB-CU-UP. FFS if IP multicast method is supported or not

- Provide the MBS Session id, QoS profile from gNB-CU to gNB-DU

- Provide the MBS Session id, QoS profile from gNB-CU-CP to gNB-CU-UP

- F1/E1 MBS Bearer management procedure can be discussed, but details on e.g. information to signal are pending RAN2/SA2 progress

In this paper, we discuss the remaining open issues on MBS Bearer Management over F1/E1.

2	Discussion

Issue 1: F1-U Tunnel for PTP transmission and retransmission

In last meeting, it was agreed that use a shared F1-U tunnel for PTM transmission of an MBS radio bearer for an MBS Session. However, it is still open on whether a shared F1-U tunnel or a individual F1-U tunnel is used for PTP transmission. Additionally, there may be retransmission for the MBS radio bearer e.g. PDCP transmission due to handover or PTM/PTP switch.

Alternative 1: a shared F1-U tunnel for PTM, an individual F1-U tunnel for PTP and retransmission.

In this alternative, a shared F1-U tunnel is used for PTM, while individual F1-U tunnel is used for both PTP and retransmission between the gNB-CU(-UP) and the gNB-DU. This solution is more suitable for PTM and PTP switch in the gNB-CU. If the gNB-CU decides to switch PTM to PTP for a UE, then the gNB-CU starts to send the MBS data over the individual F1-U tunnel to the gNB-DU. Similarly, the gNB-CU sends the retransmission data over the individual F1-U tunnel to the gNB-DU with the ‘retransmission flag’ in the F1-U user plane protocol.

Figure 1. A shared F1-U tunnel for PTM, an individual F1-U tunnel for PTP and retransmission

Alternative 2: a shared F1-U tunnel for both PTM and PTP, an individual F1-U tunnel for retransmission

In this alternative, a shared F1-U tunnel is used for both PTM and PTP, while individual F1-U tunnel for retransmission is used between the gNB-CU(-UP) and the gNB-DU. This solution is more suitable for PTM and PTP switch in the gNB-DU. If the gNB-DU decides to switch PTM to PTP for a UE, then the gNB-DU starts to duplicate the packets received from the shared tunnel to PTP leg. While, the gNB-CU sends the retransmission data over the individual F1-U tunnel to the gNB-DU.

Figure 2. A shared F1-U tunnel for both PTM and PTP, an individual F1-U tunnel for retransmission

If the PTM/PTP switch is decided by the gNB-CU, a shared F1-U tunnel is used for PTM, which an individual F1-U tunnel for PTP and retransmission should be used.

- If the PTM/PTP switch is decided by the gNB-DU, a shared F1-U tunnel is used for both PTM and PTP, while an individual F1-U tunnel is used for retransmission

Issue 2: the shared F1-U tunnel is per cell or per gNB-DU

For PTM, there may be two modes SC-PTM and MC-PTM. In SC-PTM mode, the gNB schedules the multicast traffic in a single cell via a cell specific G-RNTI. In the MC-PTM mode, the gNB schedules the multicast traffic among multiple cells using a same G-RNTI and radio resources among these cells. Taking MC-PTM mode into account, it would be better to have a shared F1-U tunnel for the same MBS Session established in multiple cells of the same gNB-DU.

A shared F1-U tunnel is used for the same MBS bearer in multiple cells of the same gNB-DU.

Issue 3: IP Multicast method for F1-U transport establishment

In RAN3#110e, it was agreed that DL GTP-U Tunnel method is supported for F1-U transport establishment. But it is FFS on whether IP multicast method is standardized or not. In the IP Multicast method, the gNB NB-CU assigns the multicast address and provides it to gNB-CU-CP, gNB-CU-CP forward it to gNB-DU, and then gNB-DU joins the IP multicast group. In order to align with the NG-U, it could be better to standardize the IP multicast method for F1-U transport.

To support IP multicast method for F1-U transport establishment, gNB-CU assigns the multicast address, forwards it to gNB-DU, and then gNB-DU joins the IP multicast group.

Issue 4: shared F1-U tunnel establishment procedure

Following the same method with NG interface, it is straight forward to use a non UE associated F1AP procedures to setup the shared F1-U transport.

Define a new class 1 non-UE associated F1AP procedure for shared F1-U tunnel establishment.

Issue 5: G-RNTI allocation

For PTM, there may be two modes SC-PTM and MC-PTM. In SC-PTM mode, the gNB schedules the multicast traffic in a single cell via a cell specific G-RNTI. In the MC-PTM mode, the gNB schedules the multicast traffic among multiple cells using a same G-RNTI and radio resources among these cells. For SC-PTM mode, the G-RNTI can be allocated in the gNB-DU. For MC-PTM mode, a same G-RNTI is used across multiple cells then it would be better that gNB-CU needs to coordinate the G-RNTI allocation for multiple cells.

In case of MC-PTM mode, gNB-CU needs to coordinate the G-RNTI allocation for multiple cells.

Issue 6: Details of MBS bearer management procedure over E1/F1

The details MBS bearer management procedure over F1/E1 interface should wait for the basic discussions on session management over NG interface.

3	Conclusion

In this paper, we discuss the remaining open issues on MBS Bearer Management over F1/E1. The following proposals are proposed:

- If the PTM/PTP switch is decided by the gNB-CU, a shared F1-U tunnel is used for PTM, which an individual F1-U tunnel for PTP and retransmission should be used.

- If the PTM/PTP switch is decided by the gNB-DU, a shared F1-U tunnel is used for both PTM and PTP, while an individual F1-U tunnel is used for retransmission

- A shared F1-U tunnel is used for the same MBS bearer in multiple cells of the same gNB-DU.

- To support IP multicast method for F1-U transport establishment, gNB-CU assigns the multicast address, forwards it to gNB-DU, and then gNB-DU joins the IP multicast group.

- Define a new class 1 non-UE associated F1AP procedure for shared F1-U tunnel establishment

- In case of MC-PTM mode, gNB-CU needs to coordinate the G-RNTI allocation for multiple cells.

Reference

[1] RAN3 Chairman Notes on RAN3#110e.