3GPP TSG-RAN3 #111-e	R3-210883

25th January – 5th February 2021

Online

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1586 | rev | - | Current version: | 15.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rejected PSCell on X2 for insufficient UE capabilities CR 36.423 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Verizon Wireless, Deutsche Telekom, CMCC, BT, AT&T, China Unicom, Telecom Italia, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-01-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The SgNB cannot indicate that a rejection of SGNB ADDITION REQUEST message is due to insufficient UE capabilities. As a result, new information should be introduced in the X2: SgNB ADDITION REQUEST REJECT message clarifying that the failure is due to insufficient UE capabilities and indicating the PSCells for which the rejection of X2: SGNB ADDITION REQUEST occurred. In this way the MeNB will know the reason of the rejection as well as the rejected PSCell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a list of PSCells for which the rejection of X2: SGNB ADDITION REQUEST occurred, namely a list of the PSCells that were tried, but where the procedure failed and a new cause value, Insufficient UE Capabilities. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has limited impact under funtional point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the issue remains unsolved the MeNB will not know the reason of the rejection nor the rejected PSCell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 8.7.4, 9.1.4.3, 9.2.6, 9.3.4, 9.3.5 and 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR 0575 TS 38.473 CR 0734 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<< NEXT CHANGE >>>>>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.401: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Architecture Description".

[3]	3GPP TS 36.420: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 General Aspects and Principles".

[4]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[5]	ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER) ".

[6]	3GPP TS 32.422: "Telecommunication Management; Subscriber and Equipment Trace; Trace Control and Configuration Management".

[7]	3GPP TS 32.421: "Telecommunication Management; Subscriber and Equipment Trace; Trace concepts and requirements".

[8]	3GPP TS 36.424: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 data transport".

[9]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRAN); Radio Resource Control (RRC) Protocol Specification".

[10]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation".

[11]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures ".

[12]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[13]	3GPP TS 23.203: "Policy and charging control architecture".

[14]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System; Stage 3".

[15]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA), Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; stage 2".

[16]	3GPP TS 36.104: "Base Station (BS) radio transmission and reception ".

[17]	Void.

[18]	3GPP TS 33.401: "Security architecture".

[19]	3GPP TS 36.414: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 data transport".

[20]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC)".

[21]	3GPP TS 36.422: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 signaling transport".

[22]	3GPP TS 36.314: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Layer 2 - Measurements".

[23]	Void.

[24]	3GPP TS 25.413: "UTRAN Iu interface RANAP signalling"

[25]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT);Overall description; Stage 2".

[26]	3GPP TS 29.281: "General Packet Radio Service (GPRS); Tunnelling Protocol User Plane (GTPv1-U)".

[27]	ITU-T Recommendation X.680 (2002-07): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[28]	ITU-T Recommendation X.681 (2002-07): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[29]	3GPP TS 23.003: "Technical Specification Group Core Network and Terminals; Numbering, addressing and identification".

[30]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error handling".

[31]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[32]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[33]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[34]	3GPP TS 38.401: "NG-RAN; Architecture description".

[35]	IETF RFC 5905: "Network Time Protocol Version 4: Protocol and Algorithms Specification".

[36]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[37]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[38]	3GPP TS 23.501: "System Architecture for the 5G System"

[39]	3GPP TS 38.413: "NG Radio Access Network (NG-RAN); NG Application Protocol (NGAP)".

[40]	3GPP TS 36.322: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Link Control (RLC) protocol specification".

[xx]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

<<<<<< NEXT CHANGE >>>>>>

### 8.7.4	SgNB Addition Preparation

#### 8.7.4.1	General

The purpose of the SgNB Addition Preparation procedure is to request the en-gNB to allocate resources for EN-DC connectivity operation for a specific UE.

The procedure uses UE-associated signalling.

#### 8.7.4.2	Successful Operation

Figure 8.7.4.2-1: SgNB Addition Preparation, successful operation

The MeNB initiates the procedure by sending the SGNB ADDITION REQUEST message to the en-gNB. When the MeNB sends the SGNB ADDITION REQUEST message, it shall start the timer TDCprep.

The allocation of resources according to the values of the Allocation and Retention Priority IE included in the Full E-RAB Level QoS Parameters IE or in the Requested MCG E-RAB Level QoS Parameters IE or in the Requested SCG E-RAB Level QoS Parameters IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [4].

If the SGNB ADDITION REQUEST message contains the Serving PLMN IE, the en-gNB may use it for RRM purposes.

If the SGNB ADDITION REQUEST message contains the Expected UE Behaviour IE, the en-gNB shall, if supported, store this information and may use it to optimize resource allocation.

If the SGNB ADDITION REQUEST message contains the Handover Restriction List IE, the en-gNB node, if supported, shall store this information and use it to select an appropriate NR cell.

If the SGNB ADDITION REQUEST message contains the MeNB Resource Coordination Information IE, the en-gNB should forward it to lower layers and it may use it for the purpose of resource coordination with the MeNB. The en-gNB shall consider the received UL Coordination Information IE value valid until reception of a new update of the IE for the same UE. The en-gNB shall consider the received DL Coordination Information IE value valid until reception of a new update of the IE for the same UE. If the MeNB Coordination Assistance Information IE is contained in the MeNB Resource Coordination Information IE, the en-gNB shall, if supported, use the information to determine further coordination of resource utilisation between the en-gNB and the MeNB.

The en-gNB shall choose the ciphering algorithm based on the information in the NR UE Security Capabilities IE and locally configured priority list of AS encryption algorithms and apply the key indicated in the SgNB Security Key IE as specified in the TS 33.401 [18].

If the SGNB ADDITION REQUEST message contains the Subscriber Profile ID for RAT/Frequency Priority IE, the en-gNB may use it for RRM purposes.

The en-gNB shall search for the target NR cell among the NR neighbour cells of the E-UTRAN cell indicated in MeNB Cell ID IE, as specified in the TS 37.340 [32].

If the Masked IMEISV IE is contained in the SGNB ADDITION REQUEST message the en-gNB shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

The en-gNB shall report to the MeNB, in the SGNB ADDITION REQUEST ACKNOWLEDGE message, the result for all the requested E-RABs in the following way:

-	a list of E-RABs which are successfully established shall be included in the E-RABs Admitted To Be Added List IE;

-	a list of E-RABs which failed to be established shall be included in the E-RABs Not Admitted List IE.

NOTE:	The MeNB may trigger the SgNB Addition Preparation procedure in the course of the Inter-MeNB handover without SgNB change procedure as described in TS 37.340 [32]. The deleted E-RABs are not included in the E-RABs To Be Added List IE in the SGNB ADDITION REQUEST message, from MeNB point of view. If the en-gNB reports a certain E-RAB to be successfully established, respective SCG resources, from an en-gNB point of view, may be actually successfully established or modified or kept; if a certain E-RAB is reported to be failed to be established, respective SCG resources, from an en-gNB point of view, may be actually failed to be established or modified or kept.

For each E-RAB successfully established in the en-gNB, the en-gNB shall report to the MeNB, in the SGNB ADDITION REQUEST ACKNOWLEDGE message, the same value in the EN-DC Resource Configuration IE as received in the SGNB ADDITION REQUEST message.

For each E-RAB for which allocation of the PDCP entity is requested at the en-gNB:

-	the MeNB may propose to apply forwarding of downlink data by including the DL Forwarding IE within the E-RABs To be Added Item IE of the SGNB ADDITION REQUEST message. For each E-RAB that it has decided to admit, the en-gNB may include the DL Forwarding GTP Tunnel Endpoint IE within the E-RABs Admitted To Be Added Item IE of the SGNB ADDITION REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding of downlink data for this bearer. This GTP tunnel endpoint may be different from the corresponding GTP tunnel endpoint, i.e the information contained in the Transport Layer Address IE and the DL GTP TEID IE in the E-RAB To Be Modified List IE of the E-RAB MODIFICATION INDICATION message (see TS 36.413 [4]) depending on implementation choice;

-	the en-gNB may include for each bearer in the E-RABs Admitted To Be Added List IE the UL Forwarding GTP Tunnel Endpoint IE to indicate that it requests data forwarding of uplink packets to be performed for that bearer.

-	the en-gNB shall use the S1 UL GTP Tunnel Endpoint IE of the SGNB ADDITION REQUEST message as the UL S1-U address.

-	the MeNB shall use the SgNB UL GTP Tunnel Endpoint at PDCP IE of the SGNB ADDITION REQUEST ACKNOWLEDGE message as the UL X2-U address.

-	if the SGNB ADDITION REQUEST message contains for an E-RAB to be added which is requested to be configured with MCG resources the MeNB DL GTP Tunnel Endpoint at MCG IE the en-gNB shall use it as the DL X2-U address for delivery of DL PDCP PDUs.

-	the en-gNB shall include in the SGNB ADDITION REQUEST ACKNOWLEDGE message the S1 DL GTP Tunnel Endpoint at the SgNB IE.

-	the en-gNB shall include in the SGNB ADDITION REQUEST ACKNOWLEDGE message the RLC Mode IE.

-	the en-gNB may include for each bearer in the E-RABs Admitted To Be Added List IE in the SGNB ADDITION REQUEST ACKNOWLEDGE the PDCP SN Length IE to indicate the PDCP SN length for that bearer.

-	If the RLC Mode IE is included for an E-RAB within the E-RABs To be Added List IE in the SGNB ADDITION REQUEST message, it indicates the mode that the MeNB used for the E-RAB when it was hosted at the MeNB.

-	If the Bearer Type IE for the concerned E-RAB is received by the en-gNB and is set to "non IP", the en-gNB shall, if supported,  not perform header compression for the concerned E-RAB.

Upon reception of the SGNB ADDITION REQUEST ACKNOWLEDGE message the MeNB shall stop the timer TDCprep.

If the SGNB ADDITION ACKNOWLEDGE message contains the SgNB Resource Coordination Information IE, the MeNB may use it for the purpose of resource coordination with the en-gNB. The MeNB shall consider the received UL Coordination Information IE value valid until reception of a new update of the IE for the same UE. The MeNB shall consider the received DL Coordination Information IE value valid until reception of a new update of the IE for the same UE. If the SgNB Coordination Assistance Information IE is contained in the SgNB Resource Coordination Information IE, the MeNB shall, if supported, use the information to determine further coordination of resource utilisation between the en-gNB and the MeNB.

If the SgNB UE X2AP ID IE is contained in the SGNB ADDITION REQUEST message, the en-gNB shall, if supported, store this information and use it as defined in TS 37.340 [32].

If the SGNB ADDITION REQUEST message contains the SGNB Addition Trigger Indication, the en-gNB shall include the RRC config indication IE in the SGNB ADDITION REQUEST ACKNOWLEDGE message to inform the MeNB if the en-gNB applied full or delta configuration, as specified in TS 37.340 [32].

If the en-gNB receives for an E-RAB for which the PDCP entiy is allocated at the MeNB the Secondary MeNB UL GTP Tunnel Endpoint at PDCP IE and the Duplication Activation IE in the SGNB ADDITION REQUEST message, it may provide the Secondary SgNB DL GTP Tunnel Endpoint at SCG IE and the LCID IE to the MeNB in the SGNB ADDITION REQUEST ACKNOWLEDGE message if PDCP duplication is configured at the en-gNB.

If the SGNB ADDITION REQUEST message contains the UL PDCP SN Length IE and the DL PDCP SN Length IE, the en-gNB shall, if supported, store this information and use it for lower layer configuration of the concerned MN terminated bearer.

The SgNB may include the Location Information at SgNB IE in the SGNB ADDITION REQUEST ACKNOWLEDGE message, if respective information is available at the SgNB.

If the Location Information at SgNB Reporting IE set to "pscell" is included in the SGNB ADDITION REQUEST, the SgNB shall start providing information about the current location of the UE. If the Location Information at SgNB IE is included in the SGNB ADDITION REQUEST ACKNOWLEDGE, the MeNB shall store the included information so that it may be transferred towards the MME.

If Trace Activation IE has previously been received for this UE, it shall be included in the SGNB ADDITION REQUEST message. If the Trace Activation IE is included in the SGNB ADDITION REQUEST message, the en-gNB shall, if supported, initiate the requested trace function as described in TS 32.422 [6].

##### Interactions with the MeNB initiated SgNB Modification procedure:

If the en-gNB provides for an E-RAB for which the PDCP entiy is allocated at the MeNB the Secondary SgNB DL GTP Tunnel Endpoint at SCG IE to the MeNB in the SGNB ADDITION REQUEST ACKNOWLEDGE message and the MeNB has not provided the Secondary MeNB UL GTP Tunnel Endpoint at PDCP IE in the SGNB ADDITION REQUEST message, the MeNB shall trigger the MeNB initiated SgNB Modification procedure to provide the Secondary MeNB UL GTP Tunnel Endpoint at PDCP IE to the SgNB.

##### Interactions with the SgNB Reconfiguration Completion procedure:

If the en-gNB admits at least one E-RAB, the en-gNB shall start the timer TDCoverall when sending the SGNB ADDITION REQUEST ACKNOWLEDGE message to the MeNB. The reception of the SGNB RECONFIGURATION COMPLETE message shall stop the timer TDCoverall.

Interaction with the Activity Notification procedure

Upon receiving an SGNB ADDITION REQUEST message containing the Desired Activity Notification Level IE, the en-gNB shall, if supported, use this information to decide whether to trigger subsequent SgNB Activitity Notification procedures according to the requested notification level.

#### 8.7.4.3	Unsuccessful Operation

Figure 8.7.4.3-1: SgNB Addition Preparation, unsuccessful operation

If the en-gNB is not able to accept any of the bearers or a failure occurs during the SgNB Addition Preparation, the en-gNB sends the SGNB ADDITION REQUEST REJECT message with an appropriate cause value to the MeNB.

If the en-gNB includes the Rejected SpCell List IE in the SGNB ADDITION REQUEST REJECT message the MeNB shall, if supported, deduce that the failure was due to insufficient UE capabilities and that the cells identified in the list were attempted to be configured for the UE.

#### 8.7.4.4	Abnormal Conditions

If the en-gNB receives a SGNB ADDITION REQUEST message containing multiple E-RAB ID IEs (in the E-RABs To Be Added List IE) set to the same value, the en-gNB shall consider the establishment of the corresponding E-RAB as failed.

If the en-gNB receives a SGNB ADDITION REQUEST message containing a E-RAB Level QoS Parameters IE which contains a QCI IE indicating a GBR bearer (as defined in TS 23.203 [13]), and which does not contain the GBR QoS Information IE, the en-gNB shall consider the establishment of the corresponding E-RAB as failed.

If the supported algorithms for encryption defined in the NR Encryption Algorithms IE in the NR UE Security Capabilities IE, plus the mandated support of NEA0 in all UEs (TS 33.401 [18]), do not match any algorithms defined in the configured list of allowed encryption algorithms in the en-gNB (TS 33.401 [18]), the en-gNB shall reject the procedure using the SGNB ADDITION REQUEST REJECT message.

If the supported algorithms for integrity defined in the NR Integrity Protection Algorithms IE in the NR UE Security Capabilities IE do not match any algorithms defined in the configured list of allowed integrity protection algorithms in the en-gNB (TS 33.401 [18]), the en-gNB shall reject the procedure using the SGNB ADDITION REQUEST REJECT message.

If the en-gNB receives a SGNB ADDITION REQUEST message containing a SgNB UE X2AP ID IE that does not match any existing UE Context that has such ID, the en-gNB shall reject the procedure using the SGNB ADDITION REQUEST REJECT message.

If the MeNB has provided the en-gNB for an E-RAB for which the PDCP entiy is allocated at the MeNB the Secondary MeNB UL GTP Tunnel Endpoint at PDCP IE and the Duplication Activation IE in the SGNB ADDITION REQUEST message, and the en-gNB does not provide the Secondary SgNB DL GTP Tunnel Endpoint at SCG IE and the LCID IE to the MeNB in the SGNB ADDITION REQUEST ACKNOWLEDGE message, the MeNB shall assume that PDCP duplication was not configured at the en-gNB and releases duplication resources.

If the en-gNB provides for an E-RAB for which the PDCP entiy is allocated at the MeNB the Secondary SgNB DL GTP Tunnel Endpoint at SCG IE and the LCID IE to the MeNB in the SGNB ADDITION REQUEST ACKNOWLEDGE message and the MeNB has not provided the Secondary MeNB UL GTP Tunnel Endpoint at PDCP IE and the Duplication Activation IE in the SGNB ADDITION REQUEST message, and the MeNB does not trigger the MeNB initiated SgNB Modification procedure to provide the Secondary MeNB UL GTP Tunnel Endpoint at PDCP IE and the Duplication Activation IE to the SgNB the en-gNB before the SgNB Reconfigurationi Completion procedure was triggered, the en-gNB shall trigger the release of the concerned E-RAB.

##### Interactions with the SgNB Reconfiguration Completion and SgNB initiated SgNB Release procedure:

If the timer TDCoverall expires before the en-gNB has received the SGNB RECONFIGURATION COMPLETE or the SGNB RELEASE REQUEST message, the en-gNB shall regard the requested RRC connection reconfiguration as being not applied by the UE and shall trigger the SgNB initiated SgNB Release procedure.

##### Interactions with the MeNB initiated SgNB Release procedure:

If the timer TDCprep expires before the MeNB has received the SGNB ADDITION REQUEST ACKNOWLEDGE message, the MeNB shall regard the SgNB Addition Preparation procedure as being failed and shall trigger the MeNB initiated SgNB Release procedure.

<<<<<< NEXT CHANGE >>>>>>

#### 9.1.4.3	SGNB ADDITION REQUEST REJECT

This message is sent by the en-gNB to inform the MeNB that the SgNB Addition Preparation has failed.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | O |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | reject |
| Rejected SpCell List |  | 0..1 |  |  | YES | ignore |
| >Rejected SpCell Item IEs |  | 0 .. <maxnoofRejectedSpCells> |  |  | EACH | ignore |
| >>Rejected SpCell ID | M |  | NR CGI 9.2.111 | Special Cell as defined in TS 38.321 [xx] | - |  |

<<<<<< NEXT CHANGE >>>>>>

### 9.2.6	Cause

The purpose of the cause information element is to indicate the reason for a particular event for the whole protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Handover Desirable for Radio Reasons, Time Critical Handover, Resource Optimisation Handover, Reduce Load in Serving Cell, Partial Handover, Unknown New eNB UE X2AP ID, Unknown Old eNB UE X2AP ID, Unknown Pair of UE X2AP ID, HO Target not Allowed, TX2RELOCoverall Expiry, TRELOCprep Expiry, Cell not Available, No Radio Resources Available in Target Cell, Invalid MME Group ID, Unknown MME Code, Encryption And/Or Integrity Protection Algorithms Not Supported, ReportCharacteristicsEmpty, NoReportPeriodicity, ExistingMeasurementID, Unknown eNB Measurement ID, Measurement Temporarily not Available, Unspecified,...,Load Balancing, Handover Optimisation, Value out of allowed range, Multiple E-RAB ID instances, Switch Off Ongoing, Not supported QCI value, Measurement not supported for the object,TDCoverall Expiry, TDCprep Expiry, Action Desirable for Radio Reasons, Reduce Load, Resource Optimisation, Time Critical action, Target not Allowed, No Radio Resources Available, Invalid QoS combination, Encryption Algorithms Not Supported, Procedure cancelled, RRM purpose, Improve user bit rate, User Inactivity, Radio Connection With UE Lost, Failure in the Radio Interface Procedure, Bearer Option not Supported, MCG Mobility, SCG Mobility, Count reaches max value, Unknown Old en-gNB UE X2AP ID, PDCP Overload , Insufficient UE Capabilities) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Transport Resource Unavailable, Unspecified,...) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error,Abstract Syntax Error (Reject),Abstract Syntax Error (Ignore and Notify),Message not Compatible with Receiver State,Semantic Error,Unspecified,Abstract Syntax Error (Falsely Constructed Message),...) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Hardware Failure,O&M Intervention,Not enough User Plane Processing Resources,Unspecified,...) |  |

The meaning of the different cause values is described in the following table. In general, "not supported" cause values indicate that the concerned capability is missing. On the other hand, "not available" cause values indicate that the concerned capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Cell not Available | The concerned cell is not available. |
| Handover Desirable for Radio Reasons | The reason for requesting handover is radio related. |
| Handover Target not Allowed | Handover to the indicated target cell is not allowed for the UE in question |
| Invalid MME Group ID | The target eNB doesn’t belong to the same pool area of the source eNB i.e. S1 handovers should be attempted instead. |
| No Radio Resources Available in Target Cell | The target cell doesn’t have sufficient radio resources available. |
| Partial Handover | Provides a reason for the handover cancellation. The target eNB did not admit all E-RABs included in the HANDOVER REQUEST and the source eNB estimated service continuity for the UE would be better by not proceeding with handover towards this particular target eNB. |
| Reduce Load in Serving Cell | Load in serving cell needs to be reduced. When applied to handover preparation, it indicates the handover is triggered due to load balancing. |
| Resource Optimisation Handover | The reason for requesting handover is to improve the load distribution with the neighbour cells. |
| Time Critical Handover | Handover is requested for time critical reason i.e. this cause value is reserved to represent all critical cases where the connection is likely to be dropped if handover is not performed. |
| TX2RELOCoverall Expiry | The reason for the action is expiry of timer TX2RELOCoverall. |
| TRELOCprep Expiry | Handover Preparation procedure is cancelled when timer TRELOCprep expires. |
| Unknown MME Code | The target eNB belongs to the same pool area of the source eNB and recognizes the MME Group ID. However, the MME Code is unknown to the target eNB. |
| Unknown New eNB UE X2AP ID | The action failed because the New eNB UE X2AP ID or the MeNB UE X2AP ID is unknown. |
| Unknown Old eNB UE X2AP ID | The action failed because the Old eNB UE X2AP ID or the SeNB UE X2AP ID is unknown. |
| Unknown Pair of UE X2AP ID | The action failed because the pair of UE X2 AP IDs is unknown. |
| Encryption And/Or Integrity Protection Algorithms Not Supported | The target eNB is unable to support any of the encryption and/or integrity protection algorithms supported by the UE, or the en-gNB is unable to support any of the NR encryption and/or integrity protection algorithms supported by the UE for EN-DC operation. |
| ReportCharacteristicsEmpty | The action failed because there is no characteristic reported. |
| NoReportPeriodicity | The action failed because the periodicity is not defined. |
| ExistingMeasurementID | The action failed because measurement-ID is already used. |
| Unknown eNB Measurement ID | The action failed because some eNB Measurement-ID is unknown. |
| Measurement Temporarily not Available | The eNB can temporarily not provide the requested measurement object. |
| Load Balancing | The reason for mobility settings change is load balancing. |
| Handover Optimisation | The reason for mobility settings change is handover optimisation. |
| Value out of allowed range | The action failed because the proposed Handover Trigger parameter change in the eNB2 Proposed Mobility Parameters IE is too low or too high. |
| Multiple E-RAB ID Instances | The action failed because multiple instances of the same E-RAB had been provided to the eNB. |
| Switch Off Ongoing | The reason for the action is an ongoing switch off i.e. the concerned cell will be switched off after offloading and not be available. It aides the receiving eNB in taking subsequent actions, e.g. selecting the target cell for subsequent handovers. |
| Not supported QCI value | The action failed because the requested QCI is not supported. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Radio Network Layer related. |
| Measurement not Supported For The Object | At least one of the concerned cell(s) does not support the requested measurement. |
| TDCoverall Expiry | The reason for the action is expiry of timer TDCoverall. |
| TDCprep Expiry | The reason for the action is expiry of timer TDCprep. |
| Action Desirable for Radio Reasons | The reason for requesting the action is radio related. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Reduce Load | Load in the cell(group) served by the requesting node needs to be reduced. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Resource Optimisation | The reason for requesting this action is to improve the load distribution with the neighbour cells. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Time Critical action | The action is requested for time critical reason i.e. this cause value is reserved to represent all critical cases where radio resources are likely to be dropped if the requested action is not performed. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Target not Allowed | Requested action towards the indicated target cell is not allowed for the UE in question. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| No Radio Resources Available | The cell(s) in the requested node don’t have sufficient radio resources available. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Invalid QoS combination | The action was failed because of invalid QoS combination. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Encryption Algorithms Not Supported | The requested eNB is unable to support any of the encryption algorithms supported by the UE. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Procedure cancelled | The sending node cancelled the procedure due to other urgent actions to be performed. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| RRM purpose | The procedure is initiated due to node internal RRM purposes. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Improve User Bit Rate | The reason for requesting this action is to improve the user bit rate. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| User Inactivity | The action is requested due to user inactivity on all E-RABs, e.g., S1 is requested to be released in order to optimise the radio resources; or SeNB/en-gNB didn’t see activity on the DRB recently. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Radio Connection With UE Lost | The action is requested due to losing the radio connection to the UE. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Failure in the Radio Interface Procedure | Radio interface procedure has failed. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Bearer Option not Supported | The requested bearer option is not supported by the sending node. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| MCG Mobility | The procedure is initiated due to mobility related at MCG radio resource. |
| SCG Mobility | The procedure is initiated due to mobility related at SCG radio resource. |
| Count reaches max value | Indicates the PDCP COUNT for UL or DL reached the max value and the bearer may be released. |
| Unknown Old en-gNB UE X2AP ID | The action failed because the Old en-gNB UE X2AP ID or the SeNB UE X2AP ID is unknown. |
| PDCP Overload | The procedure is initiated due to PDCP resource limitation. |
| Insufficient UE Capabilities | The procedure can’t proceed due to insufficient UE capabilities. |

| Transport Network Layer cause | Meaning |
| --- | --- |
| Transport resource unavailable | The required transport resources are not available. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Transport Network Layer related |

| Protocol cause | Meaning |
| --- | --- |
| Abstract Syntax Error (Reject) | The received message included an abstract syntax error and the concerned criticality indicated "reject" (see sub clause 10.3 of TS 36.413 [4]). |
| Abstract Syntax Error (Ignore and Notify) | The received message included an abstract syntax error and the concerned criticality indicated "ignore and notify" (see sub clause 10.3 of TS 36.413 [4]). |
| Abstract syntax error (falsely constructed message) | The received message contained IEs or IE groups in wrong order or with too many occurrences (see sub clause 10.3 of TS 36.413 [4]). |
| Message not Compatible with Receiver State | The received message was not compatible with the receiver state (see sub clause 10.4 of TS 36.413 [4]). |
| Semantic Error | The received message included a semantic error (see sub clause 10.4 of TS 36.413 [4]). |
| Transfer Syntax Error | The received message included a transfer syntax error (see sub clause 10.2 of TS 36.413 [4]). |
| Unspecified | Sent when none of the above cause values applies but still the cause is Protocol related |

| Miscellaneous cause | Meaning |
| --- | --- |
| Control Processing Overload | eNB control processing overload |
| Hardware Failure | eNB hardware failure |
| Not enough User Plane Processing Resources | eNB has insufficient user plane processing resources available. |
| O&M Intervention | Operation and Maintenance intervention related to eNB equipment |
| Unspecified | Sent when none of the above cause values applies and the cause is not related to any of the categories Radio Network Layer, Transport Network Layer or Protocol |

<<<<<< NEXT CHANGE >>>>>>

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for X2AP.

--

-- **************************************************************

X2AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ABSInformation,

ABS-Status,

AS-SecurityInformation,

BearerType,

Cause,

CompositeAvailableCapacityGroup,

Correlation-ID,

COUNTvalue,

CellReportingIndicator,

AerialUEsubscriptionInformation,

CriticalityDiagnostics,

CRNTI,

CSGMembershipStatus,

CSG-Id,

DeactivationIndication,

DL-Forwarding,

DynamicDLTransmissionInformation,

ECGI,

E-RAB-ID,

E-RAB-Level-QoS-Parameters,

E-RAB-List,

EUTRANTraceID,

GlobalENB-ID,

GTPtunnelEndpoint,

GUGroupIDList,

GUMMEI,

HandoverReportType,

HandoverRestrictionList,

Masked-IMEISV,

InvokeIndication,

LocationReportingInformation,

MDT-Configuration,

ManagementBasedMDTallowed,

MDTPLMNList,

Neighbour-Information,

PCI,

PDCP-SN,

PLMN-Identity,

ReceiveStatusofULPDCPSDUs,

Registration-Request,

RelativeNarrowbandTxPower,

RadioResourceStatus,

RLC-Status,

RRCConnReestabIndicator,

RRCConnSetupIndicator,

UE-RLF-Report-Container,

UEAppLayerMeasConfig,

RRC-Context,

ServedCell-Information,

ServedCells,

ShortMAC-I,

SRVCCOperationPossible,

SubscriberProfileIDforRFP,

TargetCellInUTRAN,

TargeteNBtoSource-eNBTransparentContainer,

TimeToWait,

TraceActivation,

TraceDepth,

TransportLayerAddress,

UEAggregateMaximumBitRate,

UE-HistoryInformation,

UE-HistoryInformationFromTheUE,

UE-S1AP-ID,

UESecurityCapabilities,

UEsToBeResetList,

UE-X2AP-ID,

UL-HighInterferenceIndicationInfo,

UL-InterferenceOverloadIndication,

HWLoadIndicator,

S1TNLLoadIndicator,

Measurement-ID,

ReportCharacteristics,

MobilityParametersInformation,

MobilityParametersModificationRange,

ReceiveStatusOfULPDCPSDUsExtended,

COUNTValueExtended,

SubframeAssignment,

ExtendedULInterferenceOverloadInfo,

ExpectedUEBehaviour,

SeNBSecurityKey,

MeNBtoSeNBContainer,

SeNBtoMeNBContainer,

SCGChangeIndication,

CoMPInformation,

ReportingPeriodicityRSRPMR,

RSRPMRList,

UE-RLF-Report-Container-for-extended-bands,

ProSeAuthorized,

CoverageModificationList,

ReportingPeriodicityCSIR,

CSIReportList,

ReceiveStatusOfULPDCPSDUsPDCP-SNlength18,

COUNTvaluePDCP-SNlength18,

LHN-ID,

UE-ContextKeptIndicator,

UE-X2AP-ID-Extension,

SIPTOBearerDeactivationIndication,

TunnelInformation,

V2XServicesAuthorized,

X2BenefitValue,

ResumeID,

EUTRANCellIdentifier,

MakeBeforeBreakIndicator,

WTID,

WT-UE-XwAP-ID,

UESidelinkAggregateMaximumBitRate,

SgNBSecurityKey,

MeNBtoSgNBContainer,

SgNBtoMeNBContainer,

SplitSRBs,

RRCContainer,

SRBType,

GlobalGNB-ID,

GNB-ID,

SCGConfigurationQuery,

SplitSRB,

NRUeReport,

EN-DC-ResourceConfiguration,

TAC,

NRFreqInfo,

NRCGI,

NRPCI,

NRUESecurityCapabilities,

PDCPChangeIndication,

ULConfiguration,

SgNB-UE-X2AP-ID,

SecondaryRATUsageReportList,

ActivationID,

MeNBResourceCoordinationInformation,

SgNBResourceCoordinationInformation,

NR-TxBW,

BroadcastPLMNs-Item,

AdditionalPLMNs-Item,

RLCMode,

GBR-QosInformation,

DRB-ID,

FiveGS-TAC,

SULInformation,

Packet-LossRate,

ResourceType,

DataTrafficResourceIndication,

SpectrumSharingGroupID,

RRC-Config-Ind,

SGNB-Addition-Trigger-Ind,

UserPlaneTrafficActivityReport,

ERABActivityNotifyItemList,

PDCPSnLength,

Subscription-Based-UE-DifferentiationInfo,

LCID,

DuplicationActivation,

GNBOverloadInformation,

NewDRBIDrequest,

DesiredActNotificationLevel,

LocationInformationSgNB,

LocationInformationSgNBReporting,

EndcSONConfigurationTransfer,

NRNeighbour-Information,

InterfaceInstanceIndication,

BPLMN-ID-Info-NR,

EPCHandoverRestrictionListContainer,

Rejected-SpCell-Item,

FROM X2AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

X2AP-PRIVATE-IES,

X2AP-PROTOCOL-EXTENSION,

X2AP-PROTOCOL-IES,

X2AP-PROTOCOL-IES-PAIR

FROM X2AP-Containers

id-ABSInformation,

id-ActivatedCellList,

id-BearerType,

id-Cause,

id-CellInformation,

id-CellInformation-Item,

id-CellMeasurementResult,

id-CellMeasurementResult-Item,

id-CellToReport,

id-CellToReport-Item,

id-CompositeAvailableCapacityGroup,

id-AerialUEsubscriptionInformation,

id-CriticalityDiagnostics,

id-DeactivationIndication,

id-DynamicDLTransmissionInformation,

id-E-RABs-Admitted-Item,

id-E-RABs-Admitted-List,

id-E-RABs-NotAdmitted-List,

id-E-RABs-SubjectToStatusTransfer-List,

id-E-RABs-SubjectToStatusTransfer-Item,

id-E-RABs-ToBeSetup-Item,

id-GlobalENB-ID,

id-GUGroupIDList,

id-GUGroupIDToAddList,

id-GUGroupIDToDeleteList,

id-GUMMEI-ID,

id-Masked-IMEISV,

id-InvokeIndication,

id-New-eNB-UE-X2AP-ID,

id-Old-eNB-UE-X2AP-ID,

id-Registration-Request,

id-ReportingPeriodicity,

id-RLC-Status,

id-ServedCells,

id-ServedCellsToActivate,

id-ServedCellsToAdd,

id-ServedCellsToModify,

id-ServedCellsToDelete,

id-SRVCCOperationPossible,

id-TargetCell-ID,

id-TargeteNBtoSource-eNBTransparentContainer,

id-TimeToWait,

id-TraceActivation,

id-UE-ContextInformation,

id-UE-HistoryInformation,

id-UE-X2AP-ID,

id-Measurement-ID,

id-ReportCharacteristics,

id-ENB1-Measurement-ID,

id-ENB2-Measurement-ID,

id-ENB1-Cell-ID,

id-ENB2-Cell-ID,

id-ENB2-Proposed-Mobility-Parameters,

id-ENB1-Mobility-Parameters,

id-ENB2-Mobility-Parameters-Modification-Range,

id-FailureCellPCI,

id-Re-establishmentCellECGI,

id-FailureCellCRNTI,

id-ShortMAC-I,

id-SourceCellECGI,

id-FailureCellECGI,

id-HandoverReportType,

id-UE-RLF-Report-Container,

id-PartialSuccessIndicator,

id-MeasurementInitiationResult-List,

id-MeasurementInitiationResult-Item,

id-MeasurementFailureCause-Item,

id-CompleteFailureCauseInformation-List,

id-CompleteFailureCauseInformation-Item,

id-CSGMembershipStatus,

id-CSG-Id,

id-MDTConfiguration,

id-ManagementBasedMDTallowed,

id-ABS-Status,

id-RRCConnSetupIndicator,

id-RRCConnReestabIndicator,

id-TargetCellInUTRAN,

id-MobilityInformation,

id-SourceCellCRNTI,

id-ManagementBasedMDTPLMNList,

id-ReceiveStatusOfULPDCPSDUsExtended,

id-ULCOUNTValueExtended,

id-DLCOUNTValueExtended,

id-IntendedULDLConfiguration,

id-ExtendedULInterferenceOverloadInfo,

id-RNL-Header,

id-x2APMessage,

id-UE-HistoryInformationFromTheUE,

id-ExpectedUEBehaviour,

id-MeNB-UE-X2AP-ID,

id-SeNB-UE-X2AP-ID,

id-UE-SecurityCapabilities,

id-SeNBSecurityKey,

id-SeNBUEAggregateMaximumBitRate,

id-ServingPLMN,

id-E-RABs-ToBeAdded-List,

id-E-RABs-ToBeAdded-Item,

id-MeNBtoSeNBContainer,

id-E-RABs-Admitted-ToBeAdded-List,

id-E-RABs-Admitted-ToBeAdded-Item,

id-SeNBtoMeNBContainer,

id-ResponseInformationSeNBReconfComp,

id-UE-ContextInformationSeNBModReq,

id-E-RABs-ToBeAdded-ModReqItem,

id-E-RABs-ToBeModified-ModReqItem,

id-E-RABs-ToBeReleased-ModReqItem,

id-E-RABs-Admitted-ToBeAdded-ModAckList,

id-E-RABs-Admitted-ToBeModified-ModAckList,

id-E-RABs-Admitted-ToBeReleased-ModAckList,

id-E-RABs-Admitted-ToBeAdded-ModAckItem,

id-E-RABs-Admitted-ToBeModified-ModAckItem,

id-E-RABs-Admitted-ToBeReleased-ModAckItem,

id-SCGChangeIndication,

id-E-RABs-ToBeReleased-ModReqd,

id-E-RABs-ToBeReleased-ModReqdItem,

id-E-RABs-ToBeReleased-List-RelReq,

id-E-RABs-ToBeReleased-RelReqItem,

id-E-RABs-ToBeReleased-List-RelConf,

id-E-RABs-ToBeReleased-RelConfItem,

id-E-RABs-SubjectToCounterCheck-List,

id-E-RABs-SubjectToCounterCheckItem,

id-CoMPInformation,

id-ReportingPeriodicityRSRPMR,

id-RSRPMRList,

id-UE-RLF-Report-Container-for-extended-bands,

id-ProSeAuthorized,

id-CoverageModificationList,

id-ReportingPeriodicityCSIR,

id-CSIReportList,

id-ReceiveStatusOfULPDCPSDUsPDCP-SNlength18,

id-ULCOUNTValuePDCP-SNlength18,

id-DLCOUNTValuePDCP-SNlength18,

id-LHN-ID,

id-Correlation-ID,

id-SIPTO-Correlation-ID,

id-UE-ContextReferenceAtSeNB,

id-UE-ContextReferenceAtWT,

id-UE-ContextKeptIndicator,

id-UEs-ToBeReset,

id-UEs-Admitted-ToBeReset,

id-WT-UE-ContextKeptIndicator,

id-New-eNB-UE-X2AP-ID-Extension,

id-Old-eNB-UE-X2AP-ID-Extension,

id-MeNB-UE-X2AP-ID-Extension,

id-SeNB-UE-X2AP-ID-Extension,

id-SIPTO-BearerDeactivationIndication,

id-Tunnel-Information-for-BBF,

id-SIPTO-L-GW-TransportLayerAddress,

id-GW-TransportLayerAddress,

id-X2RemovalThreshold,

id-CellReportingIndicator,

id-V2XServicesAuthorized,

id-resumeID,

id-UE-ContextInformationRetrieve,

id-E-RABs-ToBeSetupRetrieve-Item,

id-NewEUTRANCellIdentifier,

id-MakeBeforeBreakIndicator,

id-UESidelinkAggregateMaximumBitRate,

id-uL-GTPtunnelEndpoint,

id-SgNBSecurityKey,

id-SgNBUEAggregateMaximumBitRate,

id-E-RABs-ToBeAdded-SgNBAddReqList,

id-MeNBtoSgNBContainer,

id-SgNB-UE-X2AP-ID,

id-RequestedSplitSRBs,

id-E-RABs-ToBeAdded-SgNBAddReq-Item,

id-E-RABs-Admitted-ToBeAdded-SgNBAddReqAckList,

id-SgNBtoMeNBContainer,

id-AdmittedSplitSRBs,

id-E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item,

id-ResponseInformationSgNBReconfComp,

id-UE-ContextInformation-SgNBModReq,

id-E-RABs-ToBeAdded-SgNBModReq-Item,

id-E-RABs-ToBeModified-SgNBModReq-Item,

id-E-RABs-ToBeReleased-SgNBModReq-Item,

id-E-RABs-Admitted-ToBeAdded-SgNBModAckList,

id-E-RABs-Admitted-ToBeModified-SgNBModAckList,

id-E-RABs-Admitted-ToBeReleased-SgNBModAckList,

id-E-RABs-Admitted-ToBeAdded-SgNBModAck-Item,

id-E-RABs-Admitted-ToBeModified-SgNBModAck-Item,

id-E-RABs-Admitted-ToBeReleased-SgNBModAck-Item,

id-E-RABs-Admitted-ToBeReleased-SgNBRelReqAckList,

id-E-RABs-Admitted-ToBeReleased-SgNBRelReqAck-Item,

id-E-RABs-ToBeReleased-SgNBModReqdList,

id-E-RABs-ToBeModified-SgNBModReqdList,

id-E-RABs-ToBeReleased-SgNBModReqd-Item,

id-E-RABs-ToBeModified-SgNBModReqd-Item,

id-E-RABs-ToBeReleased-SgNBChaConfList,

id-E-RABs-ToBeReleased-SgNBChaConf-Item,

id-E-RABs-ToBeReleased-SgNBRelReqList,

id-E-RABs-ToBeReleased-SgNBRelReq-Item,

id-E-RABs-ToBeReleased-SgNBRelConfList,

id-E-RABs-ToBeReleased-SgNBRelConf-Item,

id-E-RABs-ToBeReleased-SgNBRelReqdList,

id-E-RABs-ToBeReleased-SgNBRelReqd-Item,

id-E-RABs-SubjectToSgNBCounterCheck-List,

id-E-RABs-SubjectToSgNBCounterCheck-Item,

id-Target-SgNB-ID,

id-RRCContainer,

id-SRBType,

id-HandoverRestrictionList,

id-SCGConfigurationQuery,

id-SplitSRB,

id-NRUeReport,

id-InitiatingNodeType-EndcX2Setup,

id-InitiatingNodeType-EndcConfigUpdate,

id-RespondingNodeType-EndcX2Setup,

id-RespondingNodeType-EndcConfigUpdate,

id-NRUESecurityCapabilities,

id-PDCPChangeIndication,

id-ServedEUTRAcellsENDCX2ManagementList,

id-ServedEUTRAcellsToModifyListENDCConfUpd,

id-ServedEUTRAcellsToDeleteListENDCConfUpd,

id-ServedNRcellsToModifyListENDCConfUpd,

id-ServedNRcellsToDeleteListENDCConfUpd,

id-CellAssistanceInformation,

id-Globalen-gNB-ID,

id-ServedNRcellsENDCX2ManagementList,

id-Old-SgNB-UE-X2AP-ID,

id-UE-ContextReferenceAtSgNB,

id-SecondaryRATUsageReportList,

id-ActivationID,

id-ServedNRCellsToActivate,

id-ActivatedNRCellList,

id-MeNBResourceCoordinationInformation,

id-SgNBResourceCoordinationInformation,

id-UEAppLayerMeasConfig,

id-SelectedPLMN,

id-SubscriberProfileIDforRFP,

id-InitiatingNodeType-EutranrCellResourceCoordination,

id-RespondingNodeType-EutranrCellResourceCoordination,

id-DataTrafficResourceIndication,

id-SpectrumSharingGroupID,

id-ListofEUTRACellsinEUTRACoordinationReq,

id-ListofEUTRACellsinEUTRACoordinationResp,

id-ListofEUTRACellsinNRCoordinationReq,

id-ListofNRCellsinNRCoordinationReq,

id-ListofNRCellsinNRCoordinationResp,

id-RRCConfigIndication,

id-SGNB-Addition-Trigger-Ind,

id-RequestedSplitSRBsrelease,

id-AdmittedSplitSRBsrelease,

id-E-RABs-AdmittedToBeModified-SgNBModConfList,

id-E-RABs-AdmittedToBeModified-SgNBModConf-Item,

id-UEContextLevelUserPlaneActivity,

id-ERABActivityNotifyItemList,

id-MeNBCell-ID,

id-InitiatingNodeType-EndcX2Removal,

id-RespondingNodeType-EndcX2Removal,

id-uLpDCPSnLength,

id-dL-Forwarding,

id-E-RABs-DataForwardingAddress-List,

id-E-RABs-DataForwardingAddress-Item,

id-Subscription-Based-UE-DifferentiationInfo,

id-RLCMode-transferred,

id-dLPDCPSnLength,

id-secondarysgNBDLGTPTEIDatPDCP,

id-secondarymeNBULGTPTEIDatPDCP,

id-lCID,

id-duplicationActivation,

id-GNBOverloadInformation,

id-new-drb-ID-req,

id-NRNeighbourInfoToModify,

id-DesiredActNotificationLevel,

id-LocationInformationSgNB,

id-LocationInformationSgNBReporting,

id-endcSONConfigurationTransfer,

id-EUTRANTraceID,

id-additionalPLMNs-Item,

id-InterfaceInstanceIndication,

id-BPLMN-ID-Info-NR,

id-ERABs-transferred-to-MeNB,

id-EPCHandoverRestrictionListContainer,

id-Rejected-SpCell-Item,

id-Rejected-SpCell-List

maxCellineNB,

maxnoofBearers,

maxnoofPDCP-SN,

maxFailedMeasObjects,

maxnoofCellIDforMDT,

maxnoofTAforMDT,

maxCellinengNB,

maxnoofCellIDforQMC,

maxnoofTAforQMC,

maxnoofPLMNforQMC,

maxnoofProtectedResourcePatterns,

maxnoNRcellsSpectrumSharingWithE-UTRA,

maxnoofNrCellBands

<<<<<< NEXT CHANGE >>>>>>

-- **************************************************************

--

#### -- SGNB ADDITION REQUEST REJECT

--

-- **************************************************************

SgNBAdditionRequestReject ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SgNBAdditionRequestReject-IEs}},

...

}

SgNBAdditionRequestReject-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID					CRITICALITY reject	TYPE SgNB-UE-X2AP-ID			PRESENCE optional}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-Rejected-SpCell-List			CRITICALITY ignore	TYPE Rejected-SpCell-List		PRESENCE optional	},

...

}

Rejected-SpCell-List::= SEQUENCE (SIZE(0..maxnoofRejectedSpCells)) OF ProtocolIE-SingleContainer { { Rejected-SpCell-ItemIEs} }

Rejected-SpCell-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Rejected-SpCell-Item				CRITICALITY ignore	TYPE Rejected-SpCell-Item					PRESENCE mandatory	},

...

}

<<<<<< NEXT CHANGE >>>>>>

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

transport			CauseTransport,

protocol			CauseProtocol,

misc				CauseMisc,

...

}

CauseMisc ::= ENUMERATED {

control-processing-overload,

hardware-failure,

om-intervention,

not-enough-user-plane-processing-resources,

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

unspecified,

abstract-syntax-error-falsely-constructed-message,

...

}

CauseRadioNetwork ::= ENUMERATED {

handover-desirable-for-radio-reasons,

time-critical-handover,

resource-optimisation-handover,

reduce-load-in-serving-cell,

partial-handover,

unknown-new-eNB-UE-X2AP-ID,

unknown-old-eNB-UE-X2AP-ID,

unknown-pair-of-UE-X2AP-ID,

ho-target-not-allowed,

tx2relocoverall-expiry,

trelocprep-expiry,

cell-not-available,

no-radio-resources-available-in-target-cell,

invalid-MME-GroupID,

unknown-MME-Code,

encryption-and-or-integrity-protection-algorithms-not-supported,

reportCharacteristicsEmpty,

noReportPeriodicity,

existingMeasurementID,

unknown-eNB-Measurement-ID,

measurement-temporarily-not-available,

unspecified,

...,

load-balancing,

handover-optimisation,

value-out-of-allowed-range,

multiple-E-RAB-ID-instances,

switch-off-ongoing,

not-supported-QCI-value,

measurement-not-supported-for-the-object,

tDCoverall-expiry,

tDCprep-expiry,

action-desirable-for-radio-reasons,

reduce-load,

resource-optimisation,

time-critical-action,

target-not-allowed,

no-radio-resources-available,

invalid-QoS-combination,

encryption-algorithms-not-supported,

procedure-cancelled,

rRM-purpose,

improve-user-bit-rate,

user-inactivity,

radio-connection-with-UE-lost,

failure-in-the-radio-interface-procedure,

bearer-option-not-supported,

mCG-Mobility,

sCG-Mobility,

count-reaches-max-value,

unknown-old-en-gNB-UE-X2AP-ID,

pDCP-Overload,

insufficient-ue-capabilities

}

CauseTransport ::= ENUMERATED {

transport-resource-unavailable,

unspecified,

...

}

<<<<<< NEXT CHANGE >>>>>>

Rejected-SpCell-Item ::= SEQUENCE {

rejected-SpCell-ID			NRCGI	,

iE-Extensions	ProtocolExtensionContainer { { Rejected-SpCell-ItemExtIEs } }	OPTIONAL,

...

}

Rejected-SpCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<< NEXT CHANGE >>>>>>

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

X2AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM X2AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-handoverCancel												ProcedureCode ::= 1

id-loadIndication												ProcedureCode ::= 2

id-errorIndication												ProcedureCode ::= 3

id-snStatusTransfer												ProcedureCode ::= 4

id-uEContextRelease												ProcedureCode ::= 5

id-x2Setup														ProcedureCode ::= 6

id-reset														ProcedureCode ::= 7

id-eNBConfigurationUpdate										ProcedureCode ::= 8

id-resourceStatusReportingInitiation							ProcedureCode ::= 9

id-resourceStatusReporting										ProcedureCode ::= 10

id-privateMessage												ProcedureCode ::= 11

id-mobilitySettingsChange										ProcedureCode ::= 12

id-rLFIndication												ProcedureCode ::= 13

id-handoverReport												ProcedureCode ::= 14

id-cellActivation												ProcedureCode ::= 15

id-x2Release													ProcedureCode ::= 16

id-x2APMessageTransfer											ProcedureCode ::= 17

id-x2Removal													ProcedureCode ::= 18

id-seNBAdditionPreparation										ProcedureCode ::= 19

id-seNBReconfigurationCompletion								ProcedureCode ::= 20

id-meNBinitiatedSeNBModificationPreparation						ProcedureCode ::= 21

id-seNBinitiatedSeNBModification								ProcedureCode ::= 22

id-meNBinitiatedSeNBRelease										ProcedureCode ::= 23

id-seNBinitiatedSeNBRelease										ProcedureCode ::= 24

id-seNBCounterCheck												ProcedureCode ::= 25

id-retrieveUEContext											ProcedureCode ::= 26

id-sgNBAdditionPreparation										ProcedureCode ::= 27

id-sgNBReconfigurationCompletion									ProcedureCode ::= 28

id-meNBinitiatedSgNBModificationPreparation						ProcedureCode ::= 29

id-sgNBinitiatedSgNBModification									ProcedureCode ::= 30

id-meNBinitiatedSgNBRelease										ProcedureCode ::= 31

id-sgNBinitiatedSgNBRelease										ProcedureCode ::= 32

id-sgNBCounterCheck												ProcedureCode ::= 33

id-sgNBChange													ProcedureCode ::= 34

id-rRCTransfer													ProcedureCode ::= 35

id-endcX2Setup													ProcedureCode ::= 36

id-endcConfigurationUpdate										ProcedureCode ::= 37

id-secondaryRATDataUsageReport									ProcedureCode ::= 38

id-endcCellActivation											ProcedureCode ::= 39

id-endcPartialReset												ProcedureCode ::= 40

id-eUTRANRCellResourceCoordination								ProcedureCode ::= 41

id-SgNBActivityNotification										ProcedureCode ::= 42

id-endcX2Removal												ProcedureCode ::= 43

id-dataForwardingAddressIndication								ProcedureCode ::= 44

id-gNBStatusIndication											ProcedureCode ::= 45

id-deactivateTrace												ProcedureCode ::= 46

id-traceStart													ProcedureCode ::= 47

id-endcConfigurationTransfer										ProcedureCode ::= 48

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxEARFCN									INTEGER ::= 65535

maxEARFCNPlusOne							INTEGER ::= 65536

newmaxEARFCN								INTEGER ::= 262143

maxInterfaces								INTEGER ::= 16

maxCellineNB								INTEGER ::= 256

maxnoofBands								INTEGER ::= 16

maxnoofBearers								INTEGER ::= 256

maxNrOfErrors								INTEGER ::= 256

maxnoofPDCP-SN								INTEGER ::= 16

maxnoofEPLMNs								INTEGER ::= 15

maxnoofEPLMNsPlusOne						INTEGER ::= 16

maxnoofForbLACs								INTEGER ::= 4096

maxnoofForbTACs								INTEGER ::= 4096

maxnoofBPLMNs								INTEGER ::= 6

maxnoofAdditionalPLMNs						INTEGER ::= 6

maxnoofNeighbours							INTEGER ::= 512

maxnoofPRBs									INTEGER ::= 110

maxPools									INTEGER ::= 16

maxnoofCells								INTEGER ::= 16

maxnoofMBSFN								INTEGER ::= 8

maxFailedMeasObjects						INTEGER ::= 32

maxnoofCellIDforMDT							INTEGER ::= 32

maxnoofTAforMDT								INTEGER ::= 8

maxnoofMBMSServiceAreaIdentities			INTEGER ::= 256

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofCoMPHypothesisSet					INTEGER ::= 256

maxnoofCoMPCells							INTEGER ::= 32

maxUEReport									INTEGER ::= 128

maxCellReport								INTEGER ::= 9

maxnoofPA									INTEGER ::= 3

maxCSIProcess								INTEGER ::= 4

maxCSIReport								INTEGER ::= 2

maxSubband									INTEGER ::= 14

maxofNRNeighbours							INTEGER ::= 1024

maxCellinengNB								INTEGER ::= 16384

--	maxnoofNRCarriers						INTEGER ::= 32

maxnooftimeperiods							INTEGER ::= 2

maxnoofCellIDforQMC							INTEGER ::= 32

maxnoofTAforQMC								INTEGER ::= 8

maxnoofPLMNforQMC							INTEGER ::= 16

maxUEsinengNBDU								INTEGER ::= 8192

maxnoofProtectedResourcePatterns			INTEGER ::= 16

maxnoNRcellsSpectrumSharingWithE-UTRA		INTEGER ::= 64

maxnoofNrCellBands							INTEGER ::= 32

maxnoofBluetoothName						INTEGER ::= 4

maxnoofWLANName								INTEGER ::= 4

maxnoofextBPLMNs							INTEGER ::= 12

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-E-RABs-Admitted-Item														ProtocolIE-ID ::= 0

id-E-RABs-Admitted-List														ProtocolIE-ID ::= 1

id-E-RAB-Item																ProtocolIE-ID ::= 2

id-E-RABs-NotAdmitted-List													ProtocolIE-ID ::= 3

id-E-RABs-ToBeSetup-Item													ProtocolIE-ID ::= 4

id-Cause																	ProtocolIE-ID ::= 5

id-CellInformation															ProtocolIE-ID ::= 6

id-CellInformation-Item														ProtocolIE-ID ::= 7

id-New-eNB-UE-X2AP-ID														ProtocolIE-ID ::= 9

id-Old-eNB-UE-X2AP-ID														ProtocolIE-ID ::= 10

id-TargetCell-ID															ProtocolIE-ID ::= 11

id-TargeteNBtoSource-eNBTransparentContainer								ProtocolIE-ID ::= 12

id-TraceActivation															ProtocolIE-ID ::= 13

id-UE-ContextInformation													ProtocolIE-ID ::= 14

id-UE-HistoryInformation													ProtocolIE-ID ::= 15

id-UE-X2AP-ID																ProtocolIE-ID ::= 16

id-CriticalityDiagnostics													ProtocolIE-ID ::= 17

id-E-RABs-SubjectToStatusTransfer-List										ProtocolIE-ID ::= 18

id-E-RABs-SubjectToStatusTransfer-Item										ProtocolIE-ID ::= 19

id-ServedCells																ProtocolIE-ID ::= 20

id-GlobalENB-ID																ProtocolIE-ID ::= 21

id-TimeToWait																ProtocolIE-ID ::= 22

id-GUMMEI-ID																ProtocolIE-ID ::= 23

id-GUGroupIDList															ProtocolIE-ID ::= 24

id-ServedCellsToAdd															ProtocolIE-ID ::= 25

id-ServedCellsToModify														ProtocolIE-ID ::= 26

id-ServedCellsToDelete														ProtocolIE-ID ::= 27

id-Registration-Request														ProtocolIE-ID ::= 28

id-CellToReport																ProtocolIE-ID ::= 29

id-ReportingPeriodicity														ProtocolIE-ID ::= 30

id-CellToReport-Item														ProtocolIE-ID ::= 31

id-CellMeasurementResult													ProtocolIE-ID ::= 32

id-CellMeasurementResult-Item												ProtocolIE-ID ::= 33

id-GUGroupIDToAddList														ProtocolIE-ID ::= 34

id-GUGroupIDToDeleteList													ProtocolIE-ID ::= 35

id-SRVCCOperationPossible													ProtocolIE-ID ::= 36

id-Measurement-ID															ProtocolIE-ID ::= 37

id-ReportCharacteristics													ProtocolIE-ID ::= 38

id-ENB1-Measurement-ID														ProtocolIE-ID ::= 39

id-ENB2-Measurement-ID														ProtocolIE-ID ::= 40

id-Number-of-Antennaports													ProtocolIE-ID ::= 41

id-CompositeAvailableCapacityGroup											ProtocolIE-ID ::= 42

id-ENB1-Cell-ID																ProtocolIE-ID ::= 43

id-ENB2-Cell-ID																ProtocolIE-ID ::= 44

id-ENB2-Proposed-Mobility-Parameters										ProtocolIE-ID ::= 45

id-ENB1-Mobility-Parameters													ProtocolIE-ID ::= 46

id-ENB2-Mobility-Parameters-Modification-Range								ProtocolIE-ID ::= 47

id-FailureCellPCI															ProtocolIE-ID ::= 48

id-Re-establishmentCellECGI													ProtocolIE-ID ::= 49

id-FailureCellCRNTI															ProtocolIE-ID ::= 50

id-ShortMAC-I																ProtocolIE-ID ::= 51

id-SourceCellECGI															ProtocolIE-ID ::= 52

id-FailureCellECGI															ProtocolIE-ID ::= 53

id-HandoverReportType														ProtocolIE-ID ::= 54

id-PRACH-Configuration														ProtocolIE-ID ::= 55

id-MBSFN-Subframe-Info														ProtocolIE-ID ::= 56

id-ServedCellsToActivate													ProtocolIE-ID ::= 57

id-ActivatedCellList														ProtocolIE-ID ::= 58

id-DeactivationIndication													ProtocolIE-ID ::= 59

id-UE-RLF-Report-Container													ProtocolIE-ID ::= 60

id-ABSInformation															ProtocolIE-ID ::= 61

id-InvokeIndication															ProtocolIE-ID ::= 62

id-ABS-Status																ProtocolIE-ID ::= 63

id-PartialSuccessIndicator													ProtocolIE-ID ::= 64

id-MeasurementInitiationResult-List											ProtocolIE-ID ::= 65

id-MeasurementInitiationResult-Item											ProtocolIE-ID ::= 66

id-MeasurementFailureCause-Item												ProtocolIE-ID ::= 67

id-CompleteFailureCauseInformation-List										ProtocolIE-ID ::= 68

id-CompleteFailureCauseInformation-Item										ProtocolIE-ID ::= 69

id-CSG-Id																	ProtocolIE-ID ::= 70

id-CSGMembershipStatus														ProtocolIE-ID ::= 71

id-MDTConfiguration															ProtocolIE-ID ::= 72

id-ManagementBasedMDTallowed												ProtocolIE-ID ::= 74

id-RRCConnSetupIndicator													ProtocolIE-ID ::= 75

id-NeighbourTAC																ProtocolIE-ID ::= 76

id-Time-UE-StayedInCell-EnhancedGranularity									ProtocolIE-ID ::= 77

id-RRCConnReestabIndicator													ProtocolIE-ID ::= 78

id-MBMS-Service-Area-List													ProtocolIE-ID ::= 79

id-HO-cause																	ProtocolIE-ID ::= 80

id-TargetCellInUTRAN														ProtocolIE-ID ::= 81

id-MobilityInformation														ProtocolIE-ID ::= 82

id-SourceCellCRNTI															ProtocolIE-ID ::= 83

id-MultibandInfoList														ProtocolIE-ID ::= 84

id-M3Configuration															ProtocolIE-ID ::= 85

id-M4Configuration															ProtocolIE-ID ::= 86

id-M5Configuration															ProtocolIE-ID ::= 87

id-MDT-Location-Info														ProtocolIE-ID ::= 88

id-ManagementBasedMDTPLMNList												ProtocolIE-ID ::= 89

id-SignallingBasedMDTPLMNList												ProtocolIE-ID ::= 90

id-ReceiveStatusOfULPDCPSDUsExtended										ProtocolIE-ID ::= 91

id-ULCOUNTValueExtended														ProtocolIE-ID ::= 92

id-DLCOUNTValueExtended														ProtocolIE-ID ::= 93

id-eARFCNExtension															ProtocolIE-ID ::= 94

id-UL-EARFCNExtension														ProtocolIE-ID ::= 95

id-DL-EARFCNExtension														ProtocolIE-ID ::= 96

id-AdditionalSpecialSubframe-Info											ProtocolIE-ID ::= 97

id-Masked-IMEISV															ProtocolIE-ID ::= 98

id-IntendedULDLConfiguration												ProtocolIE-ID ::= 99

id-ExtendedULInterferenceOverloadInfo										ProtocolIE-ID ::= 100

id-RNL-Header																ProtocolIE-ID ::= 101

id-x2APMessage																ProtocolIE-ID ::= 102

id-ProSeAuthorized															ProtocolIE-ID ::= 103

id-ExpectedUEBehaviour														ProtocolIE-ID ::= 104

id-UE-HistoryInformationFromTheUE											ProtocolIE-ID ::= 105

id-DynamicDLTransmissionInformation											ProtocolIE-ID ::= 106

id-UE-RLF-Report-Container-for-extended-bands								ProtocolIE-ID ::= 107

id-CoMPInformation															ProtocolIE-ID ::= 108

id-ReportingPeriodicityRSRPMR												ProtocolIE-ID ::= 109

id-RSRPMRList																ProtocolIE-ID ::= 110

id-MeNB-UE-X2AP-ID															ProtocolIE-ID ::= 111

id-SeNB-UE-X2AP-ID															ProtocolIE-ID ::= 112

id-UE-SecurityCapabilities													ProtocolIE-ID ::= 113

id-SeNBSecurityKey															ProtocolIE-ID ::= 114

id-SeNBUEAggregateMaximumBitRate											ProtocolIE-ID ::= 115

id-ServingPLMN																ProtocolIE-ID ::= 116

id-E-RABs-ToBeAdded-List													ProtocolIE-ID ::= 117

id-E-RABs-ToBeAdded-Item													ProtocolIE-ID ::= 118

id-MeNBtoSeNBContainer														ProtocolIE-ID ::= 119

id-E-RABs-Admitted-ToBeAdded-List											ProtocolIE-ID ::= 120

id-E-RABs-Admitted-ToBeAdded-Item											ProtocolIE-ID ::= 121

id-SeNBtoMeNBContainer														ProtocolIE-ID ::= 122

id-ResponseInformationSeNBReconfComp										ProtocolIE-ID ::= 123

id-UE-ContextInformationSeNBModReq											ProtocolIE-ID ::= 124

id-E-RABs-ToBeAdded-ModReqItem												ProtocolIE-ID ::= 125

id-E-RABs-ToBeModified-ModReqItem											ProtocolIE-ID ::= 126

id-E-RABs-ToBeReleased-ModReqItem											ProtocolIE-ID ::= 127

id-E-RABs-Admitted-ToBeAdded-ModAckList										ProtocolIE-ID ::= 128

id-E-RABs-Admitted-ToBeModified-ModAckList									ProtocolIE-ID ::= 129

id-E-RABs-Admitted-ToBeReleased-ModAckList									ProtocolIE-ID ::= 130

id-E-RABs-Admitted-ToBeAdded-ModAckItem										ProtocolIE-ID ::= 131

id-E-RABs-Admitted-ToBeModified-ModAckItem									ProtocolIE-ID ::= 132

id-E-RABs-Admitted-ToBeReleased-ModAckItem									ProtocolIE-ID ::= 133

id-E-RABs-ToBeReleased-ModReqd												ProtocolIE-ID ::= 134

id-E-RABs-ToBeReleased-ModReqdItem											ProtocolIE-ID ::= 135

id-SCGChangeIndication														ProtocolIE-ID ::= 136

id-E-RABs-ToBeReleased-List-RelReq											ProtocolIE-ID ::= 137

id-E-RABs-ToBeReleased-RelReqItem											ProtocolIE-ID ::= 138

id-E-RABs-ToBeReleased-List-RelConf											ProtocolIE-ID ::= 139

id-E-RABs-ToBeReleased-RelConfItem											ProtocolIE-ID ::= 140

id-E-RABs-SubjectToCounterCheck-List										ProtocolIE-ID ::= 141

id-E-RABs-SubjectToCounterCheckItem											ProtocolIE-ID ::= 142

id-CoverageModificationList													ProtocolIE-ID ::= 143

id-ReportingPeriodicityCSIR													ProtocolIE-ID ::= 145

id-CSIReportList															ProtocolIE-ID ::= 146

id-UEID																		ProtocolIE-ID ::= 147

id-enhancedRNTP																ProtocolIE-ID ::= 148

id-ProSeUEtoNetworkRelaying													ProtocolIE-ID ::= 149

id-ReceiveStatusOfULPDCPSDUsPDCP-SNlength18									ProtocolIE-ID ::= 150

id-ULCOUNTValuePDCP-SNlength18												ProtocolIE-ID ::= 151

id-DLCOUNTValuePDCP-SNlength18												ProtocolIE-ID ::= 152

id-UE-ContextReferenceAtSeNB												ProtocolIE-ID ::= 153

id-UE-ContextKeptIndicator													ProtocolIE-ID ::= 154

id-New-eNB-UE-X2AP-ID-Extension												ProtocolIE-ID ::= 155

id-Old-eNB-UE-X2AP-ID-Extension												ProtocolIE-ID ::= 156

id-MeNB-UE-X2AP-ID-Extension												ProtocolIE-ID ::= 157

id-SeNB-UE-X2AP-ID-Extension												ProtocolIE-ID ::= 158

id-LHN-ID																	ProtocolIE-ID ::= 159

id-FreqBandIndicatorPriority												ProtocolIE-ID ::= 160

id-M6Configuration															ProtocolIE-ID ::= 161

id-M7Configuration															ProtocolIE-ID ::= 162

id-Tunnel-Information-for-BBF												ProtocolIE-ID ::= 163

id-SIPTO-BearerDeactivationIndication										ProtocolIE-ID ::= 164

id-GW-TransportLayerAddress													ProtocolIE-ID ::= 165

id-Correlation-ID															ProtocolIE-ID ::= 166

id-SIPTO-Correlation-ID														ProtocolIE-ID ::= 167

id-SIPTO-L-GW-TransportLayerAddress											ProtocolIE-ID ::= 168

id-X2RemovalThreshold														ProtocolIE-ID ::= 169

id-CellReportingIndicator													ProtocolIE-ID ::= 170

id-BearerType																ProtocolIE-ID ::= 171

id-resumeID																	ProtocolIE-ID ::= 172

id-UE-ContextInformationRetrieve											ProtocolIE-ID ::= 173

id-E-RABs-ToBeSetupRetrieve-Item											ProtocolIE-ID ::= 174

id-NewEUTRANCellIdentifier													ProtocolIE-ID ::= 175

id-V2XServicesAuthorized													ProtocolIE-ID ::= 176

id-OffsetOfNbiotChannelNumberToDL-EARFCN									ProtocolIE-ID ::= 177

id-OffsetOfNbiotChannelNumberToUL-EARFCN									ProtocolIE-ID ::= 178

id-AdditionalSpecialSubframeExtension-Info									ProtocolIE-ID ::= 179

id-BandwidthReducedSI														ProtocolIE-ID ::= 180

id-MakeBeforeBreakIndicator													ProtocolIE-ID ::= 181

id-UE-ContextReferenceAtWT													ProtocolIE-ID ::= 182

id-WT-UE-ContextKeptIndicator												ProtocolIE-ID ::= 183

id-UESidelinkAggregateMaximumBitRate										ProtocolIE-ID ::= 184

id-uL-GTPtunnelEndpoint														ProtocolIE-ID ::= 185

id-DL-scheduling-PDCCH-CCE-usage											ProtocolIE-ID ::= 193

id-UL-scheduling-PDCCH-CCE-usage											ProtocolIE-ID ::= 194

id-UEAppLayerMeasConfig														ProtocolIE-ID ::= 195

id-extended-e-RAB-MaximumBitrateDL											ProtocolIE-ID ::= 196

id-extended-e-RAB-MaximumBitrateUL											ProtocolIE-ID ::= 197

id-extended-e-RAB-GuaranteedBitrateDL										ProtocolIE-ID ::= 198

id-extended-e-RAB-GuaranteedBitrateUL										ProtocolIE-ID ::= 199

id-extended-uEaggregateMaximumBitRateDownlink								ProtocolIE-ID ::= 200

id-extended-uEaggregateMaximumBitRateUplink									ProtocolIE-ID ::= 201

id-NRrestrictioninEPSasSecondaryRAT											ProtocolIE-ID ::= 202

id-SgNBSecurityKey															ProtocolIE-ID ::= 203

id-SgNBUEAggregateMaximumBitRate												ProtocolIE-ID ::= 204

id-E-RABs-ToBeAdded-SgNBAddReqList											ProtocolIE-ID ::= 205

id-MeNBtoSgNBContainer														ProtocolIE-ID ::= 206

id-SgNB-UE-X2AP-ID															ProtocolIE-ID ::= 207

id-RequestedSplitSRBs														ProtocolIE-ID ::= 208

id-E-RABs-ToBeAdded-SgNBAddReq-Item											ProtocolIE-ID ::= 209

id-E-RABs-Admitted-ToBeAdded-SgNBAddReqAckList								ProtocolIE-ID ::= 210

id-SgNBtoMeNBContainer														ProtocolIE-ID ::= 211

id-AdmittedSplitSRBs															ProtocolIE-ID ::= 212

id-E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item								ProtocolIE-ID ::= 213

id-ResponseInformationSgNBReconfComp											ProtocolIE-ID ::= 214

id-UE-ContextInformation-SgNBModReq											ProtocolIE-ID ::= 215

id-E-RABs-ToBeAdded-SgNBModReq-Item											ProtocolIE-ID ::= 216

id-E-RABs-ToBeModified-SgNBModReq-Item										ProtocolIE-ID ::= 217

id-E-RABs-ToBeReleased-SgNBModReq-Item										ProtocolIE-ID ::= 218

id-E-RABs-Admitted-ToBeAdded-SgNBModAckList									ProtocolIE-ID ::= 219

id-E-RABs-Admitted-ToBeModified-SgNBModAckList								ProtocolIE-ID ::= 220

id-E-RABs-Admitted-ToBeReleased-SgNBModAckList								ProtocolIE-ID ::= 221

id-E-RABs-Admitted-ToBeAdded-SgNBModAck-Item									ProtocolIE-ID ::= 222

id-E-RABs-Admitted-ToBeModified-SgNBModAck-Item								ProtocolIE-ID ::= 223

id-E-RABs-Admitted-ToBeReleased-SgNBModAck-Item								ProtocolIE-ID ::= 224

id-E-RABs-ToBeReleased-SgNBModReqdList										ProtocolIE-ID ::= 225

id-E-RABs-ToBeModified-SgNBModReqdList										ProtocolIE-ID ::= 226

id-E-RABs-ToBeReleased-SgNBModReqd-Item										ProtocolIE-ID ::= 227

id-E-RABs-ToBeModified-SgNBModReqd-Item										ProtocolIE-ID ::= 228

id-E-RABs-ToBeReleased-SgNBChaConfList										ProtocolIE-ID ::= 229

id-E-RABs-ToBeReleased-SgNBChaConf-Item										ProtocolIE-ID ::= 230

id-E-RABs-ToBeReleased-SgNBRelReqList										ProtocolIE-ID ::= 231

id-E-RABs-ToBeReleased-SgNBRelReq-Item										ProtocolIE-ID ::= 232

id-E-RABs-ToBeReleased-SgNBRelConfList										ProtocolIE-ID ::= 233

id-E-RABs-ToBeReleased-SgNBRelConf-Item										ProtocolIE-ID ::= 234

id-E-RABs-SubjectToSgNBCounterCheck-List										ProtocolIE-ID ::= 235

id-E-RABs-SubjectToSgNBCounterCheck-Item										ProtocolIE-ID ::= 236

id-RRCContainer																ProtocolIE-ID ::= 237

id-SRBType																	ProtocolIE-ID ::= 238

id-Target-SgNB-ID															ProtocolIE-ID ::= 239

id-HandoverRestrictionList													ProtocolIE-ID ::= 240

id-SCGConfigurationQuery														ProtocolIE-ID ::= 241

id-SplitSRB																	ProtocolIE-ID ::= 242

id-NRUeReport																ProtocolIE-ID ::= 243

id-InitiatingNodeType-EndcX2Setup											ProtocolIE-ID ::= 244

id-InitiatingNodeType-EndcConfigUpdate										ProtocolIE-ID ::= 245

id-RespondingNodeType-EndcX2Setup											ProtocolIE-ID ::= 246

id-RespondingNodeType-EndcConfigUpdate										ProtocolIE-ID ::= 247

id-NRUESecurityCapabilities													ProtocolIE-ID ::= 248

id-PDCPChangeIndication														ProtocolIE-ID ::= 249

id-ServedEUTRAcellsENDCX2ManagementList										ProtocolIE-ID ::= 250

id-CellAssistanceInformation													ProtocolIE-ID ::= 251

id-Globalen-gNB-ID															ProtocolIE-ID ::= 252

id-ServedNRcellsENDCX2ManagementList											ProtocolIE-ID ::= 253

id-UE-ContextReferenceAtSgNB													ProtocolIE-ID ::= 254

id-SecondaryRATUsageReport													ProtocolIE-ID ::= 255

id-ActivationID																ProtocolIE-ID ::= 256

id-MeNBResourceCoordinationInformation										ProtocolIE-ID ::= 257

id-SgNBResourceCoordinationInformation										ProtocolIE-ID ::= 258

id-ServedEUTRAcellsToModifyListENDCConfUpd									ProtocolIE-ID ::= 259

id-ServedEUTRAcellsToDeleteListENDCConfUpd									ProtocolIE-ID ::= 260

id-ServedNRcellsToModifyListENDCConfUpd										ProtocolIE-ID ::= 261

id-ServedNRcellsToDeleteListENDCConfUpd										ProtocolIE-ID ::= 262

id-E-RABUsageReport-Item														ProtocolIE-ID ::= 263

id-Old-SgNB-UE-X2AP-ID														ProtocolIE-ID ::= 264

id-SecondaryRATUsageReportList												ProtocolIE-ID ::= 265

id-SecondaryRATUsageReport-Item												ProtocolIE-ID ::= 266

id-ServedNRCellsToActivate													ProtocolIE-ID ::= 267

id-ActivatedNRCellList														ProtocolIE-ID ::= 268

id-SelectedPLMN																ProtocolIE-ID ::= 269

id-UEs-ToBeReset															ProtocolIE-ID ::= 270

id-UEs-Admitted-ToBeReset													ProtocolIE-ID ::= 271

id-RRCConfigIndication														ProtocolIE-ID ::= 272

id-DownlinkPacketLossRate													ProtocolIE-ID ::= 273

id-UplinkPacketLossRate														ProtocolIE-ID ::= 274

id-SubscriberProfileIDforRFP												ProtocolIE-ID ::= 275

id-serviceType																ProtocolIE-ID ::= 276

id-AerialUEsubscriptionInformation											ProtocolIE-ID ::= 277

id-SGNB-Addition-Trigger-Ind												ProtocolIE-ID ::= 278

id-MeNBCell-ID																ProtocolIE-ID ::= 279

id-RequestedSplitSRBsrelease												ProtocolIE-ID ::= 280

id-AdmittedSplitSRBsrelease													ProtocolIE-ID ::= 281

id-NRS-NSSS-PowerOffset														ProtocolIE-ID ::= 282

id-NSSS-NumOccasionDifferentPrecoder										ProtocolIE-ID ::= 283

id-ProtectedEUTRAResourceIndication											ProtocolIE-ID ::= 284

id-InitiatingNodeType-EutranrCellResourceCoordination						ProtocolIE-ID ::= 285

id-RespondingNodeType-EutranrCellResourceCoordination						ProtocolIE-ID ::= 286

id-DataTrafficResourceIndication											ProtocolIE-ID ::= 287

id-SpectrumSharingGroupID													ProtocolIE-ID ::= 288

id-ListofEUTRACellsinEUTRACoordinationReq									ProtocolIE-ID ::= 289

id-ListofEUTRACellsinEUTRACoordinationResp									ProtocolIE-ID ::= 290

id-ListofEUTRACellsinNRCoordinationReq										ProtocolIE-ID ::= 291

id-ListofNRCellsinNRCoordinationReq											ProtocolIE-ID ::= 292

id-ListofNRCellsinNRCoordinationResp										ProtocolIE-ID ::= 293

id-E-RABs-AdmittedToBeModified-SgNBModConfList								ProtocolIE-ID ::= 294

id-E-RABs-AdmittedToBeModified-SgNBModConf-Item								ProtocolIE-ID ::= 295

id-UEContextLevelUserPlaneActivity											ProtocolIE-ID ::= 296

id-ERABActivityNotifyItemList												ProtocolIE-ID ::= 297

id-InitiatingNodeType-EndcX2Removal											ProtocolIE-ID ::= 298

id-RespondingNodeType-EndcX2Removal											ProtocolIE-ID ::= 299

id-RLC-Status																ProtocolIE-ID ::= 300

id-CNTypeRestrictions														ProtocolIE-ID ::= 301

id-uLpDCPSnLength															ProtocolIE-ID ::= 302

id-BluetoothMeasurementConfiguration										ProtocolIE-ID ::= 303

id-WLANMeasurementConfiguration												ProtocolIE-ID ::= 304

id-NRrestrictionin5GS														ProtocolIE-ID ::= 305

id-dL-Forwarding															ProtocolIE-ID ::= 306

id-E-RABs-DataForwardingAddress-List										ProtocolIE-ID ::= 307

id-E-RABs-DataForwardingAddress-Item										ProtocolIE-ID ::= 308

id-Subscription-Based-UE-DifferentiationInfo								ProtocolIE-ID ::= 309

id-GNBOverloadInformation													ProtocolIE-ID ::= 310

id-dLPDCPSnLength															ProtocolIE-ID ::= 311

id-secondarysgNBDLGTPTEIDatPDCP												ProtocolIE-ID ::= 312

id-secondarymeNBULGTPTEIDatPDCP												ProtocolIE-ID ::= 313

id-lCID																		ProtocolIE-ID ::= 314

id-duplicationActivation													ProtocolIE-ID ::= 315

id-ECGI																		ProtocolIE-ID ::= 316

id-RLCMode-transferred														ProtocolIE-ID ::= 317

id-E-RABs-Admitted-ToBeReleased-SgNBRelReqAckList							ProtocolIE-ID ::= 318

id-E-RABs-Admitted-ToBeReleased-SgNBRelReqAck-Item							ProtocolIE-ID ::= 319

id-E-RABs-ToBeReleased-SgNBRelReqdList										ProtocolIE-ID ::= 320

id-E-RABs-ToBeReleased-SgNBRelReqd-Item										ProtocolIE-ID ::= 321

id-NRCGI																	ProtocolIE-ID ::= 322

id-MeNBCoordinationAssistanceInformation									ProtocolIE-ID ::= 323

id-SgNBCoordinationAssistanceInformation									ProtocolIE-ID ::= 324

id-new-drb-ID-req															ProtocolIE-ID ::= 325

id-endcSONConfigurationTransfer												ProtocolIE-ID ::= 326

id-NRNeighbourInfoToAdd														ProtocolIE-ID ::= 327

id-NRNeighbourInfoToModify													ProtocolIE-ID ::= 328

id-DesiredActNotificationLevel												ProtocolIE-ID ::= 329

id-LocationInformationSgNBReporting											ProtocolIE-ID ::= 330

id-LocationInformationSgNB													ProtocolIE-ID ::= 331

id-LastNG-RANPLMNIdentity													ProtocolIE-ID ::= 332

id-EUTRANTraceID															ProtocolIE-ID ::= 333

id-additionalPLMNs-Item														ProtocolIE-ID ::= 334

id-InterfaceInstanceIndication												ProtocolIE-ID ::= 335

id-BPLMN-ID-Info-EUTRA														ProtocolIE-ID ::= 336

id-BPLMN-ID-Info-NR															ProtocolIE-ID ::= 337

id-NBIoT-UL-DL-AlignmentOffset												ProtocolIE-ID ::= 338

id-ERABs-transferred-to-MeNB												ProtocolIE-ID ::= 339

id-EPCHandoverRestrictionListContainer										ProtocolIE-ID ::= 360

id-Rejected-SpCell-List							    						ProtocolIE-ID ::= xxx

id-Rejected-SpCell-Item						        						ProtocolIE-ID ::= yyy

END

-- ASN1STOP

<<<<<< END OF CHANGES >>>>>>