3GPP TSG-RAN WG3 Meeting #111e	R3-210889

# Online Meeting, 25th January – 5th February 2021

Agenda Item:	9.2.1

Source:	Ericsson

Title: 	Indicating E-RABs that cannot be handed over to 2G/3G or 5G

Document for:	Discussion and Approval

# 1	Discussion

In customer network, there is one issue reported with SRVCC to 3G:

- UE moves from 5G to LTE, with both IMS PDN voice bearer, and other normal PDNs. The handover is successfully to LTE.

- UE moves to 3G, and eNode B triggers SRVCC with PS handover procedure.

- eNB uses CS+PS handover so the “Number of Iu Instances IE = 2 (means CS+PS)” in IE “Source to Target Transparent Container”,

- But the non-voice bearers in 5G do not have TI (Transaction Identifier) allocated

- According to TS 29.274, TI is mandatory when the PS bearer is handed over to 3G

| Transaction Identifier | C | This IE shall be sent over S3/S10/S16 if the UE supports A/Gb and/or Iu mode. | TI | 0 |
| --- | --- | --- | --- | --- |

- MME can’t send the PS bearer without TI in Forward Relocation Request message to SGSN, but the target RNC is notified that there will be a CS+PS handover

- The result would be that RNC always needs to wait for PS bearers relocation according the specification.

- It is unclear how to handle the “never received PS handover”, RNC may fail the SRVCC.

With the introduction of NR, the PS bearers set up to 5G may not be able to handover to 3G, e.g.: 5G without TI cannot be handed over to 2/3G; PDNs setup in LTE with flags: “5GSIWK” or “5GCNRI” is set to 1 and sent to PGW/SMF, also can’t handover to 2/3G, because these PDNs are allocated with 5G PDU session ID and 5G QoS, etc. Similarly, some E-RAB from 2G/3G cannot be handed over to 5G.

It is beneficial if during E-RAB setup or handover, eNB is indicated that weather the E-RAB can be handled over to 2G/3G or 5G.

Proposal 1: RAN3 to discuss and agree to indicate to eNB if the E-RAB can be handed over to 2G/3G or 5G.

# 2	Proposals

Proposal 1: RAN3 to discuss and agree to indicate to eNB if the E-RAB can be handed over to 2G/3G or 5G.

# 3	Reference

[1]	R3-210890	Indicating E-RABs that cannot be handed over to 2G/3G or 5G, Ericsson

[2]	R3-210891	Indicating E-RABs that cannot be handed over to 2G/3G or 5G, Ericsson