3GPP TSG-RAN WG3 #111-e	R3-211116

Online, 25 January – 4 February 2021

Agenda Item:	9.3.8.1

Source:	CATT  (moderator)

Title:	Summary of offline discussion on trigger condition for trace failure Indication procedure

Document for:	Approval

# Introduction

CB: # 104_TraceFailTriggering

- acknowledge issue? If so,

- check vs. st2 -> liaise SA5?

- check CR details

(CATT - moderator)

Summary of offline disc R3-211116

# For the Chairman’s Notes

Propose the following:

# Discussion

In TS 32.421, there is a text as below:

- The Trace collection entity shall be notified, in case of theTrace Session activation has failed, by the response message with the specific cause (e.g. overload) from the NE on which the Trace Session activation failure happened.

In TS 32.422, there is a text about Trace Session activation in case of overload in NG-RAN as below:

Tracing starts immediately at NG-RAN node upon reception of the trace control and configuration parameters Trace Activation IE). The NG-RAN node may not start a Trace Recording Session if there are insufficient resources available for the recording, however, the NG-RAN node shall store the trace control and configuration parameters, and forward these parameters when the UE handovers to other NG-RAN nodes over Xn or when other NG-RAN node retrieves the UE Context over Xn.

If we follow the description in 32.421, NG-RAN node shall inform AMF of the occurrence of Trace Session activation failure.

However, according to the description in 32.422, NG-RAN node the NG-RAN node shall store the trace control and configuration parameters, and forward these parameters when the UE handovers to other NG-RAN nodes.

Then, based on the description in 32.421 and 32.422, in case of overload, the behavior of NG-RAN node should be as below:

NG-RAN node should first inform AMF via Trace Failure Indication message, and then store the trace control and configuration parameters. If Xn based handover procedure happened, NG-RAN node should forward the trace configuration.

Question 1: Whether the behavior of NG-RAN node described above is reasonable or not?

| Company | Yes/No | Comment |
| --- | --- | --- |
| CATT | Not completely | Normally, when a configuration failed in the NG-RAN node, NG-RAN node would not keep the corresponding configuration. However, for this case, it is required for NG-RAN node to keep the trace configuration even the procedure failed. |
| Samsung | No | We think the above description in 32.422 means NG-RAN will not send Trace Failure, just not start the measurement in case of insufficient resource. From 32.422, the case when NG-RAN sends failure is as below: If the NG-RAN node is not able to activate the trace session due to ongoing handover of the UE to another NG-RAN node, the NG-RAN node shall inform the AMF with the TRACE FAILURE INDICATION message using NG interface. |

Question 2: Whether the CR in [3] is agreeable?

| Company | Yes/No | Comment |
| --- | --- | --- |
| CATT | Yes | Anyway, the information is needed for AMF |
| Samsung | No | We make further checking in F1 and E1, in F1 and E1, there is no Trace Activation Failure message. If NG-RAN can fail the Trace Activation in NG due to resource reason, probably means DU or CU-UP also can fail the Trace Activation due to resource reason. If NG-RAN/DU/CU-UP don’t have enough resource, they can temporary not make the measurement but keep the context. We think this understanding is much aligning with the current stage 2 and stage 3. |

Question 3: Is there anything which is unclear or ambugius for the overload case which needs to contact SA5?

| Company | Yes/No | Comment |
| --- | --- | --- |
| CATT | Yes | 3 points from our side: 1 Normally, when a configuration failed in the NG-RAN node, NG-RAN node would not keep the corresponding configuration. However, for this case, it is required for NG-RAN node to keep the trace configuration even the procedure failed. 2 What would be the behavior of AMF when it receives a trace failure indication message with overload cause value? 3 If the load the NG-RAN node is not heavy anymore and it could implement the trace procedure again, should NG-RAN node inform AMF? |
| Samsung | No |  |

# Conclusion, Recommendations [if needed]

If needed

# References

| 1 | TS 32.421 | Trace concepts and requirements |
| --- | --- | --- |
| 2 | TS 32.422 | Trace control and configuration management |
| 3 | R3-210300 | “Clarification on trigger condition for trace Failure Indication procedure” RAN3#111-e |

# Appendix