3GPP TSG-RAN WG3 #111-e	R3-210973

Online, 25 January - 5 February 2021

Agenda Item:	20.2.4

Source:	Ericsson (moderator)

Title:	CB: # 28_NTN_FeederSwitch - Summary of email discussion

Document for:	Approval

# Introduction

This is the chair’s summary:

CB: # 28_NTN_FeederSwitch

CT

The impact of XnAP should be minimized in the execution of feeder link switch over for NTN.

The impact of feeder link switch over on RAN3 can wait for RAN2 progress.

CATT

NTN control functions (or OAM) could control the feeder link switch by providing the strategy of feeder link switch for corresponding gNBs and NTN GWs.

Introduce a new non-UE Xn procedure for feeder link switch, to exchange the necessary info between the gNBs, including satellite information, served cell(s) information.

the order of the serving cell list should be kept same between the source and target gNBs to maintain the correct neighbour relationship.

In NG, introduce a Container to transfer the satellite configuration in UPLINK RAN CONFIGURATION TRANSFER and DOWNLINK RAN CONFIGURATION TRANSFER NGAP messages.

existing Xn/NG handover procedure could be taken as the baseline, whether and how to further minimize signalling overhead for feeder link switch are pending to RAN1 and RAN2.

NTN System could be treated as a repeater of the gNB, the figure x.y in [3] could be endorsed in the annex of TS 38.300 as the reference deployment.

Nok

To Support feeder link switch, current NG/Xn based HO procedure can be reused, and no need to introduce enhancement to XnAP specification and NGAP specification.

To Support feeder link switch, wait for RAN2 decision regarding the impact to F1AP specification.

SS

discuss the exchange of available RACH resources between source and target to support RACH attempts distribution.

discuss the exchange of handover UE list and handover policy between source and target to support RACH attempts distribution.

discuss the procedures of group handover preparation and group path switch for collective hand-over to reduce the signalling overhead in Xn and NG.

E///

To support periodic switchover, add to Xn Setup and NG-RAN Configuration Update procedures the list of satellites to which the gNB connects, and for each satellite on the list include at least the list of cells from the gNB served through the satellite, and the ephemeris data.

To support event-triggered switchover, a new XnAP Class 1, non-UE-associated Satellite Connection Preparation procedure can be introduced.

Discuss the related XnAP CR (which includes the complete signaling “package” to support both periodic and event-triggered switchover); we welcome further discussion especially from operators on introducing the Satellite Connection Preparation procedure.

If switchover involving NTN gNBs which do not connect to the same AMF set (e.g. inter-PLMN) needs to be supported, it seems necessary to introduce another mechanism (e.g. transparent containers through the core network); we welcome further discussion on this aspect.

If we assume Xn to be deployed and the appropriate configuration information to be exchanged beforehand (see previous proposals), the “soft” switchover can be considered to be supported reusing the existing NR mobility mechanisms, at least for what concerns RAN3.

For RAN3 purposes, the NTN GW and the connected satellite(s) can be considered as part of the “NG-RAN node” logical node; the information related to their configuration can therefore be considered part of the NG-RAN node configuration.

ID

XN Satellite Connection Request, Satellite Connection Request Acknowledge and Satellite Connection Request Reject are needed.

The same XN Satellite Connection Request, Satellite Connection Request Acknowledge and Satellite Connection Request Reject are needed with a time reference for the hard switch.

Whether these switchover procedures are needed over the NG interface in addition to the XN is FFS, if it is needed this can be done by the use of transparent containers to handle the inter-PLMN information.

Target cell id and proper RRM measurement configuration should be signaled over open interfaces to allow for a common method of handling changes in cell configuration from switchover (soft or hard) Probably at cell setup since the relations will be active at predetermined times and thus can be either valid for a fixed time-period or switched easily if needed, instead of a complete reconfiguration.

HW

info related to feeder link switch is not dynamic and should be available into the gNB long time before the switch

feeder link switch may result of cell schedule management indication over Xn

main feeder link hard switch impact on RAN3 is pending to RAN2 progress.

Intel

enhance Xn to transfer NTN cell coverage information, together with the information about how cell coverage changes over time.

enhance Xn Setup and NG-RAN Configuration Update procedures to transfer NTN cell coverage information, together with the information about how cell coverage changes over time.

satellite beam footprint is signaled by the coordinates of its two focal points.

together with the satellite beam footprint coordinates, its velocity (speed and direction) are signaled.

ZTE

gNB could be informed about the scheduling of switch-over based on the ephemeris data via OAM.

potential enhancement for feeder link switch-over should be pending to RAN2.

feeder link switch-over procedure captured in TR 38.821 could be reused as baseline, and the details should be further discussed.

CMCC

no matter what kinds of deployment of satellite and NTN-gateway, the main task is ensuring the service continuity.

gNB can be informed about the scheduling information through NTN control functions by the control of signaling as well as OAM.

some information that be used for exchanging on cell relation between RAN nodes via XN/NG. For example, satellite ID, a list of served cells information from the gNB covered by the satellite, and the ephemeris data for the satellite.

One solution may be left to network implementation, e.g., setting proper event A5 thresholds for conditional handover to enable handover, or to rely on radio propagation time instead or in combination with the RSRP/RSRQ radio measurements.

Feeder link hard switch procedure is based on accurate time control and conditional RRC re-establishment.

- Consensus to exchange satellite-related info at Xn setup / node config update?

- If so, which info? E.g. list of cells? Ephemeris? Beam focal points? How cell coverage varies over time?

- consensus for XnAP support for switchover?

- Periodic vs. event-triggered switchover: no signaling impact for periodic switchover?

- hard vs. soft switchover: no RAN3 impact for hard switchover?

- select st2, st3 BL?

- check details, revise/merge as needed

(E/// - moderator)

Summary of offline disc R3-210973

# For the Chairman’s Notes

Propose the following: (copy from the agreements below)

# Discussion

## Existing Xn/NG Handover procedures baseline for transfer of UE contexts to the target gNB and execution of handover

There seems to be the common understanding, that the Xn and NG Handover procedures can serve as baseline. Any necessary addition for the Handover function needs to be awaited from RAN2 discussions. It is also assumed that the current parameterization (e.g. target identification) can be used for NTN purposes. The following is proposed. Whether NG Handover is actually used (e.g. due to lack of Xn interface) is of secondary matter, as long as there are no changes to those procedures.

The moderator proposes the following basic agreement as a warm up for further discussions:

Existing Xn and NG Handover functions are used to support the feeder link switch on a per-UE basis. It is assumed that the information exchanged in existing Handover procedures can be used for NTN purposes. Discussions on addition to the existing handover functions will be triggered from decisions made outside RAN3.

Q3.1: Please provide your view below:

| Company | Comment |
| --- | --- |

## Is there a Xn procedure needed to indicate the feeder link switch over?

It appears to the moderator, that the answer to this question lies within the assumption on how the satellite network is operated.

1)	The “centralized deployment option”: the decision to perform a feeder link switch is coordinated in a central way and assumed to be exactly predictable, then there is no need to signal an event that is known by all serving gNBs.

-	In case of soft switch over, the source gNB can configure UEs appropriately to execute the Handover, because it can assume that the handover at the target gNB will be successful with high probability.

-	In case of hard switch over, a scalable solution would need to be in place, which requires a very well synchronized HO execution in order to minimize communication gaps in the transition phase from the source to the target feeder link. Nevertheless, details of such schemes have to be discussed outside RAN3.

2)	The “de-centralized deployment option”: In this option, the feeder link switch decision is considered to be based on local (gNB/NTN Gateway) decisions (with centralized configuration of all sorts of satellite system information, as in option 1). The timing of the feeder link switch would predictable only within a certain timing range, but the actual switch (i.e. availability of the target feeder link and source feeder link in case of soft switch, and exact hard switch time) is not known in advance.

Satellite systems with a certain degree of redundancy could benefit from this kind of flexibility and apply maintenance, load balancing schemes, emergency maneuvers, or what not else is possible. OAM does not seem to be the best option for this kind of de-centralized operation.

Ad 1) It appears that for this “centralized” option, there is no need to indicate the feeder link switch between gNBs. What could be discussed is the benefit of exchanging configuration information. However, with the assumption of depending on configuration data stemming from a central OAM entity, this wouldn’t make much sense.

3GPP supports NTNs with central coordination of feeder link switch overs. In case of centrally coordinated switch over, no signaling is needed on Xn/NG, neither to exchange configuration information nor to coordinate the actual feeder link switch.

Q3.2-1: please provide your view below:

| Company | Comment |
| --- | --- |

Ad 2) For this “de-centralized” option, there seems to be the need for introducing signaling between the gNBs. The question is whether we assume the deployment of such systems.

The following proposed agreement may appear a bit provocative to some, but the moderator intends to trigger a good discussion:

3GPP supports NTNs with de-centralized coordination of feeder link switch overs. In case of de-centralized coordinated switch over, inter-gNB signaling is needed. Source and target gNB aspects have to be further discussed. Given to semi-static nature of the satellite system, there is no need to exchange configuration information between the gNBs.

Q3.2-2: please provide your view below:

| Company | Comment |
| --- | --- |

## Setup of Xn interface to enable Xn Handover

Given the common understanding to support Xn handover for feeder link switch, many documents discuss the actual content of Xn interface management messages. Given the fact, that XN SETUP REQUEST / RESPONSE only foresee the Node ID, TAI Support List and AMF Region Information to be mandatory present, the Served Cell IEs however optional, one option could be to foresee the possibility to not include any Served Cell information at all (assuming that information to come directly from OAM) and have an explicit indicator like “Served Cell information for NTN provided by OAM” or similar.

The moderator proposes to agree on the following statement:

3GPP supports Xn-connected gNBs providing non-terrestrial NR access. Served Cell Information and Neighbour Cell Information for cells providing non-terrestrial NR access may be provided to the gNBs exclusively by OAM without being exchanged via XnAP means. If this option is deployed, XnAP should provide means to indicate explicitly the deployment of that option.

Further discussions on how to enhance/modify existing XnAP Served Cell Information and Neighbour Cell Information for cells providing non-terrestrial NR access are not precluded.

| Company | Comment |
| --- | --- |

## Text Proposals for stage 2 and stage 3

To be seen after Thursday discussions

| Company | Comment |
| --- | --- |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-210119	Discussion on feeder link switch-over for NTN (China Telecommunication)

- R3-210472	(TP for BL CR for TS 38.300) Support of Feeder Link Switch (CATT)

- R3-210495	Discussion on Feeder Link Switchover (Nokia, Nokia Shanghai Bell)

- R3-210511	Discussion on enhancements for feeder link switch over (Samsung)

- R3-210519	Further Discussion on Feeder Link Switching (Ericsson LM)

- R3-210521	Further Stage 2 Details for Feeder Link Switchover (Ericsson LM)

- R3-210520	Support for Feeder Link Switchover for Transparent Architecture (Ericsson LM), CR TS 38.423

- R3-210671	Feeder Link Switch (InterDigital Communications)

- R3-210706	Update on feeder link switch for NTN for stage3 (Huawei)

- R3-210751	Satellite information transfer over Xn (Intel Corporation)

- R3-210752	NTN support in Xn (Intel Corporation), draftCR TS 38.423

- R3-210808	Further Discussion on LEO Feeder Link Switch-over (ZTE)

- R3-210912	Discussion on feeder link switch for NTN (CMCC)

- R3-210151	NTN switch overs (THALES)