3GPP TSG-RAN WG3 #111-e	R3-210957

Online, 25 January – 4 February 2021

Agenda Item:	8.3.5

Source:	Huawei (moderator)

Title:	Summary of Offline Discussion on Updating QoS parameters at Xn Handover

Document for:	Approval

# Introduction

This contribution is to kick off the following discussion.

CB: # 4_QoSupdate@XnHO

HW,E///,ZTE,CATT,SS

Add the following in the PATH SWITCH REQUEST ACKNOWLEDGE message in TS 38.413:

- Uplink CN PDB

- Downlink CN PDB

- Downlink Burst Arrival Time

The NG-RAN uses the PDU Session Resource Notify message to notify that the updated QoS parameters during the Path Switch Request procedure are not successfully accepted by the NG-RAN node.

- discuss and check details; revise as needed

(HW - moderator)

Summary of offline disc R3-210957

# For the Chairman’s Notes

Propose to capture the following:

Proposal 1:

It is proposed to implement the proposal 1 as follows

Proposal a:

# Discussion

## Update the Path Switch Request Acknowledge message

In [1], it is proposed to add the updated parameters in the Path Switch Request Acknowledge message, with the following proposal, based on the section 4.9.1.2.2 of TS 23.502, and the LS from SA2 in [2].

- Add the following in the PATH SWITCH REQUEST ACKNOWLEDGE message in TS 38.413:

  - Uplink CN PDB

  - Downlink CN PDB

  - Downlink Burst Arrival Time

The proposed update is highlighted in the following table.

- 9.3.4.9 Path Switch Request Acknowledge Transfer This IE is transparent to the AMF. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. <Unchanged Text Omitted>. QoS Flow Parameters List; 0..1; YES; ignore. >QoS Flow Parameters Item; 1..<maxnoofQoSFlows>; -. >>QoS Flow Identifier; M; 9.3.1.51; -. >>Alternative QoS Parameters Set List; O; 9.3.1.151; Indicates alternative sets of QoS parameters for the QoS flow.; -. >>CN Packet Delay Budget Downlink; O; Extended Packet Delay Budget 9.3.1.135; Core Network Packet Delay Budget is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise.; YES; ignore. >>CN Packet Delay Budget Uplink; O; Extended Packet Delay Budget 9.3.1.135; Core Network Packet Delay Budget is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise.; YES; ignore. >>Burst Arrival Time Downlink; O; Burst Arrival Time 9.3.1.133; Indicates the downlink Burst Arrival Time of the TSC QoS flow; YES; ignore.

Please provide your view on this.

| Company | Comment |
| --- | --- |
| Huawei | OK |

Moderator’s summary:

## Notification to the CN when the updated QoS flow parameters cannot be successfully accepted

In [1], it is proposed that when the NG-RAN receives the updated QoS parameters in the Path Switch Request Acknowledge message, it performs admission control, and should inform the CN in case these updated QoS parameters cannot be successfully accepted. Then, the NG-RAN node can use the PDU Session Resource Notify procedure to feedback to the CN.

Also it is mentioned that, in this case, the NG-RAN should continue to use the old values received from the source NG-RAN node, and regards this parameter update as an erroneous parameter modification. And the feedback to the CN here has nothing to do with the Notification control or the Alternative QoS profile concept.

The proposal is given as follows.

- The NG-RAN uses the PDU Session Resource Notify message to feedback that the updated QoS parameters during the Path Switch Request procedure are not successfully accepted by the NG-RAN node.

The proposed update to the tabular is highlighted in the following table.

- 9.3.4.5 PDU Session Resource Notify Transfer This IE is transparent to the AMF. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. QoS Flow Notify List; 0..1; -. >QoS Flow Notify Item; 1..<maxnoofQoSFlows>; -. >>QoS Flow Identifier; M; 9.3.1.51; -. >>Notification Cause; M; ENUMERATED (fullfilled, not fulfilled, …); -. >>Current QoS Parameters Set Index; O; Alternative QoS Parameters Set Notify Index 9.3.1.153; Index to the currently fulfilled alternative QoS parameters set. Value 0 indicates that NG-RAN cannot even fulfil the lowest alternative parameters set.; YES; Ignore. QoS Flow Released List; O; QoS Flow List with Cause 9.3.1.13; -. Secondary RAT Usage Information; O; 9.3.1.114; YES; ignore. QoS Flow Feedback List; 0..1; YES; ignore. >QoS Flow Feedback Item; 1..<maxnoofQoSFlows>; -. >>QoS Flow Identifier; M; 9.3.1.51; -. >>Feedback Cause; O; ENUMERATED (QoS parameters not updated, …); -.

Please provide your view on this.

| Company | Comment |
| --- | --- |
| Huawei | OK |

Moderator’s summary:

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-210584 Introducing QoS parameters update at Xn handover, Huawei, Ericsson, ZTE, CATT, Samsung

- R3-210585 Introducing QoS parameters update at Xn handover, Huawei, Ericsson, ZTE, CATT, Samsung

- R3-206569 Reply LS to RAN3 for clarification of QoS Parameters Update at Xn Handover， SA WG2