3GPP TSG-RAN WG3 #111-e							R3-21xxxx

25 January – 4 February 2021

Online

Agenda Item:	15.2

Source:	Huawei - Moderator

Title:	Summary of Offline Discussion on RAN Configuration and Reporting of QoE measurement

Document for:	Approval

# Introduction

CB: # NRQoE3-RANConfig_Report

- QoE support for new services, SA4 LS in R3-210041? Support non-3GPP standardized application? Support Time Sensitive Communication and liaise SA4?

- Support Management-based QoE solution where a UE identifier is optionally included in the QoE measurement configuration, to enable collecting QoE reports from an individual UE? Yes, E///. No. Nokia.

- Signalling for Management-based and Signalling-based QoE supports multiple QoE measurements for different service types for the same UE? Whether and how to deactivate one of the multiple QoE measurements? Need check by RAN?

- There is no need to introduce RAN autonomously triggered QoE measurement mechanism?

- Radio related assistance measurements used for QoE management are only meaningful when there is at least one on-going QoE recording session?

- QoE handling at RAN overload? QoE report suspending in RAN overload and RRC_IDLE/INACTIVE? Reply to SA4 LS in R3-210042

- Can RAN and/or OAM start/stop/pause/resume measurement collection at the UE? Independently or as per triggering conditions? For both legacy and RAN-visible QoE?

- Can RAN and/or OAM start/stop/pause/resume measurement reporting from the UE? Independently or as per triggering conditions? For both legacy and RAN-visible QoE?

- How to align radio measurements and QoE reports?

- Capture agreements as TP for TR

(HW - moderator)

Summary of offline disc R3-211013

# For the Chairman’s Notes

Propose to capture the following:

To be continued

# Discussion

Since this should be the last time for this SI, it seems that there are still many open issues, but some of them are pure stage 3 details, here moderator tried to, according to the guidance of this CB from chair, directly extract proposals from contributions and reformate to issues.

Issue 1: Support for new service, e.g. XR, URLLC, also including non-3GPP standardized application and TSC

Moderator’s note: we see some discussions in [6] [9] [10], also we see reply LS from SA4 in [1], we may need some consensus, e.g. to work on something which is also being discussed in SA4. Companies are invited to provide view for each service type mentioned here.

| Company | Comment |
| --- | --- |
| Huawei | In general, RAN3 should work on the service type(s) which is also being discussed in SA4. Based on this understanding, we think: - XR could be introduced - URLLC is a general description of a service category, we need to see which exact service type we are discussing; - No need to discuss non-3GPP standardized application now, we could start this from SA4. - No need to discuss TSC now, since TSC as a service type is knows by RAN via QoS parameter, and SA4 has not discussed TSC yet. |

Issue 2: Whether to support RAN autonomously triggered QoE measurement

Moderator’s note: we see some discussions in [9] [11] about RAN autonomously triggered QoE measurement. Please companies provide your view (yes/no) and comments if any.

| Company | View | Comments |
| --- | --- | --- |

Issue 3: Whether a UE identifier is optionally included in the management based QoE measurement configuration

Moderator’s note: we see some discussions in [6] [8] about optionally including a UE identifier in the management based QoE measurement configuration, in order to enable collecting QoE reports from an individual UE. Please companies provide your view (yes/no) and comments if any.

| Company | View | Comments |
| --- | --- | --- |

Issue 4: Supports for multiple QoE measurements for the same UE? Whether and how to deactivate one of them? Need check by RAN2?

Moderator’s note: we see some discussions in [5] [6] [8] about supporting for multiple QoE measurements for the same UE for both siganlling based and management based, whether and how to deactivate one of them, whether to need check by RAN2. Please companies provide your view (yes/no) and comments if any.

| Company | View | Comments |
| --- | --- | --- |

Issue 5: QoE handling at RAN overload

Moderator’s note: we see some discussions in [6] about how QoE measurement is handled at RAN overload scenario, the proposals could be summarized as follows:

For SA operation or EN-DC/NR-DC when both nodes are overloaded, RAN is allowed to

- Not to trigger new QoE measurement if requested

- Release/suspend ongoing QoE measurement, fulfilling SA4 and RAN2 agreements

- Suspend QoE reporting

For EN-DC/NR-DC when only one node is overloaded, RAN is allowed to

- QoE measurement is reported towards the non-overloaded RAN node, with reconfigured MCG/SCG SRB bearer

- previous overloaded node may request the QoE report from the other node when overload is solved, if analysis justify

Please companies provide your comments on each of the proposals above..

| Company | Comments |
| --- | --- |

Issue 6: Handling for QoE collection and QoE reporting

Moderator’s note: we see some discussions in [6] [7] about Handling for QoE collection and QoE reporting. The issues could be summarized as follows:

For both reporting and collection:

- Can RAN and/or OAM stop/re-start, pause/resume the operation at the UE? Are there any conditions for triggering such operation?

- Is such operation applicable to both legacy and RAN-visible QoE?

Please companies provide your view (yes/no) and comments if any.

| Company | Comments |
| --- | --- |

Issue 7: Handling for QoE measurement and its corresponding radio assisted measurement

Moderator’s note: we see some discussions in [12] [13] about handling for QoE measurement and its corresponding radio assisted measurement. The issues could be summarized as follows:

- If QoE measurement is stopped/released, this corresponding radio related assistance measurements should also be stopped/released. According to the indication from UE application layer?

- Bearer information (e.g. DRB list or QoS flow ID) related to the QoE measurement should be indicated to the gNB or QoE server for Correlation.

- the collection of the radio related assistance information should include all the serving gNBs that the UE went through during the corresponding time of one QoE report.

- the radio related assistance information should be provided along with the QoE report by UE, inside or outside of the QoE report container is FFS

Please companies provide your view and comments for each proposal above, if any.

| Company | Comments |
| --- | --- |

Issue 8: General rules for MR-DC

Moderator’s note: we see some discussions in [9] about some general rules for MR DC operation when triggering QoE measurement. The issues could be summarized as follows:

- For MR-DC operation including NSA, only one node is allowed to configure the QoE measurement, and the MN could decide which node to configure the QoE measurement for a certain service type.

- For a gNB acting as SN, it could configure the QoE measurement directly towards that UE over SN leg.

| Company | Comments |
| --- | --- |

Other issues:

If there are any other issues, please list here.

# Conclusion, Recommendations

See section 2.

# References

- R3-210041, LS Reply on New service type of NR QoE, (SA WG4)

- R3-210042, LS reply on QoE Measurement Collection, (SA WG4)

- R3-210355, QoE report suspending in RAN overload and RRC_IDLE/INACTIVE, (Qualcomm Incorporated)

- R3-210356, LS reply on QoE Measurement Collection, Qualcomm Incorporated

- R3-210507, Open issues in NR QoE solutions, (Samsung)

- R3-210527, pCR for TR 38.890: Handling of QoE Measurement and Reporting and Support for New Services, (Ericsson)

- R3-210658, (TP for TR 38.890) QoE measurement collection continuity vs. reporting continuity, (Nokia, Nokia Shanghai Bell)

- R3-210659, (TP for TR 38.890) QoE measurement configuration for specific UEs, (Nokia, Nokia Shanghai Bell)

- R3-210820, Further discussions on the remaining open issues of QoE configuration and reporting, (Huawei)

- R3-210848, Further consideration on NR QoE service and procedure, (ZTE)

- R3-210772, Discussion on RAN visible QoE configuration and reporting, CATT

- R3-210849, TP for TR 38890, (ZTE)

- R3-210509, Discussion on radio related measurements and information in NR QoE, Samsung